3GPP TSG RAN WG2 Meeting #123	R2-2309007

Toulouse, France, August 21-25, 2023

Title:	Reply LS on XR capacity enhancements

Response to:	R2-2307014 (R1-2306233) LS on XR capacity enhancements

Release:	Rel-18

Work Item:	NR_XR_enh-Core

Source:	RAN2

To:	RAN1

Cc:

Contact person:	Pradeep Jose

pradeep dot jose at mediatek dot com

# 1	Overall description

RAN2 would like to thank RAN1 for agreeing on RAN2 specification impact of HARQ process ID determination for multi-PUSCH configured grant. RAN2 would like to provide the following feedback.

1. RAN2 has identified an error in the RAN1 agreement as below:

The HARQ process ID of the Kth (1 < K ≤ numberOfPUSCH_PerPeriod) valid CG PUSCH occasion in the period is determined by:

HARQ Process ID = [numberOfPUSCH_PerPeriod *floor((CURRENT_symbol ) / periodicity) + (K-1)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

RAN2 will capture a corrected version of the HARQ process ID formula in TS38.321.

2. RAN2 would respectfully request RAN1 to capture the definition of an invalid CG PUSCH as below, and would also like to know where the definition will be captured in the RAN1 specifications (so that RAN2 specifications can refer to that definition).

Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

# 2	Actions

To RAN1

RAN2 respectfully asks RAN1 to provide their response to the question above.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 #123-bis		October 9th – 13th, 2023		Xiamen, China

TSG RAN WG2 #124		November 13th – 17th, 2023	Chicago, U.S.A.