3GPP TSG-RAN WG2 #123bis	R2-2309494

# Xiamen, China, October 9th - 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | XXXX | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running PDCP CR for NR Sidelink Evolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce NR sidelink evolution into PDCP Specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of NR sidelink PDCP duplicaiton feature into PDCP Specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 NR sidelink evolution is not supported by PDCP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 [5.11.2] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.2.3	Sidelink transmit operation

For NR sidelink transmission of the SLRB, the UE shall follow the procedures in clause 5.2.1 with following modification:

-	perform the header compression using ROHC as specified in clause 5.7.4, if SDU Type is IP;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[sl-PDCP-SN-Size];

- 	if the transmitting PDCP entity is associated with two RLC entities [for sidelink DRB and for sidelink SRB1/2/3], consider PDCP duplication as activated and perform transmit operation for PDCP duplication as specified in clause 5.2.1 with the associated RLC entities.

Editor’s NOTE:	FFS whether the PDCP duplication is supported also for sidelink SRB3 (including the WA “Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started”.

Editor’s NOTE: Applicability of SL PDCP duplication for sidelink SRB1/2.

Editor’s NOTE: FFS whether any transmit operation with primary and secondary leg distinction is needed for SL PDCP duplication (e.g. for PDCP control PDU transmission, etc.).

Editor’s NOTE: Impact of TX profile to SL PDCP duplication, including “FFS whether to use PDCP duplication or not is up to UE implementation”.

NEXT CHANGE

5.11.2	Duplicate PDU discard

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities:

-	indicate to the other AM RLC entities to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	indicate to the RLC entities other than the primary RLC entity to discard all duplicated PDCP Data PDUs;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	indicate to the RLC entities deactivated for PDCP duplication to discard all duplicated PDCP Data PDUs.

Editor’s NOTE: FFS whether ACK-based duplicate PDU discard operation also needs to be specified for a sidelink DRBs/SRBs associated with AM RLC entities for NR SL unicast.

CHANGE END