3GPP TSG-RAN WG2 Meeting #123bis	R2-2309526

# Xiamen, China, 9 – 13 October 2023

Agenda item:		7.23.2

Source:	Xiaomi

Title:	Remaining issues of timing synchronization status and reporting

Document for:		Discussion and Decision

Introduction

In RAN2#123 meeting, following was agreed regarding 5GS network timing synchronization status and reporting:

Agreement

- Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms.

- Confirm the gNB is always broadcasting Event ID in SIB9 if it supports the feature.  Send LS to SA2 to information.

- DLInformationTransfer message is extended to include clock quality information reporting towards the UE (i.e., clock quality metrics or clock quality indication).  FFS if event ID is included to be discussed in email discussion for 331

- As a baseline no change to SDT will be support.

After RAN2#123 meeting, email discussion “[POST123][309][R18 URLLC] Running 38.331 (Ericsson)” [2] was held to discuss stage-3 RRC signalling details.

In this contribution, we discuss remaining issues of 5GS network timing synchronization status and reporting.

Discussion

In draft RRC running CR [1], the handling of eventID is as below:

Upon receiving SIB9 with eventID the UE shall:

1>	if VarEventID has an entry with a storedEventID value:

2>	if the storedEventID value within VarEventID is different from the eventID value received within SIB9;

3>	consider that the content of clockQualityDetailsLevel has changed;

3> notify upper layers that clockQualityDetailsLevel has changed;

3> replace the storedEventID value within VarEventID with the eventID value received within SIB9;

1>	else:

2>	add a new entry of storedEventID within the VarEventID with a value set as the one of eventID value received within SIB9;

Editor’s Note: FFS if and how to use gNB ID as gNB ID is not currently visible to the UE in RRC.

The potential issue with current running CR is that only eventID is used to determine whether UE needs to retrieve clock quality information, and gNB ID is not taken into account. There is a risk that different gNBs use the same eventID and UE skips retrieving clock quality information after cell reselection to a neighboring gNB. RAN2 previously agreed: “Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer.” Similarly, SA2 TS 23.501 specifies that “Uniqueness of the event ID is ensured by combining with a gNB ID.” Therefore both gNB ID and event ID should be considered.

Proposal 1: In addition to event ID, gNB ID should be considered when checking the clock synchronization status.

Now we discuss how gNB ID can be identified by the UE. In RAN2#121 meeting, RAN2 concluded that “Identifying cells within the same gNB for option a) can already be supported with gNB ID length broadcasted”. The signalling of gNB-ID-Length and cellIdentity are in IE PLMN-IdentityInfo, broadcasted in SIB1, as below:

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode     OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode         OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

[Omitted]

[[

trackingAreaList-r17                SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL,      -- Need R

gNB-ID-Length-r17                   INTEGER (22..32)         OPTIONAL       -- Need R

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity.

It is specified in RAN3 TS 38.413 (as copied below) that gNB ID is the leftmost bits of the cell identity.

- 9.3.1.6	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

- 9.3.1.7	NR CGI

This IE is used to globally identify an NR cell (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) | The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.3.1.6). |

Proposal 2: gNB ID is the leftmost gNB-ID-Length bits of cellIdentity broadcasted in SIB1.

One example to implement Proposal 1 in the running RRC CR is as follows:

Upon receiving SIB9 with eventID the UE shall:

1>	if the gNB ID (leftmost gNB-ID-Length bits of cellIdentity in SIB1) is changed compared to the last time UE received SIB9 with eventID:

3>	consider that the content of clockQualityDetailsLevel has changed;

3> notify upper layers that clockQualityDetailsLevel has changed;

1>	if VarEventID has an entry with a storedEventID value:

2>	if the storedEventID value within VarEventID is different from the eventID value received within SIB9;

3>	consider that the content of clockQualityDetailsLevel has changed;

3> notify upper layers that clockQualityDetailsLevel has changed;

3> replace the storedEventID value within VarEventID with the eventID value received within SIB9;

1>	else:

2>	add a new entry of storedEventID within the VarEventID with a value set as the one of eventID value received within SIB9;

Conclusion

In this contribution, we discuss remaining issues of 5GS network timing synchronization status and reporting, and propose the following:

Proposal 1: In addition to event ID, gNB ID should be considered when checking the clock synchronization status.

Proposal 2: gNB ID is the leftmost gNB-ID-Length bits of cellIdentity broadcasted in SIB1.

- References

[1] R2-2310785, Ericsson, “Introduction of URLLC and Timing Resiliency”

[2] R2-2310907, Ericsson, “Summary of [POST123][309][R18 URLLC]RunningCR_38.331”