3GPP TSG-RAN WG2 #123bis	R2-2309687

Xiamen, China, October 2023

Agenda Item:	7.25.1

Source:	OPPO

Title:	Discussion on BS signaling support for (non-)collocated scenarios

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue from RP-232692.

Discussion

Based on the latest LS RP-232692,

This new BS signalling is applied to the below cases.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”.

For TDD-TDD intra-band NR-CA

This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

When it comes to the -r16 capability, it seems beneficial to consider backward compatibility.

If the network is a legacy network, but the UE is new, then it means the UE will work in the legacy mode, i.e.,

- Although the interBandMRDC-WithOverlapDL-Bands-r16 is reported

- But there is no BS signaling to differentiate the usage of type-1 and type-2

Or if the network is a new network, but the UE is legacy UE, it means the UE has to work in the new mode

- The interBandMRDC-WithOverlapDL-Bands-r16 is reported

- And there is no BS signaling to differentiate the usage of type-1 and type-2

- There is a backward compatibility issue for Rel-16 UE working in Rel-18 network, where the UE supports type-2 requirement, but does not support the new BS signaling.

So technically, unless there is no implementation of the r16 inter-band capability in the field, there is a need to introduce another capability for the inter-band case, to reflect that the UE supports the new BS signaling.

- R2 discusses the need for UE capability for inter-band case, to indicate whether the UE supports the new BS signaling for (non-)collocated scenario (i.e., type-1/2 requirement) differentiation.

On the other hand, if one believes there is a case that UE supporting both type-1/2 requirements, for the inter-band scenario, works without BS signaling, i.e., the legacy UE supporting interBandMRDC-WithOverlapDL-Bands-r16, it is questionable whether the UE supporting the new intra-band type-1/2 related capability, can work without BS signaling.

- R2 discusses whether the BS signaling is mandatorily present, for the intra-band case, to differentiate (non-)collocated scenario (i.e., type-1/2 requirement), for UE supporting both type-1/2 requirements.

Conclusion

We have the following proposals:

Proposal 1	R2 discusses the need for UE capability for inter-band case, to indicate whether the UE supports the new BS signaling for (non-)collocated scenario (i.e., type-1/2 requirement) differentiation.

Proposal 2	R2 discusses whether the BS signaling is mandatorily present, for the intra-band case, to differentiate (non-)collocated scenario (i.e., type-1/2 requirement), for UE supporting both type-1/2 requirements.