3GPP TSG-RAN WG2 Meeting #123b	 R2- 239808

Xiamen, China, 9 – 13 Oct, 2023

Agenda Item:	7.16.2.2

Source:	Xiaomi

Title:	Discussion on data collection

Document for:	Discussion

Introduction

In [1], Artificial Intelligence and machine learning for air interface is proposed. In last meeting, several data collection tools were proposed. In this contribution, we give our understanding on the data collection.

Discussion

Data collection can be used for model training, inference and performance monitoring. In the following part, we would discuss the data collection for different purposes.

Data collection for training

In this release we understand the offline training should be prioritized over on-line training. In offline training, the model can be trained before model delivery or deployment. The data for offline training can be collected any time before training. The training node can store the collected data during a long period. From latency point of view, all the tools listed in the table in [1] can be used to collect data for training.

Observation 1: From latency point of view, data collection for offline training doesn’t require strict latency. All the tools listed in the table in [1] can be used to collect data for training.

It’s more reasonable to assume the model training is done at NW side/OTT server rather than UE, considering the NW may have higher computing capability. The model training may require large amount data, e.g. millions of samples, to obtain a stable model. Note the data can be collected from different UEs to achieve maximum model commonality. Also, each UE can report multiple times. But, it’s also beneficial to enable one UE to store multiple samples and report stored multiple samples each time, so the report signalling can be reduced.

Observation 2: Signalling can be reduced if one UE can store multiple samples and report stored multiple samples each time.

However, it’s not clear whether the existing tools can support reporting multiple samples. Although the payload size is included in the table, but it doesn’t necessarily imply multiple samples can be reported each time. For example, in spatial domain beam prediction, each sample may include the measurement result of multiple beams. It’s possible that NW can configure beam measurement corresponding to multiple samples. Therefore, UE can report multiple samples by L3 measurement report. But in temporal beam prediction, each sample may include multiple measurement results of the same beam on different timing. In current measurement report, UE would only report the latest measurement result. Therefore, UE can’t report multiple samples for temporal beam prediction by L3 measurement report. RAN2 should study how to enable one UE report multiple samples of data for training.

Proposal 1: RAN2 to study how to enable one UE report multiple samples of data for training in one report.

Considering the positioning for RRC IDLE/INACTIVE UE had already introduced in the Rel-17 and Rel-18, the legacy procedures for RRC IDLE/INACTIVE UE positioning can be reused for the data collection for AI model training. Moreover, data collection from RRC IDLE/INACTIVE UE is friendly for UE power consumption.

Proposal 2: The data collection from RRC IDLE/INACTIVE UE should be considered for training AI model for positioning.

In the current design, UE’s report can be initiated in following ways,

- Periodic

- Event fulfilment

- gNB request

The data may be associated with certain NW configuration. When the associated NW configuration changes, UE shall report the collected data. Option 2 and 3 may be useful in such cases. If the data is not associated with certain NW configuration, e.g. universal data, UE can report the collected data periodically. Therefore, we understand RAN2 can study all these 3 options for UE to initiate report of collected data for training.

Proposal 3: RAN2 can study following options for UE to initiate report of collected data for training,

- Periodic

- Event fulfilment

- gNB request

Data collection for inference

In beam management and CSI use cases, the inference result is expected to be used very shortly after data for inference is collected, e.g. same slot or several slots later. The inference result may be outdated very quickly. Therefore, only the L1 report can be considered to report data for inference. Alternatively, the node which performs inference shall collect the data for inference locally. Either way, the existing tools in RAN2 can’t fulfil the latency requirement.

Observation 3: The existing tools in RAN2 can’t fulfil the latency requirement of data collection for inference for beam and CSI use case.

The next question is whether RAN2 should define new tools to report data for inference. We understand the data for inference may not require large size, so L1 report can fulfil the requirement.

Observation 4: L1 report can fulfil the latency and size requirement of data collection for inference for beam and CSI use case.

Therefore, we propose RAN2 doesn’t define new tools to report data for inference unless asked by RAN1.

Proposal 4: RAN2 doesn’t define new tool to report data for inference unless asked by RAN1 for beam and CSI use case. Data collection for inference relies on L1 report for beam and CSI use case.

For LMF based positioning, UE reports the measurements to LMF by LPP and/or gNB reports the measurement to LMF by NRPPa respectively, and then LMF calculates the UE location according to the positioning measurements. For UE based positioning, UE get the measurement according to the LMF requirement and then reports the estimated location to the LMF. Therefore, for AI based positioning, if the AI model is deployed at LMF, the UE and/or gNB should provide the input to the LMF by LPP and NRPPa respectively, if AI model deployed at UE, UE can get the input based on the PRS measurement and the data and then sends the output to LMF by LPP message, if AI model deployed at gNB, the gNB gets the input for AI model based on the SRS measurement and then sends the output to the LMF by NRPPa message.  Therefore, For the AI based positioning, the LPP and NRPPa messages can be reused for data collection for inference.

Proposal 5: The legacy LPP and NRPPa messages can be reused for data collection for inference for positioning use case.

Data collection for performance monitoring

There is no strict latency requirement for performance monitoring. However, RAN1 is considering ‘fast’ performance monitoring. It’s not clear how ‘fast’ is expected. Further input from RAN1 on the latency requirement of performance monitoring is needed to evaluate whether existing tools can fulfil the requirement.

Observation 5: Further input from RAN1 on the latency requirement of performance monitoring is needed to evaluate whether existing tools can fulfil the requirement.

There are two ways to evalute the performance of AI/ML performance considered by RAN1,

- by the radio system performance;

- by specific KPIs related to AI/ML, e.g. prediction accuracy or difference between prediction and ground truth.

If option 1 is used, legacy report may be reused to indicate the radio system performance, e.g. RSRP or throughput. If option 2 is used, new information should be introduced to report specific KPI. However, this still needs RAN1 input on the definition of performance monitoring KPIs.

Observation 6: Further input from RAN1 on the definition of performance monitoring KPIs is needed to design the data collection for performance monitoring in RAN2.

Proposal 6: RAN2 wait for RAN1 input on performance monitoring, e.g. latency requirement and definition of performance monitoring KPIs.

Conclusion

Based on the discussion in section 2, we have following proposals:

Observation 1: From latency point of view, data collection for offline training doesn’t require strict latency. All the tools listed in the table in [1] can be used to collect data for training.

Observation 2: Signalling can be reduced if one UE can store multiple samples and report stored multiple samples each time.

Observation 3: The existing tools in RAN2 can’t fulfil the latency requirement of data collection for inference for beam and CSI use case.

Observation 4: L1 report can fulfil the latency and size requirement of data collection for inference for beam and CSI use case.

Observation 5: Further input from RAN1 on the latency requirement of performance monitoring is needed to evaluate whether existing tools can fulfil the requirement.

Observation 6: Further input from RAN1 on the definition of performance monitoring KPIs is needed to design the data collection for performance monitoring in RAN2.

Proposal 1: RAN2 to study how to enable one UE report multiple samples of data for training in one report.

Proposal 2: The data collection from RRC IDLE/INACTIVE UE should be considered for training AI model for positioning.

Proposal 3: RAN2 can study following options for UE to initiate report of collected data for training,

- Periodic

- Event fulfilment

- gNB request

Proposal 4: RAN2 doesn’t define new tool to report data for inference unless asked by RAN1 for beam and CSI use case. Data collection for inference rely on L1 report for beam and CSI use case.

Proposal 5: The legacy LPP and NRPPa messages can be reused for data collection for inference for positioning use case.

Proposal 6: RAN2 wait for RAN1 input on performance monitoring, e.g. latency requirement and definition of performance monitoring KPIs.

Reference

[1] R2-2302401	RAN2#121 Meeting Report	MCC	report