3GPP TSG-RAN WG2 Meeting#123bis                               	R2-2309868

Xiamen, China, October 9-13, 2023

Agenda item:	7.3.5

Source: 	Google Inc.

Title: 	Deactivation and activation of a CHO event for NES

Document for:	Discussion and Decision

# Introduction

The NES techniques in time and frequency domains aim to reduce the power consumption by turning off some symbols/slots/frames on one or more carriers, and hence the gNB is allowed to perform some micro/light/deep sleep, depending on the interval between the contiguous active transmission/reception occasions. However, these NES techniques may increase the data transfer latency for a connected UE, as allowing a gNB to sleep means the UE is not always able to communicate with the gNB. It is especially intolerable for the UEs having stringent data latency requirement if the gNB has to sleep for a very long period of time. For those UEs not tolerable of such NES techniques, it is important to handover the UEs in time to another gNB that does not enable these NES techniques, before the serving gNB goes to sleep. Regarding the CHO enhancements, RAN2 has made the following agreements during in the RAN2#122 and RAN2#123 meetings [1][2].

Agreements in RAN2#122 - We will have a CHO solution that considers NES mode of at least source cell. - We can have a specific NES CHO execution condition based on source cell NES mode. FFS how the UE determines is in NES mode. FFS on how this is achieved in RRC - We will not introduce new L1 signalling for the purpose of CHO - Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario. We will study the time based mechanism

Agreements in RAN2#123 - We will support the CHO triggers for the use case of turning off the cell. - (At least for cell DTX/DRX) Time-based CHO is not to be considered in NES - Do not consider using an indication in SIB1 for triggering NES CHO execution condition

Based on the above agreements, this paper explores some more details that need to be done in order to accomplish the CHO scheme/design supporting the NES feature.

# Discussion

Since the gNB may need to handover a large number of UEs before starting a cell non-active period, it might be better if the gNB can start configuring the candidate cells and/or the CHO execution conditions to different UEs as early as possible, and let these UEs autonomously activate the CHO execution conditions a short time before the gNB starts the cell non-active period. In order to avoid the CHO execution being blocked by the signal strength/quality of the serving cell, condEventA4 was agreed in the RAN2#122 meeting as one of the CHO execution since it only evaluates the signal strength/quality of the candidate cell. In addition, in order to assure a successful handover, the network can configure condEventA4 with a small threshold value. The small threshold value in condEventA4 makes it easier for the UE to execute the CHO toward a candidate cell, but still blocks the CHO execution if the candidate cell does not exist or has very poor signal strength. However, a condEventA4 with a small threshold value could make the handover occur immediately upon being configured to the UE.

CondEventA4 with a low threshold value can assure a successful handover before the start of a cell non-active period, but also makes the handover occur sooner than needed.

Given that the interval between when the UE is provided with the CHO configuration and when the cell non-active period actually starts could be quite long, the UE may not want to move to the candidate cell immediately upon being provided with the CHO configuration, as the serving cell may still be better in terms of the signal strength/quality. Therefore, it is very reasonable for the NW to deactivate the MeasId associated to condEventA4 upon configuring the MeasId to the UE, and then to activate the deactivated MeasId at a later time before the occurrence of the cell non-active period. The NW may deactivate the MeasId explicitly or autonomously upon configuring the MeasId to the UE. For instance, the NW may signal a MeasId together with a flag associated to the MeasId, which indicates whether this MeasId is being activated or deactivated. On the other hand, a MeasId can be considered (by both the UE and NW) as being deactivated by default if the MeasId is associated with condEventA4.

The MeasId associated to condEventA4 can be deactivated upon being configured to a NES-capable UE. FFS on how to deactivate the MeasID associated to condEventA4 (e.g., autonomously or explicitly deactivate).

Regarding the mechanism or the timing to activate a deactivated MeasId, we think a deactivated MeasId can be activated upon the activation of the serving cell DTX/DRX/OFF mode, if the serving cell DTX/DRX/OFF mode is being activated explicitly using the L1 signal agreed in RAN1 (i.e., the DCI format 2_X). Similarly, an activated MeasId can be deactivated upon UE receiving the L1 signal deactivating the serving cell DTX/DRX/OFF mode. On the other hand, if the serving cell DTX/DRX/OFF mode will not be activated/deactivated through the L1 signal, a deactivated MeasId can be activated upon UE receiving the RRC message configuring the cell DTX/DRX/OFF mode.

The MeasId associated to CondEventA4 can be activated/deactivated upon UE receiving the DCI format 2_X for activating/deactivating the cell DTX/DRX/OFF mode for the serving cell.

The MeasId associated to CondEventA4 can be activated upon UE receiving the RRC message configuring the cell DTX/DRX/OFF mode, if the cell DTX/DRX/OFF mode is not to be activated/deactivated by the L1 signal.

Alternatively, RAN2 can further consider if a deactivated MeasId can be activated autonomously X time units before the occurrence of a cell non-active period, since the UE should be able to determine exactly the arrival time of a cell non-active period, where the value X can be a pre-defined or a configurable value.

The MeasId associated to CondEventA4 can be activated autonomously upon X time units before the serving cell enters a cell non-active or a cell off period.

On the other hand, as a neighbor cell may also activate the cell DTX/DRX/OFF mode after being configured by the serving cell as a CHO candidate cell for a UE, it is better if the serving cell can inform the UE of whether a neighbor cell is or will be operating in these NES modes in a dynamic manner. With such information, the UE will become able to dynamically adjust the CHO execution condition (e.g., activating or deactivating the MeasId associated to condEventA4) for the neighbor cell, based on the whether the neighbor cell is/will be activating the NES mode.

The network can inform the UE of which neighbor cell(s) is/are or will be operating in the cell DTX/DRX/OFF mode. FFS on the signaling.

Since it has been agreed in RAN2 that the use case of turning off the cell can trigger a CHO, and that the time-based CHO is not applicable in NES, the UE will need to know when the serving cell is going to turn off the cell in order to trigger the CHO in time. We believe the current signal framework used to configure and activate/deactivate a cell DTX/DRX mode can be reused to inform the UEs of an upcoming cell-off event, as the cell-off mode is merely a special case of the cell DTX/DRX mode. Of course, it should be possible for the cell-off mode to be activated automatically upon being configured by the RRC message, without being explicitly activated by the L1 signal (i.e., DCI format 2_X). It should be also possible for the RRC message to configure a one-shot cell-off event (i.e., without periodicity), with or without the duration of the cell-off event being configured in the RRC message.

The serving cell can inform the UE of its cell-off period(s) by using the same signal framework as the cell DTX/DRX mode (i.e. configured by RRC and activated by DCI format 2_X). Send an LS to RAN1 for the feasibility check if needed.

# Conclusions

In this paper, the CHO enhancements for supporting NES are discussed, with the following observation being made.

Observation 1	CondEventA4 with a low threshold value can assure a successful handover before the start of a cell non-active period, but also makes the handover occur sooner than needed.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	The MeasId associated to condEventA4 can be deactivated upon being configured to a NES-capable UE. FFS on how to deactivate the MeasID associated to condEventA4 (e.g., autonomously or explicitly deactivate).

Proposal 2	The MeasId associated to CondEventA4 can be activated/deactivated upon UE receiving the DCI format 2_X for activating/deactivating the cell DTX/DRX/OFF mode for the serving cell.

Proposal 3	The MeasId associated to CondEventA4 can be activated upon UE receiving the RRC message configuring the cell DTX/DRX/OFF mode, if the cell DTX/DRX/OFF mode is not to be activated/deactivated by the L1 signal.

Proposal 4	The MeasId associated to CondEventA4 can be activated autonomously upon X time units before the serving cell enters a cell non-active or a cell off period.

Proposal 5	The network can inform the UE of which neighbor cell(s) is/are or will be operating in the cell DTX/DRX/OFF mode. FFS on the signaling.

Proposal 6	The serving cell can inform the UE of its cell-off period(s) by using the same signal framework as the cell DTX/DRX mode (i.e. configured by RRC and activated by DCI format 2_X). Send an LS to RAN1 for the feasibility check if needed.

# Reference

R2-2307001, Report of 3GPP TSG RAN WG2 meeting #122.

Draft Report of 3GPP TSG RAN WG2 meeting #123.