3GPP TSG-RAN WG2 Meeting #123bis	R2-2309871

Xiamen, China, 9 - 13 October 2023

Agenda item:	7.4.2.2

Source: 	Huawei, HiSilicon

Title: 	Early TA acquisition and LTM MAC CE format

Document for:	Discussion and Decision

- Introduction

L1/L2-triggered mobility (LTM) aims at reducing the latency during cell switch [1]. Main steps are: 1) the network provides candidate cells pre-configuration to the UE, 2) the UE reports L1 measurement results, 3) the network triggers a LTM cell switch MAC CE, and 4) the UE switches to the target cell. In this contribution, we further discuss the format of the LTM cell switch MAC CE.

- Discussion

  - CFRA information in LTM MAC CE

In RAN2 #123 meeting [1], RAN2 agreed that the CFRA resource related information can be included in the LTM cell switch MAC CE.

RAN2 #123: - Will have CFRA resource related information field in LTM cell switch MAC CE (unless serious issues are found).

In the candidate RRC pre-configuration, the candidate cell already can provide CFRA resources to the UE. So the main motivation of having CFRA resource in the LTM MAC CE is to let the target cell dynamically allocate CFRA resource, in order to mitigate the CFRA resource shortages. Note that RAN2 agreed that the Approach 1 is the baseline procedure for inter-DU LTM, i.e., “the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. The gNB-CU then informs the target gNB-DU of the serving cell switch.” In this approach, there is no request and response procedure between the source DU and the target DU immediately before sending the LTM cell switch MAC CE. Therefore, for inter-DU case, it is not preferable to let the source DU request the dynamically allocated CFRA resource from the target DU, which may delay the triggering of the cell switch and incur radio link failure.

Observation 1: In inter-DU case, if the source DU requests the dynamically allocated CFRA resource from the target DU, it delays the triggering of the cell switch and may cause radio link failure on the source cell.

If the target DU provides CFRA resource to the source DU in advance, to mitigate the CFRA resource shortage issue, the provided CFRA resources can be shared by multiple UEs, which is similar with the PDCCH-ordered early RACH design. However, if the UE uses a shared CFRA resource, the target cell does not know the exact UE identity from the preamble transmission. To solve this problem, either the UE reports its identity or the source DU notifies the UE identity to the target DU. If it is the UE who reports its identity, then this makes CFRA like a 4-step CBRA procedure and increases the handover latency. If it is the source DU who notifies the UE identity, then this requires the target cell determine the UE identity only after receiving F1 notification. And the RAR transmission could be delayed and may fall outside the ra-ResponseWindow. This also increases the handover latency.

Observation 2: In inter-DU case, if the candidate DU provides CFRA resources shared by multiple UEs in advance, additional handover delay is expected.

Based on the observations, we propose to support the CFRA resource information in LTM cell switch MAC CE only for intra-DU case.

Proposal 1: The CFRA resource information in LTM cell switch MAC CE is only considered for intra-DU case.

The following information is necessary for CFRA resource:

-	Random Access Preamble index – 6 bits.

-	UL/SUL indicator – 1 bit.

-	SSB index – 6 bits.

-	PRACH Mask index – 4 bits.

As shown in the following extracted RAN1 LS [2], RAN1 agreed that the beam indication is always present, even for RACH-based cell switch. But RAN1 still FFS on the UE behaviour for the beam indication field for the RACH-based handover scenario after cell switch. So whether the beam indication in the LTM cell switch MAC CE can be used as the SSB index in CFRA resources. Further RAN1 input is needed.

Agreement On top the confirmed working assumption, on the presence of beam indication within cell switch command, at least for scenario 2 following is supported: - A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command. - FFS UE behaviour for the beam indication field for the RACH-based handover scenario after cell switch command

Proposal 2: The following is introduced as the CFRA resource related information in LTM cell switch MAC CE (like the information in the legacy PDCCH order triggered CFRA RACH):

-	6 bits Random Access Preamble index,

-	1 bit UL/SUL indicator,

-	6 bits SSB index, FFS whether this SSB index can be replaced by the TCI state ID in the LTM MAC CE.

-	4 bits PRACH Mask index, which refers to the legacy RACH-ConfigCommon in target RRC configuration.

  - UE based TA measurement

In RACH-less cell switch, the UE needs to obtain a valid TA towards the target cell. RAN1 agreed the UE based TA measurement and RAN4 analysed the accuracy for this method and sent a LS R4-2314455, as extracted below:

UE based TA measurement: Existing time alignment error (TAE) (from TS 38.104) is provided below. For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.. Due to TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution error and TA adjustment error, though UE may be able to derive the TA, the actual UL Rx timing error at the target gNB may be larger than CP and may cause performance degradation at the gNB. However, in some specific scenarios, for example, in FR1, where the TAE between serving cell and candidate cell is within 260ns, UE may be able to derive the TA based on UE based TA measurement and may meet the UL transmit timing requirements under good SNR condition and may not cause any performance degradation at the gNB. Note: TAE of 260ns corresponds to Intra-band contiguous carrier aggregation, where the CC are collocated.

It seems that the UE based TA measurement is feasible in some specific scenarios from RAN4 perspective. According to the following RAN1 agreement, we imply that whether to enable this method is configured by the RRC. And the UE will use the obtained TA during cell switch and start the TA timer for the target cell.

RAN1 #113 Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

Proposal 3: For a UE supporting UE based TA measurement, whether to enable the UE based TA measurement is configured by RRC. Once supported and configured by RRC, such UE applies the measured TA upon LTM cell switch and starts the TA timer for the target cell (to be captured in MAC spec).

For a UE configured with UE based TA measurement, there is no need to indicate TA related information or RACH-less indication in the LTM cell switch MAC CE, because the UE can use the measured TA and the UL grant to access to the target cell.

Proposal 4: For the UE based TA measurement case, there is no additional field in LTM cell switch MAC CE to indicate the UE to perform RACH-less LTM (i.e. only based on RRC configuration).

  - Same TA case

In LTM, there is a special case that the UE can perform RACH-less cell switch without PDCCH-ordered early RACH or UE based TA measurement, i.e., when the target PCell is one of the current serving SCells, or the target PCell is collocated with one of the current serving cells. To perform RACH-less, the UE needs to be aware of that a particular cell switch belong to the above same TA cases. The network can indicate the exact TA value to the UE, because the network has the source serving cell TA as well. Another way is that the network uses an explicit indication to indicate that this time cell switch belongs to same TA case, and the UE can directly re-use the current DL synchronization and UL synchronization state. This could be helpful to perform fast synchronization to the target cell. As for the explicit indication, a source TAG ID could be used, which is similar to the LTE RACH-less TA indication.

Observation 3: UE being aware of the “same TA value as the source” could be helpful to perform fast synchronization to the target cell (e.g. in CA cell swap scenario).

Proposal 5a: Introduce the explicit indication for “same TA value as the source” case in the LTM cell switch MAC CE (i.e. not implicitly indicated by TA value). It is up to RAN1/RAN4 on the UL/DL sync requirement for such case.

Proposal 5b: Use source TAG ID as the explicit indication for “same TA value as the source” case.

- Conclusion

In this contribution, we have the following proposals:

Observation 1: In inter-DU case, if the source DU requests the dynamically allocated CFRA resource from the target DU, it delays the triggering of the cell switch and may cause radio link failure on the source cell.

Observation 2: In inter-DU case, if the candidate DU provides CFRA resources shared by multiple UEs in advance, additional handover delay is expected.

Observation 3: UE being aware of the “same TA value as the source” could be helpful to perform fast synchronization to the target cell (e.g. in CA cell swap scenario).

CFRA information in LTM MAC CE

Proposal 1: The CFRA resource information in LTM cell switch MAC CE is only considered for intra-DU case.

Proposal 2: The following is introduced as the CFRA resource related information in LTM cell switch MAC CE (like the information in the legacy PDCCH order triggered CFRA RACH):

-	6 bits Random Access Preamble index,

-	1 bit UL/SUL indicator,

-	6 bits SSB index, FFS whether this SSB index can be replaced by the TCI state ID in the LTM MAC CE.

-	4 bits PRACH Mask index, which refers to the legacy RACH-ConfigCommon in target RRC configuration.

UE based TA measurement

Proposal 3: For a UE supporting UE based TA measurement, whether to enable the UE based TA measurement is configured by RRC. Once supported and configured by RRC, such UE applies the measured TA upon LTM cell switch and starts the TA timer for the target cell (to be captured in MAC spec).

Proposal 4: For the UE based TA measurement case, there is no additional field in LTM cell switch MAC CE to indicate the UE to perform RACH-less LTM (i.e. only based on RRC configuration).

Same TA case

Proposal 5a: Introduce the explicit indication for “same TA value as the source” case in the LTM cell switch MAC CE (i.e. not implicitly indicated by TA value). It is up to RAN1/RAN4 on the UL/DL sync requirement for such case.

Proposal 5b: Use source TAG ID as the explicit indication for “same TA value as the source” case.

- References

- RAN2 #123 Chair notes.

- R1-2308625, LS on L1 measurement and TA management for LTM.

- R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM.