3GPP TSG-RAN WG2 Meeting #123bis	R2- 2309939

Xiamen, China, 9th – 13th October, 2023

Agenda Item:	7.12.2.1.2

Source: 	Lenovo

Title: 	Mobility enhancements for mobile IAB-node and its connected UE

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, mobility enhancements for mobile IAB-node and its connected UEs are discussed, including aspects related to CHO and some other aspects.

- Discussion

Conditional handover for served UEs

Agreements from RAN2 119bis-e: 1) message withholding by the logical source IAB-DU with conditional delivery, e.g., upon on MT migration, 2) conditional execution by the UE based on, e.g., a broadcast indication such as SIB indication of service time or DCI indication of MT-migration, (includes CHO with new trigger). 3) legacy CHO (with implementation specific behaviour, e.g. using source-cell power down or target cell power up triggering the actual HO) RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

For Option 1, RRC Reconfiguration messages are firstly sent to source IAB-DU of mobile IAB-node, and the source IAB-DU withholds the RRC Reconfiguration messages until some specific conditions are met and then delivers the RRC Reconfiguration messages to each connected UEs. Because similar delay RRC configuration procedure has been defined in RAN3 during R17 for reconfiguration of descendent nodes during intra-donor migration, and the Option 1 has no impacts on legacy UEs. However, for the deliver condition, MT migration may not feasible as defined in RAN3 that IAB-MT handover and IAB-DU migration is executed independently.

For Option 2, because it’s not supported by the legacy UEs, then this option can be deprioritized.

For Option 3, legacy CHO configuration can be fully reused for UEs, and source IAB-DU can power down the cells by implementation to trigger the served UEs perform CHO to target IAB-DU, which has fewest standards impacts among 3 options. While for power up the cells of target IAB-DU, it’s not recommended that it causes additional unnecessary HO/CHO on other UEs.

Proposal 1: Legacy CHO procedure can be reused by the served UEs of mobile IAB-node, and source IAB-DU powers down the cells by implementation to trigger the CHO.

Agreements from RAN2 121bis-e: FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

Currently, CHO with CondT1 is only applied to NTN UE. While in the mIAB scenario, the motivation to introduce the CHO with CondT1 is to spread out the handover of served UEs during mIAB-DU migration, in order to alleviate the signaling storm for large amounts of handover UEs. However, during IAB-DU migration, both cells of source logical IAB-DU and target logical IAB-DU are activated at the same time, and the time to handover the served UEs of mIAB-node is determined by the source F1-terminating IAB-donor. Then, it can be left to the source IAB-donor’s implementation to distribute the time of the handover among all served UEs, there is no need to introduce CHO with CondT1 for mobile IAB.

Proposal 2: No need to introduce time-based CHO for served UEs of mIAB-node during DU migration, and source IAB-donor can spread out the UE handover via implementation.

On-board UE status identification

RAN2 has agreed that RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node. It means that the target IAB-donor-CU needs to be aware of the on-board/surrounding status of handover UE and may needs to make some optimizations for the on-board UE than the surrounding UE. Therefore, on-board/surrounding status of UE can be included in the handover request message.

Proposal 3: In case of UE handover during IAB-DU migration, to include on-board/surrounding status of UE in the handover request message.

In addition, for the on-board/surrounding status, all onboard UEs needs to perform migration together with the mobile IAB-node. While for the surrounding UEs camping on the mobile IAB-node, migration together with the mobile IAB-node is not an optimal choice. Once the surrounding UEs perform migration together with the mobile IAB-node, the measurement result of serving cell become worse gradually due to the mobility of mobile IAB-node, and the surrounding UEs may need to perform handover again within a short time.

In addition, as specified in the WID, no optimizations are for the targeting of surrounding UEs, which means all enhancements related to group mobility are only considered for the onboard UEs. In addition, we have agreed that RACH-less handover may be only considered for on-board UEs rather than surrounding UEs. Therefore, it’s necessary to differentiate onboard UEs and surrounding UEs for IAB-donor. And no matter how UE determine whether it’s in the status of onboard or surrounding, UE needs to report its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Proposal 4: UE reports its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Fixed IAB-node handover

For the mobile IAB-node, it should have no descendent IAB-nodes and only serve for UEs as defined in the WID. In RAN2 119 e-meeting, RAN2 has agreed that the method of not broadcasting “iab-Support” indication is sufficient to prevent other IAB-node from accessing mobile IAB-node. However, the “iab-Support” indication is only applied for IAB-node integration procedure, it needs to avoid that a source IAB-donor-CU to configure measurement to an IAB-node with a potential cell belonging to a mobile IAB-node, and the source IAB-donor-CU to trigger handover of the IAB-node to the mobile IAB-node cell. Therefore, the source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

Proposal 5: Source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: Legacy CHO procedure can be reused by the served UEs of mobile IAB-node, and source IAB-DU powers down the cells by implementation to trigger the CHO.

Proposal 2: No need to introduce time-based CHO for served UEs of mIAB-node during DU migration, and source IAB-donor can spread out the UE handover via implementation.

Proposal 3: In case of UE handover during IAB-DU migration, to include on-board/surrounding status of UE in the handover request message.

Proposal 4: UE reports its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Proposal 5: Source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN2 119bis-e – 123 meetings.