3GPP TSG-RAN WG2 Meeting #123b	R2- 2309954

Xiamen, China, 9th Oct – 13th Oct, 2023

Agenda item:		7.20.2

Source:		Lenovo

Title:		Discussion on the UE-initiated RACH procedure in multi-TRP operation

Document for:		Discussion and Decision

# Introduction

The related discussion has been kicked off and some agreements have been achieved in RAN2#123 [1].

Agreement

Each joint/UL TCI state is associated with either TAG1 or TAG2 by RRC configuration.

RAN2 do not assume any restriction on grouping serving cells/TRPs to TAGs unless RAN1 indication comes.

RAN2 assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs.

At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells.

At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell).

Working assumption:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

    - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

    - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

For inter-cell PDCCH order CFRA to the additionalPCI,

- PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement),

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

In this contribution, we discuss the cases for obtaining of the two TAs in multi-DCI multi-TRP operation.

# Discussion

In legacy, a RACH is initiated by a PDCCH order, by the MAC entity itself, or by RRC. A RACH can be initiated to obtain the TA when the UL data arrive at the UE in RRC_CONNECTED state while the UL is unsynchronized. Considering multi-DCI multi-TRP operation, the UE shall be in the RRC_CONNECTED state. Referring to the legacy procedure to maintain the UL TA, RRC configures the timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned. Also, the gNB can determine whether the TA is maintained according to the timeAlignmentTimer in legacy. Since two TAs are introduced in multi-TRP operation, the timeAlignmentTimer per TAG can be introduced correspondingly in order to flexibly control the UL time alignment.

One TA of two TAs is valid

It has been agreed in RAN1 that for multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Then, the gNB can trigger a PDCCH ordered CFRA to update the TA for the TRP if the gNB identifies that the uplink time for one TRP is not aligned and determines to recover the UL time alignment.

Observation 1: The gNB can trigger the UE to obtain the TA for the unsynchronized TRP via PDCCH order.

Considering each of the UL channels is configured to associate with TRP(s), it is natural to manage the UL channels per TRP. Then, once one TRP is unsynchronized, the corresponding configuration shall be cleared referring to the legacy behavior. While considering the HARQ buffers, it is not associated with TRP directly. If one TAT is running, the data in the HARQ buffer can be transmitted via dynamically scheduling the HARQ process associated with the TRP with running TAT.

Proposal 1: When one TAT is expired and the other TAT is running, 3-6 are applied to the TRP with TAT expired.

Proposal 2: When one TAT is expired and the other TAT is running, do not flush HARQ buffers.

Neither TA is valid

RACH procedure can be initiated by UE when both TATs to two TAs expire, and UL data arrives. In RACH procedure, a preamble will be transmitted in a RACH association related to one TRP. Then, the serving cell can calculate the TA value for the TRP receiving the preamble. In one RACH, the TA for the second TRP cannot be obtained. Therefore, if UE wants to keep synchronization towards two TRP, two RACH procedures are needed. However, the DL/UL transmission can be resume once one of two TRP can be recovered. The first issue is which TRP should be recovered first if both TRPs are unsynchronized.

Observation 2: one RACH procedure can be used to recover only one TRP.

Proposal 3: RAN2 to discuss which TRP should be recovered first when RACH procedure is triggered because of UL data arrival and uplink time not aligned.

# Conclusion

In this contribution, we propose the followings for obtaining of the two TAs in multi-DCI multi-TRP operation in Rel-18 NR:

Observation 1: The gNB can trigger the UE to obtain the TA for the unsynchronized TRP via PDCCH order.

Proposal 1: When one TAT is expired and the other TAT is running, 3-6 are applied to the TRP with TAT expired.

Proposal 2: When one TAT is expired and the other TAT is running, do not flush HARQ buffers.

Observation 2: one RACH procedure can be used to recover only one TRP.

Proposal 3: RAN2 to discuss which TRP should be recovered first when RACH procedure is triggered because of UL data arrival and uplink time not aligned.

# References

RAN2#123 chairman notes.