3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310003

Xiamen, China, 9th – 13th October, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues of the RRC CR for NES

Agenda Item:	7.3.1

Document for:	Information

# Introduction

In this contribution RRC open issues for NES are discussed based on the previous online discussions and companies’ inputs to the current running RRC CR email discussion.

# Discussion on open issues

## Cell DTX/DRX

Issue 1-1: Configuration of cell DTX/DRX per serving cell vs. per MAC entity.

During the [POST123][312] email discussion [1] it was discussed whether Cell DTX/DRX configuration status is per MAC entity or per Serving Cell, which has implications on the 38.331 and 38.321 specifications.

The summary of this discussion was following:

18 companies responded to this question, with 10 supporting per cell configuration and 7 supporting per MAC entity, one company was not sure. The rapporteur would like to highlight some arguments from the provided responses:

Per cell

| pros | cons |
| --- | --- |
| Flexible per cell configuration on the NW side, load balancing of active time between cells possible | Significant increase of UE complexity, possibly several sets of cell DTX/DRX timers and active durations of the MAC entity for reception and transmission need to be maintained for each serving cell. |
| Implicit activation for the single configured cell | Difficult alignment of all cell DTX/DRX patterns with one C-DRX pattern |
| Resource configuration of SPS, SR and CG is per cell |  |

Per MAC entity

| pros | cons |
| --- | --- |
| Simplified UE implementation (on top of C-DRX, which is per MAC entity) | All serving cells for this UE must have the same pattern (cannot handle variable load/configurations) |
| Similar as C-DRX configuration, which is maintained per MAC entity | Unclear activation status after RRC configuration (are all serving cells implicitly cell DTX/DRX activated) |
| PDCCH is monitored per C-DRX |  |

The following general points were also made:

- RAN1 has agreed that “UE monitor DCI format 2_X in one serving cell”, so it is assumed the configurations would be already limited to having similar parameters.

- The issue with configuration type arises only in CA scenarios. It is not predicted to be a common scenario and reasonable configuration that a PCell and different SCells would run their own misaligned cell DTX/DRX patterns. The patterns should be fully aligned but can be activated/deactivated on a per cell basis still maintaining NW flexibility.

- If the UE has one serving cell, there is no difference in the configuration method.

- Regardless of the solution, the maximum number of parallel Cell DTX/DRX patterns a UE is required to follow should be restricted.

Proposal 1: RAN2 to decide whether the configuration of cell DTX/DRX is per serving cell or per MAC entity.

Issue 1-2: Explicit vs implicit activation based on RRC and relation to L1 activation (how UE derives NW L1 capability and is there a need of explicit RRC indication that the NW will use L1).

It has been commented during the email discussion that there might be a need to have a bit in the cell DTX/DRX configuration saying that it is configuration only and the activation will be triggered by L1.

The rapporteur’s understanding is that there is a parameter positionInDCI-cellDTRX from RAN1 which will be implemented later in RRC. The presence of this parameter explicitly means that the NW supports and intends to use the L1 indication. If it is absent it is clear that L1 will not be used.

Adding 1 bit in RRC to explicitly activate a configuration (or not) is contrary to the current agreement on implicit activation. We think the feature currently works in a way that the NW configures and activates implicitly by RRC and then after the initial RRC configuration the NW can use L1 to deactivate and activate this configuration.

Proposal 2: RAN2 to discuss explicit RRC indication of activation or deactivation upon configuration of Cell DTX/DRX now that we have introduced L1 signalling.

Issue 1-3: Is deactivation by RRC performed only by deconfiguration, or should we have an explicit RRC deactivation without deconfiguration option (to later activate the same configuration by RRC).

Similarly to the abovementioned issue RAN2 can discuss the need of explicit deactivation.

Proposal 3: RAN2 to discuss whether we should introduce a deactivation bit, to deactivate a configuration without deconfiguration.

Issue 1-4: Is the stage 3 alignment description needed.

Based on the following agreement:

Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other.   FFS if we anything needs to be specified in stage 3 (i.e. in IE description)

The alignment has been specified in the stage 2 specs. The remaining issue is whether the relation between cell DTX/DRX and C-DRX periodicities should also be captured in the IE description of 38.331.

Proposal 4: RAN2 to discuss whether the relation between cell DTX/DRX and C-DRX periodicities should be captured in the IE description of 38.331.

Issue 1-5: Whether the start offset (and slot offset) configuration should be the same for cell DTX and cell DRX.

Based on the following agreement:

On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured.  FFS if we have different start offset configuration for cell DTX and cell DRX

The remaining issue is whether start offset and slot offset can be different or should be the same between cell DTX and DRX configurations.

Proposal 5: RAN2 to discuss whether start offset and slot offset can be different or should be the same between cell DTX and DRX configurations.

Issue 1-6: Whether standalone cell DRX is needed.

During the study item phase it was concluded that cell DTX has significantly higher NES gains compared to cell DRX. If the start offset is decided to be the same and based on the following agreement:

Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX)

The rapporteur’s understanding is that we can remove the explicit cell DRX configuration and rely on 1 bit in the cell DTX config to signal whether cell DRX is configured simultaneously or not.

Proposal 6: RAN2 to discuss whether standalone cell DRX configuration can be excluded.

Issue 1-7: Whether the fields in cellDTX-config should be optional or mandatory.

It was commented during the email discussion that for UE C-DRX only short DRX is optional and other fields are mandatory, which structure should be also used for cell DTX/DRX. The rapporteur’s understanding is that in making them optional we wanted to enable changing e.g. only one parameter without the need of signalling all of them. To clarify this scenario we can add a description to the parameter, e.g. ”if the parameter is not present, the UE should follow the previously stored one.”.

Proposal 7: RAN2 to discuss whether the fields in cellDTX-config should be optional or mandatory.

Issue 1-8: Consider a simplification of cell DTX/DRX RRC configuration IE once issues 1-5, 1-6 and 1-7 are resolved.

During the email discussion there were several proposals to simplify the current structure of cellDTX-config and cellDRX-config. The rapporteur is open to those suggestions when the open issues on the contents of the configurations are resolved.

Proposal 8: RAN2 to discuss a simplified structure of cellDTX-config and merging with cellDRX-config.

Issue 1-9: Whether to capture in RRC that the focus was on the case where cell DTX in RRC can only be configured when C-DRX is configured.

RAN2 agreed to focus on the case where DTX in RRC can only be configured when C-DRX is configured.  The feature is not optimized for the case where C-DRX is not configured. The remaining issue is whether to capture this information in stage 3 specs.

Proposal 9: RAN2 to discuss whether to capture in stage 3 that the focus was on the case where cell DTX in RRC can only be configured when C-DRX is configured.

Issue 1-10: Whether we specify impact to the dual C-DRX case (SCell with different onDuration/inactivity timers).

The secondary C-DRX may have different on duration timer and inactivity timer compared with primary C-DRX in CA between FR1 and FR2. When cell DTX/DRX is activated, the NW will change the behaviour of scheduling to obtain the NES gain. The UE will follow the behaviour we defined for cell DTX/DRX regardless of whether it is for primary or secondary C-DRX. As RAN2 defined alignment as a condition between UE C-DRX cycle and cell DTX/DRX cycle and the secondary C-DRX has the same cycle as the primary one there does not seem to be any issue with aligning the configurations. Considering that and the agreement not to optimise for the CA case we think there is no spec impact of cell DTX/DRX on secondary C-DRX.

Proposal 10: RAN2 to discuss whether there is specification impact of the dual C-DRX case on cell DTX/DRX.

Issue 1-11: Whether multiple configurations of cell DTX or DRX will be supported.

The current design assumes a single configuration of cell DTX/DRX. If the NW wants to change the configuration parameters of cell DTX or cell DRX, it can reconfigure it by a RRCReconfiguration message.

RAN2 should resolve the FFS from RAN2#121:

Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

Proposal 11: RAN2 to discuss whether the baseline of a single configuration of cell DTX/DRX can be confirmed.

Issue 1-12: Whether to change cellBarredNES to a single ENUM value and general procedures for NES UEs.

During the email discussion [1] there were following suggestions regarding NES cell barring behavior:

- Change cellBarredNES to a single ENUM value because there is no valid use case to set it to barred since the NES UE would respect the MIB barring if cellBarredNES is absent.

Proposal 12: RAN2 to discuss whether to change cellBarredNES to a single ENUM value {notBarred}.

- The current implementation of cellBarredNES (as highlighted below) implies that the NES UE would always bar the cell if the cell is barred in MIB, regardless of the SIB1 check.

2>	if the cellBarred in the acquired MIB is set to barred:

3>	if the UE is a RedCap UE, or a NES-capable UE, and ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]):

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	consider the cell as barred in accordance with TS 38.304 [20];

Proposal 13: RAN2 to discuss alternative wording for sections 5.2.2.4.1 and 5.2.2.4.2.

## SSB-less Scell operation

Issue 2-1: SSB-less SCell operation impact on the RRC specification.

Currently only impact identified for inter-band SSB-less is in the absoluteFrequencySSB field (“same frequency band” is currently mentioned). For further 331 spec impacts more discussion is needed.

Proposal 14: RAN2 to discuss SSB-less SCell operation impact on the RRC specification.

## Cell barring for NES

Issue 3-1: What “NES-capable UE” means.

In the last RAN2 meeting, it was agreed to use a single bit in SIB1 for controlling “NES-capable UEs” to access a cell. The remaining issue is what the “NES capable UE” bit means, whether the definition includes all UEs supporting any of the Rel-18 NES techniques, or only specific techniques that have impact on legacy UEs.

Proposal 15: RAN2 to discuss what is a NES-capable UE in the cell barring context.

## CHO enhancements

Issue 4-1: Signalling for triggering NES specific CHO execution condition.

RAN2 has discussed some candidate solutions about the exact signalling for triggering CHO execution condition, but has not concluded on the final option. RAN2 needs to continue discussing the CHO trigger signalling.

Proposal 16: RAN2 to discuss and decide on the signalling for triggering NES specific CHO execution condition.

Issue 4-2: Configuration details for the NES specific CHO execution condition (e.g. whether to add a new offset/threshold or flag to existing CHO events, or add a separate list of MeasIds for NES CHO events).

RAN2 has agreed to have the NES specific CHO execution condition. How to implement it in the configuration is not decided.

RAN2 should resolve the FFS from RAN2#122:

2. We can have a specific NES CHO execution condition based on source cell NES mode. FFS how the UE determines is in NES mode. FFS on how this is achieved in RRC

Proposal 17: RAN2 to discuss and decide on the configuration details for the NES specific CHO execution condition.

Issue 4-3: Whether target cell NES mode is considered for CHO enhancements.

RAN2 agreed to have a CHO solution that considers NES mode of at least source cell. The target cell is not decided yet.

Proposal 18: RAN2 to discuss and decide whether target cell NES mode is considered for CHO enhancements.

## Other general issues

Issue 5-1: Implementation of RAN1 parameter list (currently incomplete).

It will be implemented by the RRC rapporteur and reviewed after RAN1 provides a more complete list.

Issue 5-2: Implementation of UE capabilities.

It will be implemented by the UE capability rapporteur and reviewed after RAN1 provides a more complete list.

# Conclusion

In this contribution, RRC open issues for NES were discussed. The following proposals were made:

Proposal 1: RAN2 to decide whether the configuration of cell DTX/DRX is per serving cell or per MAC entity.

Proposal 2: RAN2 to discuss explicit RRC indication of activation or deactivation upon configuration of Cell DTX/DRX now that we have introduced L1 signalling.

Proposal 3: RAN2 to discuss whether we should introduce a deactivation bit, to deactivate a configuration without deconfiguration.

Proposal 4: RAN2 to discuss whether the relation between cell DTX/DRX and C-DRX periodicities should be captured in the IE description of 38.331.

Proposal 5: RAN2 to discuss whether start offset and slot offset can be different or should be the same between cell DTX and DRX configurations.

Proposal 6: RAN2 to discuss whether standalone cell DRX configuration can be excluded.

Proposal 7: RAN2 to discuss whether the fields in cellDTX-config should be optional or mandatory.

Proposal 8: RAN2 to discuss a simplified structure of cellDTX-config and merging with cellDRX-config.

Proposal 9: RAN2 to discuss whether to capture in stage 3 that the focus was on the case where cell DTX in RRC can only be configured when C-DRX is configured.

Proposal 10: RAN2 to discuss whether there is specification impact of the dual C-DRX case on cell DTX/DRX.

Proposal 11: RAN2 to discuss whether the baseline of a single configuration of cell DTX/DRX can be confirmed.

Proposal 12: RAN2 to discuss whether to change cellBarredNES to a single ENUM value {notBarred}.

Proposal 13: RAN2 to discuss alternative wording for sections 5.2.2.4.1 and 5.2.2.4.2.

Proposal 14: RAN2 to discuss SSB-less SCell operation impact on the RRC specification.

Proposal 15: RAN2 to discuss what is a NES-capable UE in the cell barring context.

Proposal 16: RAN2 to discuss and decide on the signalling for triggering NES specific CHO execution condition.

Proposal 17: RAN2 to discuss and decide on the configuration details for the NES specific CHO execution condition.

Proposal 18: RAN2 to discuss and decide whether target cell NES mode is considered for CHO enhancements.

# References

- R2-2310001 Report of [POST123][312][NES] Running CR 38.331 (Huawei)