3GPP TSG-RAN WG2 Meeting #123bis														R2-2310088

Xiamen, China, October 09-13, 2023

Agenda item:		7.11.3

Source:				Samsung

Title:					Shared processing for broadcast and unicast reception

Document for:		Discussion & Decision

# 1 Introduction

The following agreements were made in previous RAN2 meeting:

- R2-123: - As per the previous agreement, if the UE is able to get the additional information (i.e. its current configuration does not prevent it from doing so), the UE shall do this (if capable and configured by the network) - In case additional information (SCS, bandwidth) is not available at the time of sending the MII to the unicast serving cell (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE reports whatever is available information at that time (i.e. at least the frequency, and optionally SCS and/or BW as available). - UE reports updated MII after acquiring additional information from the non-serving cell (if previously it reported only frequency) or if the information in the non-serving cell changes. - The SCS in the MII is set to the SCS of the CORESET#0 for the MBS broadcast cell. - Combination of FreqBandIndicatorNR and ARFCN-ValueNR is used to signal the frequency information in the MII for shared processing. - At least CFR bandwidth is reported by the UE in MII. FFS whether “location” needs to be also reported and how exactly this is captured in RRC (i.e. which IE is used).

This contribution discusses the open issues and proposals for shared processing for broadcast and unicast.

# 2 Discussion

## 2.1 Rel-18 MII - Frequency of interest determination

Procedure for frequency of interest determination considers a frequency to be part of MBS frequencies of interest if certain conditions are fulfilled. For Rel-18 MII reporting enhancement, the procedure needs to be amended to consider a condition that “at least one of the MBS sessions is from non-serving cell for the concerned frequency included in SIB21 and/or USD from the non-serving cell”. This is an alternative condition for the frequency of interest determination on the basis of PCell’s SIB21 or USD in the legacy specification. The reason is that the PCell is not required to be supporting SIB21 and USD transmission, i.e. PCell is not required to support MBS. SIB21 and/or USD may be from non-serving cell.

Proposal 1: For Rel-18 MII reporting enhancement, MII procedure for frequency of interest determination is amended to consider a condition that at least one of the MBS sessions is from non-serving cell for the concerned frequency included in SIB21 and/or USD from the non-serving cell (Adopt TP#1 in annex).

## 2.2 Rel-18 MII - CFR location reporting?

In the R2#123 meeting, it was discussed but could not be concluded whether location needs also to be reported and how exactly it is captured in RRC specification. Majority companies’ views are aligned with reporting of the CFR bandwidth. However, concern raised is that in some cases the UE may not have CFR BW of the non-serving cell. We undertand that when the other information are not available to the UE for the non-serving cell, UE may report only the frequency information. Given that the frequency information is reported, location along with CFR BW is not really needed, as the location can be interpreted from the frequency information.

Proposal 2: For Rel-18 MII reporting, UE reports CFR bandwidth (number of PRBs) without the location information.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposals:

Proposal 1: For Rel-18 MII reporting, MII procedure for frequency of interest determination is amended to consider a condition that at least one of the MBS sessions is from non-serving cell for the concerned frequency included in SIB21 and/or USD from the non-serving cell (Adopt TP#1 in annex).

Proposal 2: For Rel-18 MII reporting, UE reports CFR bandwidth (number of PRBs) without the location information.

# 4 Annex (TP to TS 38.331)

## [Base document: 38.331 v17.5.0]

## 4.1 Text Proposal # 1

#### 5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session or at least one of these MBS sessions is from non-serving cell for the concerned frequency included in SIB21 and/or USD from the non-serving cell; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies, the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.