3GPP TSG-RAN WG2#123bis	R2-2310195

Xiamen, China, 9-13 October 2023

Agenda Item:	7.2.2

Source: 	MediaTek Inc.

Title:  	SLPP information forwarding

Document for:	Discussion, decision

# 1	Introduction

This document discusses the potential need for “forwarding” (in some sense) of SLPP signalling.

After the downscoping discussion at RAN#101, it is less obvious if forwarding is required.  However, as analysed in section 2.1 below, we understand that it is still necessary for the SA2 flows with an LMF even for the in-coverage case, and potentially needed/beneficial also when a server UE is used.

# 2	Discussion

## 2.1	What is actually required

The SA2 design in [1] for sidelink positioning operation is different when an LMF is involved as compared to a server UE:

- When the target UE can establish a NAS connection (directly or via a UE-to-network relay), the LMF communicates only with the target UE ([1], sections 5.5.2, 5.5.4, 6.5.1, 6.5.3), and the target UE is responsible for communication with the anchor UEs.

- When one or more anchor UEs can establish a NAS connection but the target cannot, the LMF communicates with the anchor UEs in coverage ([1], sections 5.5.3, 5.5.5, 6.5.2, 6.5.4), and it appears that these anchor UEs are responsible for communication with the target UE and any anchor UEs out of coverage.

  - In case of UE-based sidelink positioning where the target UE lacks a NAS connection, the LMF may delegate computation to a server UE ([1], section 6.5.4).

  - In case of UE-based sidelink positioning using SL-MO-LR where the target UE has a NAS connection, section 6.5.3.1 of [1] seems to imply that a server UE can be used, but the referred flow in section 6.20.1 of [2] does not contain a server UE, and it is not completely transparent how to apply this procedure.

- In UE-only operation with a server UE ([1], section 6.8), many of the details of UE interactions have been left to RAN WGs.  In particular, the description of figure 6.8.1-1 of [1] tends to use the passive voice: “Sidelink Positioning assistant[sic] data is transferred among UE1/ …/UEn and the SL Positioning Server UE” in step 6, “SL-PRS measurement data is transferred to the SL Positioning Server UE” in step 8, etc.

  - It is not clear what the detailed signalling flow was intended to be, and in particular, it seems possible either to have the server talk to many different UEs separately or to have a single “chokepoint” UE through which the server routes all communication with the other UEs.

- When a server UE is used, the target UE can always communicate directly with the server UE ([1], section 5.2.3).

The first item in the list implies that forwarding is needed even for the in-coverage scenario with an LMF; that is, the downscoping at RAN#101 of the sidelink positioning objective does not eliminate the need for forwarding.  (This is in addition to UE-only operation, where the downscoping did not address whether all UEs are assumed to be in direct communication on PC5.)

The second bullet, however, is inapplicable after downscoping, because the target can always establish a NAS connection.

We infer the following principles if the SA2 procedures are to be followed as written:

- The target UE (with a NAS connection) must be able to transfer SLPP information between the LMF and an anchor UE.

- An anchor UE (with a NAS connection) does not need to transfer SLPP information between the LMF and either another anchor UE or the target UE.  (This is a result of the downscoping.)

- The target UE (when a server UE is in use) may be able to transfer SLPP information between the server UE and an anchor UE.

  - If the target UE cannot do this, the operation may need to proceed with only those anchor UEs that can communicate directly with the server UE.

  - We understand it is within RAN2 remit to decide if this transfer operation is mandatory on the target UE.

- An anchor UE (when a server UE is in use) may be able to transfer SLPP information between the server UE and another anchor UE.

  - If an anchor UE cannot do this, it may limit the set of anchor UEs that can be used in a sidelink positioning operation.

  - We understand it is within RAN2 remit to decide if this transfer operation is mandatory on the anchor UE.

We suggest that since the transfer behaviour is already required for the target UE (principle 1 above), it may not be a problem to require it also for the target UE when a server UE is used.  However, it needs to be discussed whether the same requirement would be levied on anchor UEs.

Proposal 1: The target UE is required to support transfer of SLPP information between the server (UE or LMF) and an anchor UE.  FFS if anchor UEs have the same requirement.

We refer to this transfer functionality as “forwarding” for this paper, but it should be understood that there is no proposal to relay an SLPP PDU as such.  As discussed in sections 2.3 and 2.4, we consider mechanisms that forward specific portions of the SLPP message rather than encapsulating an entire PDU.

For the remainder of this paper, we will discuss forwarding in a somewhat role-agnostic way, i.e., considering similar behaviour for target and anchor UEs.

## 2.2	Relation to UE roles and transaction types

### 2.2.1	Capabilities

The capability transaction at least takes place between a server (LMF or UE) and a non-server UE type (target or anchor).  As articulated in section 2.1, the server UE and the target UE should always be able to communicate directly, but the server and anchor may not have direct communication.  Thus, a target UE (and potentially an anchor UE, for the server UE case only) should be able to perform forwarding of the Request Capabilities (server to anchor UE out of contact) and the Provide Capabilities (anchor UE out of contact to server), as shown in figure 1.

Figure 1: Forwarding of SLPP capabilities

We do not foresee requirements to forward the capability information in any other scenario.  If the target can request capabilities from (candidate) anchor UEs, it should do so only with UEs that it can communicate with directly; there is no benefit from selecting anchor UEs that the target UE cannot see.  However, a requirement on the target or anchor to forward the contents of the capability messages might be written in a way agnostic to the UE roles—e.g., the server might include an IE in the Request Capabilities indicating “this message is directed to UE X”, and the receiving UE would be expected to construct a Request Capabilities towards UE X containing the forwarded material, irrespective of the role of UE X.  We address this possibility further in sections 2.3 and 2.4.

Proposal 2: The Request Capabilities and Provide Capabilities messages can be forwarded.

### 2.2.2	Assistance data

SA2 are vague about how assistance data are used.  There are two possible models:

- An SL-PRS Rx UE requests assistance data from the server (LMF or UE).  The server collects assistance data (e.g., from the SL-PRS Tx UE(s)) and sends it to the SL-PRS Rx UE(s); or

- An SL-PRS Rx UE requests assistance data directly from an SL-PRS Tx UE.

#### 2.2.2.1	Model 1

Model 1 is more “LPP-like” and allows the server to know the SL-PRS configurations, which would affect the achievable QoS.  If the assistance data transaction is reused for the server to gather assistance data from the SL-PRS Tx UE(s), then model 1 requires two transactions for the full assistance data life cycle, as shown in figure 2.

Figure 2: Assistance data model 1

In this view, either the SL-PRS Rx UE or the SL-PRS Tx UE may be out of direct contact with the server and require forwarding.  Depending on the positioning method, the Rx and Tx UEs may be either target or anchor; it follows that the assistance data messages can be forwarded (agnostic to the target/anchor roles).

Proposal 3: If assistance data model 1 is supported, the Request Assistance Data and Provide Assistance Data messages can be forwarded.

#### 2.2.2.2	Model 2

Model 2 is decentralised and gives the server a reduced role.  It does not necessarily expose the assistance data to the server; for instance, in “downlink-like” (SL-PRS anchors => target) SL-TDOA positioning, the assistance data would go directly from each anchor to the target, to support measurement, as shown in figure 3.

Figure 3: Basic operation of assistance data model 2

This means that either the measurements must be carefully constructed to allow the server to compute a location estimate without knowing the assistance data, or the assistance data must be forwarded to the server.  It is not exactly clear how the latter operation would work.  If the SL-PRS Rx UE is in direct contact with the server, it can forward the Provide Assistance Data message, but if it is not in direct contact with the server, some additional procedure is needed to send the assistance data to the server—such as an unsolicited Provide Assistance Data “pushed” to the server, as shown in figure 4.

Figure 4: Assistance data model 2 with forwarding

Further discussion seems necessary on whether model 2 is supported, and if so, whether forwarding is needed.  In this case, however, there seems to be no need to forward the Request Assistance Data message.

Proposal 4: RAN2 further discusses whether assistance data model 2 is supported, and if so, whether the Provide Assistance Data message can be forwarded.

### 2.2.3	Location information

Location information needs to be delivered from the SL-PRS Rx UE(s) to the server.  Some Rx UEs may be out of direct communication with the server, so it seems straightforward to have forwarding of both the request and response messages, as shown in figure 5.

Figure 5: Forwarding of location information

Proposal 5: The Request Location Information and Provide Location Information messages can be forwarded.

### 2.2.4	Other transaction types

In accordance with the proposals in [3], we assume that there are three additional transaction types in SLPP: Server UE Selection, Error, and Abort.  It seems clear that the Server UE Selection signalling would never need to be forwarded—as noted above, the target and server UEs should always be in direct communication.

Proposal 6: The Server UE Selection Indication and Server UE Selection Confirm messages cannot be forwarded.

The Error message seems useful to forward.  For example, a Request Location Information originating from the server and forwarded to an SL-PRS Rx UE (as in figure 5 above) may cause an error at the SL-PRS Rx UE, and the SL-PRS Rx UE should be able to indicate an error to the server that originated the request.

Proposal 7: The Error message can be forwarded.

The Abort message requires further discussion to understand if an ongoing procedure that might require an abort, such as periodic location information reporting, can take place through forwarding.

Proposal 8: RAN2 further discusses whether the Abort message can be forwarded.

## 2.3	Procedural requirements

Any SLPP message whose contents can be forwarded needs to contain a forwarding flag and an associated destination, as well as the actual information for forwarding.  In some cases, a single UE may need to forward information for multiple peers (e.g., the target UE forwarding measurements from multiple anchor UEs as well as itself to the server), as shown in figure 6, and the forwarded information should be batched together for signalling efficiency.

Figure 6: Forwarding of multiple Provide Location Information messages

The figure shows measurement results originated by each of the three anchors (red/green/blue), along with measurement results from the target itself (yellow), all of which are batched into a single Provide Location Information message for forwarding to the target.  To prevent sending four individual messages to the server, the target can run a timer when the first message arrives, collect information while the timer runs, and send the batched message when the timer expires.

The procedural requirements in pseudocode might be described as follows:

- If the received message contains the “forward” flag:

  - Extract the information to be forwarded to a given destination UE (IEs dependent on message type);

  - Store the extracted information and the identity of the source UE in a new entry in a UE variable scoped to a particular destination UE;

  - Start a timer Txxx, if not already running.

- When Txxx expires:

  - For each stored instance of the information to be forwarded to a given destination UE (red/green/blue in the figure):

    - Include the stored information in the outgoing message;

    - Include the identity of the source UE in the outgoing message;

  - Include any information for the same destination UE from the UE itself (yellow in the figure);

  - Submit the outgoing message to lower layers for transmission.

## 2.4	ASN.1 encoding

The ASN.1 coding for the mechanisms described in section 2.3 can be realised in multiple ways.  In the simplest example, the SLPP-Message structure can contain a forwarding flag (the optionality bit for the field in the structure shown below) and a destination address:

-- ASN1START

-- TAG-SLPP-MESSAGE-START

SLPP-Message ::=            SEQUENCE {

transactionID           SLPP-TransactionID,

endTransaction          BOOLEAN,

sequenceNumber          SequenceNumber,

sessionID               SessionID,

acknowledgement         Acknowledgement     OPTIONAL,

forwardingDestination	NodeIdentity		OPTIONAL,

slpp-MessageBody        SLPP-MessageBody    OPTIONAL,

nonCriticalExtension    SEQUENCE {}         OPTIONAL

}

SequenceNumber ::= INTEGER (0..255)

SessionID ::= INTEGER (0..FFS)

Acknowledgement ::= SEQUENCE {

ackRequested        BOOLEAN,

ackIndicator        SequenceNumber        OPTIONAL

}

-- TAG-SLPP-MESSAGE-STOP

-- ASN1STOP

In this view, the entire SLPP-MessageBody could be forwarded to the destination indicated.  Alternatively, each message type could contain its own forwarding indication:

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

c1                      CHOICE {

requestCapabilities    RequestCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

forwardingDestination				NodeIdentity	OPTIONAL,

commonIEsRequestCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

method-A-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-A-RequestCapabilities

method-B-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-B-RequestCapabilities

method-C-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-C-RequestCapabilities

nonCriticalExtension                 SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

(Mutatis mutandis for other message types.)

This accounts for the needed indication to forward a message.  In the forwarded message, the recipient needs an indication of what the original source is:

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

c1                      CHOICE {

requestCapabilities    RequestCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

forwardingSource					NodeIdentity	OPTIONAL,

forwardingDestination				NodeIdentity	OPTIONAL,

commonIEsRequestCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

method-A-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-A-RequestCapabilities

method-B-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-B-RequestCapabilities

method-C-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-C-RequestCapabilities

nonCriticalExtension                 SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

To allow batch forwarding as described above, the message should be modified to contain a list of entries for the message contents:

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

c1                      CHOICE {

requestCapabilities    RequestCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

requestCapabilitiesList				SEQUENCE (SIZE (1..maxNrofSLPP-Messages)) OF RequestCapabilitiesEntry

}

RequestCapabilitiesEntry ::= SEQUENCE {

forwardingSource					NodeIdentity	OPTIONAL,

forwardingDestination				NodeIdentity	OPTIONAL,

commonIEsRequestCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

method-A-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-A-RequestCapabilities

method-B-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-B-RequestCapabilities

method-C-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-C-RequestCapabilities

nonCriticalExtension                 SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

(The undefined NodeIdentity type is briefly discussed below.)

In principle, it would be possible to “go to a lower level”, e.g., have the common and method-specific RequestCapabilities IEs contain forwarding information.  However, this approach has more complex impact to the ASN.1 than the approaches already described, and it poses some threat to the separate compilation principle—in this view, for a UE to forward the method-A-RequestCapabilities, the UE must compile in the method A portion of the ASN.1, even if it is not itself being used for a method A positioning operation.  It is not absolutely clear if this is a real problem, but it seems possible, for instance, that the server selects a hybrid positioning method (method A + method B) and involves some UEs in each method, without necessarily using the same set of UEs for both methods.  In such a scenario, a UE supporting only method A could be asked to forward information to a UE supporting method B, and the “lower level” approach does not support such an operation.

On balance, we suggest that the second approach described above is to be preferred.

Proposal 9: For SLPP messages that can be forwarded, the “-IEs” structure for each message contains a list of entries with forwarding information provided for each entry.

In the ASN.1 examples above, the source and destination identities are given the undefined type “NodeIdentity”.  This is intended to be a type containing a generic SLPP source or destination identity, i.e., a UE or an LMF.  It can be further discussed exactly how this type would be structured, but it seems plausible to have a CHOICE structure between a UE identity and a NULL value representing the LMF.

Proposal 10: RAN2 further discusses how to identify the source and destination of a forwarded SLPP message.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: The target UE is required to support transfer of SLPP information between the server (UE or LMF) and an anchor UE.  FFS if anchor UEs have the same requirement.

Proposal 2: The Request Capabilities and Provide Capabilities messages can be forwarded.

Proposal 3: If assistance data model 1 is supported, the Request Assistance Data and Provide Assistance Data messages can be forwarded.

Proposal 4: RAN2 further discusses whether assistance data model 2 is supported, and if so, whether the Provide Assistance Data message can be forwarded.

Proposal 5: The Request Location Information and Provide Location Information messages can be forwarded.

Proposal 6: The Server UE Selection Indication and Server UE Selection Confirm messages cannot be forwarded.

Proposal 7: The Error message can be forwarded.

Proposal 8: RAN2 further discusses whether the Abort message can be forwarded.

Proposal 9: For SLPP messages that can be forwarded, the “-IEs” structure for each message contains a list of entries with forwarding information provided for each entry.

Proposal 10: RAN2 further discusses how to identify the source and destination of a forwarded SLPP message.

# 4	References

[1]	TS 23.586

[2]	TS 23.273

[3]	R2-2310194, “SLPP signalling and procedures”, MediaTek Inc., RAN2#123bis