3GPP TSG-RAN WG2 Meeting #123bis	R2-2310212

# Xiamen, China, October 09-13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Temporary CR] [RAN1 lead features] UE capabilities for Rel-18 eRedCap WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Define the UE capabilities for Rel-18 eRedCap WI on RAN1 lead features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the UE capabilities for Rel-18 eRedCap WI on RAN1 lead features. NOTE 1: TP for FG 48-1 and 48-2 is based on RAN1 LS R1-2308523 on the latest updates of the RAN1 feature list (available in R1-2308521). It is highlighted in pink, the TP which is updated to use RAN2 terminology of eRedCap when applicable. NOTE 2: For the email discussion, it is higlighted in blue, the TP that shows the differences between FG 48-1 and 48-2; and in green, the TP associated to the following RAN2#123 agreement “to add in the list of functional components for the supportOfEnhancedRedCap-r18 the support of eRedCap early indication based on Msg3 and MsgA PUSCH”. NOTE -3: When TP is stable, the tempCR will be updated with the latest version of the TS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 eRedCap WI is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.331 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Modified section

## 4.2	UE Capability Parameters

*** OMITTED TEXT ***

### 4.2.x	eRedCap Parameters

*** OMITTED TEXT ***

#### 4.2.x.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| eRedCap-r18 Editor’s note: FFS which name to use for the new UE capability considering eRedCap-r18, supportOfEnhRedCap-18 or supportOf-eRedCap-r18. Note: current TP is using the term eRedCap-r18 Editor’s note: FFS whether any of the references to eRedCap needs to be kept (or be even updated e.g. to RedCap or (e)RedCap) in the functional components that describe eRedCap-r18 capability. Indicates that the UE is an eRedCap UE with comprised of at least the following functional components: The following functional compentents are the same as for supportOfRedCap-r17: Editor’s note: FFS whether the following points could be removed from the field description of eRedCap-r18 considering how related functional components are currently captured in supportOfRedCap-r17: (a) it includes the configuration(s) needed for eRedCap UE to perform random access, (b) enabling/disabling of frequency hopping for common PUCCH resources, (c) for separate initial DL BWP used for paging, CD-SSB is included, (d) for separate initial DL BWP only used for RACH, SSB may or may not be included and (e) for separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included. - Maximum FR1 bandwidth is 20 MHz; - Support of early indication based on Msg1 for 4-step RACH; Editor’s note: FFS whether to use the term RedCap, eRedCap or (e)RedCap as part of the following functional component “Support of eRedCap early indication based on Msg1 for 4-step RACH” - Separate initial UL BWP; Editor’s note: FFS what RAN1 referred by “separate” in relation to eRedCap UEs. - It includes the configuration(s) needed to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP; Editor’s note: FFS what RAN1 referred by “separate” in relation to eRedCap UEs. - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included Editor’s note: FFS what RAN1 referred by “option 1”. - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. The following functional compentents are new compared to supportOfRedCap-r17: - DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 3.2. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. Editor’s note: FFS whether not to remove “target” from the functional component that use the term “peak data rate target of 10Mbps - Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. - Relaxed RAR-PDSCH processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is only applicable during initial access and contention based random access. Editor’s note: FFS whether “contention based random access” should be added in relation to the following functional component of “Relaxed RAR-PDSCH processing timeline” to indicate that is only applicable during initial access and contention based random access when eRedCap UE also supports notReducedBB-BW-r18 - Network-configurable separate early indication in Msg1. Editor’s note: FFS whether not to remove “additional” from the following functional component, i.e. “Network-configurable additional separate early indication in Msg1 for eRedCap UEs". - Support of eRedCap early indication based on MsgA PUSCH, if UE indicates the support of twoStepRACH-r16, and Msg3. Editor’s note: FFS whether to add additional components (update dependent to RAN1 input). An UE supporting this feature shall set this field to supported but shall not indicate support of supportOfRedCap-r17. | UE | CY | No | FR1 only |
| eRedCapNotReducedBB-BW-r18 Editor’s note: FFS which name to use for the new UE capability considering nonReducedBB-BW-r18, eRedCapNotReducedBB-BW-r18 or eRedCapReducedBB-BW-r18. Note: current TP is using the term eRedCapNotReducedBB-BW-r18 Indicates that the UE is an eRedCap UE without reduced baseband bandwidth in FR1. DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2. UE supporting this feature shall indicate the support of eRedCap-r18. Editor’s note: FFS whether to change how to capture the field description when UE supports both eRedCap-r18 and notReducedBB-BW-r18 considering e.g., option 1) field description of eRedCap-r18 captures which components are not applicable when UE also supports notReducedBB-BW-r18, or option 2) duplication of the applicable components is done in order to capture which components of eRedCap-r18 are applicable when UE also supports notReducedBB-BW-r18. In addition, it is also discussed whether the related TP is added as part of the field description of notReducedBB-BW-r18 or eRedCap-r18). Note: current TP is follows option 1). | UE | No | No | FR1 only |

End of the modified section