3GPP TSG-RAN WG2 Meeting #123bis	 R2-2310236

Xiamen, China, 09th – 13th Otc, 2023

Agenda Item:	7.15.2

Source: 	TCL

Title: 	Remaining issue on SL Consistent LBT failure

Document for:	Discussion and decision

Introduction

In RAN2#123 [1], the agreements related to SL-U are as follows:

Agreements on SL C-LBT failure recovery (RRC connected mode 2)

1: 	C-LBT failure recovery for RRC idle/inactive mode 2 is applied.

Agreements on C-LBT failure cancellation conditions

1: 	Upon MAC reset.

2:	Upon C-LBT count and/or timer reconfiguration.

3:	Based on a timer expiry (the timer starts upon C-LBT failure)

Agreements on C-LBT failure with PSFCH

1: 	Remove “FFS when multiple PSFCH occasions are configured.” from the following agreement.

“Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.”

2:	RAN2 understands LBT failure indication can be provided from L1 once each transmission fails per PSFCH occasion in multiple PSFCH occasions.

Agreements on C-LBT failure with S-SSB and/or PSFCH

1: 	RAN2 will not do anything to handle S-SSB and/or PSFCH that does not belong to the selected SL resource pool unless RAN1 asks.

6. Need of reporting C-LBT failure indication to the peer UE ? (P5,6: 8375: IDC, P9: 7478: ZTE, P19: 7956: Lenovo)

- Noted. We’ll make decision next meeting.

Agreements on SL C-LBT failure MAC CE

1: 	RAN2 understands 5bits indication per SL carrier. Will ask how RB set index is derived, whether RB set index is unique within SL-BWP, to RAN1.

2:	LCP order of SL LBT Failure MAC CE is defined as the next of Uu LBT Failure MAC CE.

3: 	Dedicated SR configuration can be configured. FFS if we need to consider more.

Agreements on SL C-LBT failure detection and recovery counter and timer

1:	Configured per SL BWP.

2: 	Maintained per RB set.

Agreements on SL RLF report as the result of C-LBT failures

1:	No new code-point or a cause value is needed.

In this contribution, we will discuss whether SL C-LBT failure needs to be reported to the peer UE.

Discussion

Motivation for SL C-LBT failure to be reported to the peer UE.

If a SL UE who encounters C-LBT failure, it will not be able to transmit the SL data through shared spectrum to the peer UEs. In this case, it should perform resource re-selection, which will exclude the RB sets that LBT failure was detected, or carrier re-selection. The peer UE will keep monitoring the resources where C-LBT failure happened unless it is notified by the peer UE that the C-LBT was already detected. The peer UE will cause unnecessary power consumption and may cause extra delay.

Observation 1: the peer UE should be notified for the C-LBT failure.

However, so far the only mechanism to notify the peer UE is the PC5 RRC connection, so it only works for the unicast case. So the BC and GC cases won’t work as there is no PC5 unicast connection between peer UEs. On the other hand, as one RB set is detected for the C-LBT failure, all RB sets will not be available, so there also should be other resource pools for the transmission of C-LBT Failure.

Observation 2: C-LBT failure notification only works for unicast case.

Observation 3: C-LBT failure notification only works when other resource pool is configured.

In addition, when the peer UE select the resource pool, if it received the C-LBT failure indication, it will consider the C-LBT failure result for resource re-selection, otherwise there is no need to send C-LBT failure to the peer UE. But this will change the behavior of resource re-selection, which should no be touched without clear discussion and benefit.

Observation 4: C-LBT failure notification will change the resource re-selection behavior of the peer UE.

It is possible that the C-LBT failure is canceled in the future, due to the some certain conditions, e.g. timer expiry. After the RX UE sends C-LBT failure indication to the peer UE, the TX UE should not be able to use the RB set indicated in the C-LBT failure. But when the C-LBT failure is canceled, there should be a new mechanism to be introduced to make sure the proper C-LBT failure cancellation.

Observation 5: there should be a new mechanism to be introduced to make sure the proper C-LBT failure cancellation.

Due to the above discussion, we see that the sending a C-LBT failure indication to the peer UE has some benefit but there are some challenge against the solution, we may need more time to go through all details of the solution, thus we suggest not to implement this solution in Rel_18 SL, and postpone it to Rel_19 SL.

Proposal 1: not to pursue the solution of C-LBT-F indication to peer UE in Rel_18 SL.

Conclusion

In this contribution, we discussed the solution of C-LBT-F indication to peer UE, and we have the following observations and propose not to pursue the solution of C-LBT-F indication to peer UE in Rel_18 SL:

Observation 1: the peer UE should be notified for the C-LBT failure.

Observation 2: C-LBT failure notification only works for unicast case.

Observation 3: C-LBT failure notification only works when other resource pool is configured.

Observation 4: C-LBT failure notification will change the resource re-selection behavior of the peer UE.

Observation 5: there should be a new mechanism to be introduced to make sure the proper C-LBT failure cancellation.

Proposal 1: not to pursue the solution of C-LBT-F indication to peer UE in Rel_18 SL.

Reference

RAN2#123 Meeting report