3GPP TSG-RAN WG2 Meeting #123bis	R2-2310336

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.14.3

Source:	Apple

Title:	Views on Way Forward of Buffer Level Threshold Based RVQoE Reporting

WID/SID:	NR_QoE_enh-Core

Document for:	Discussion / Decision

# Introduction

During RAN3 #118, it has agreed that buffer level can be used for threshold-based RVQoE reporting. Moreover, in RAN3 #120 it was further agreed that RVQoE reporting can be triggered when the buffer level is either higher or lower than a threshold.

RAN3 #118 Agreements: Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting. RAN3 #120 Agreements: Agreements: The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

To support this feature, RAN2 think this should be triggered by Application layer and have asked SA4 to specify it in the LS R2-2302042. Initially, SA4 has replied in R2-2304658 (S4-230684) that it is feasible. However, as SA4 investigates the issue more deeply, they found that it is quite doubtful how buffer level as such could be useful as an indicator for the RAN to know that some radio resource optimization is needed for application performance [1]. The reply LS from SA4 also shows they would prefer to have a more sensible alternative for this purpose.

In light of this, RAN3 has suspended the work on buffer level threshold based RVQoE reporting, and decided to wait for more progress from SA4:

RAN3 #121 Agreements: The discussion on Threshold-based triggers for RAN visible QoE is suspend in RAN3 and whether further discussion is needed is pending on progress in RAN2 and SA4.

In the latest reply LS from SA4 [2], SA4 has informed RAN2 and RAN3 that they have not identified any buffer threshold-based RVQoE reporting mechanism. Additionally, they do not see any value in further pursuing this work. This paper provides our views on how RAN2 should handle this situation.

# Discussions

It is our understanding that RAN3 intended to introduce buffer level threshold based RVQoE reporting based on two motivations:

- As compared to periodic RVQoE reporting in Rel-17, buffer-level threshold based reporting allows the UE to report RVQoE only when this is deemed to be “useful”, which reduces the Uu interface overhead since the UE does not have to report it frequently.

- Periodic RVQoE reporting may cause some “reporting delay” as the UE may only report it at specific timings, which is not desirable if some timely radio resource optimization is needed to improve application performance.

Using buffer level threshold as a trigger for RVQoE reporting is based on the following assumption:

- Application performance (or user experience) is closely coupled with the buffer level. In particular, a low buffer level indicates a bad user experience, while a high buffer level indicates a good user experience.

Nevertheless, such assumption may be a misconception from SA4 point of view. In particular, according to [1], it is clear that SA4 does not think buffer level own its own can serve as an appropriate metric to indicate whether radio resource optimization by RAN is needed or not. To cope with the situation, SA4 has even started to conceive an alternative approach that would make more sense in their perspective. The reply from SA4 in [1] is shown below:

S4-231119 / R2-2307074 [1] : SA4 replied to RAN2 on their LS on buffer level threshold-based RVQoE reporting in S4-230684, confirming RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. However, in SA4#124 meeting, SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, e.g. - A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content. - Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level. Hence based on this, SA4 would like to update the reply sent in S4-230684, and does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. SA4 is considering appropriate alternatives for this purpose. SA4 requests RAN2 and RAN3 to take this information in account, and that SA4 will keep RAN2 and RAN3 updated on progress in this direction.

SA4 has then finally sent another reply LS to both RAN2 and RAN3 [2]:

S4-231582 [2]: In SA4 reply LS to RAN2 and RAN3 on their LS on buffer level threshold-based RVQoE reporting in S4-231119, SA4 informed that appropriate alternatives for application layer triggering of buffer level threshold-based RVQoE reporting are being considered in SA4. SA4 has not identified any buffer level threshold-based RVQoE reporting mechanism and does not see any value in working further in this direction. SA4 would also like to point to the network assistance feature related to this problem in TS 26.501. RAN2 and RAN3 are requested to take this information in account.

By looking at [1] and [2] together, we can summarize the conclusions from SA4 as following:

- Buffer level is not an appropriate RVQoE information.

- SA4 does not see any value in working further in the direction of defining buffer threshold based RVQoE reporting.

Moreover, in [2] SA4 has pointed out that some existing mechanisms are already available in TS 26.501, which allows the application to boost performance (if needed) by seeking the assistance from the network. This implies that buffer threshold based RVQoE reporting is probably redundant.

Observation: From SA4 perspective, buffer level threshold based RVQoE reporting is not useful and not needed.

Since Application is the ultimate customer for most of the QoE features we are defining in RAN, it is questionable why RAN2/RAN3 need to specify a feature that is not so useful/needful from SA4 point of view. On the other hand, since we are approaching the end of Rel-18, it is more important to make sure the other Rel-18 QoE features are properly specified by using the available time budget wisely. With these considerations, we think there is no need for RAN2 to further work on buffer threshold based RVQoE reporting, unless RAN3 is still keen to work on this.

Proposal: RAN2 does not specify buffer level threshold based RVQoE reporting in Rel-18, unless RAN3 requests RAN2 to do so.

# Conclusions

In this contribution, we have discussed the way forward regarding buffer threshold based RVQoE reporting, with the following observation:

Observation: From SA4 perspective, buffer level threshold based RVQoE reporting is not useful and not needed.

Based on such observation, we propose:

Proposal: RAN2 does not specify buffer level threshold based RVQoE reporting in Rel-18, unless RAN3 requests RAN2 to do so.

# References

[1] S4-231119, Reply LS on buffer level threshold-based RVQoE reporting, 3GPP TSG-SA WG4 Meeting #125, Gothenburg, 21-25 August 2023.

[2] S4-231582, Reply LS on buffer level threshold-based RVQoE reporting, 3GPP TSG-SA WG4 Meeting #125, Gothenburg, 21-25 August 2023.