3GPP TSG-RAN WG2 Meeting #123bis	R2-2310380

Xiamen, China, October 9th – 13th, 2023

Source: 			OPPO

Title: 	Consideration on RAT-dependent positioning integrity

Agenda item:		7.2.3

Document for: 	Discussion and Decision

# Introduction

In the RAN2#121, RAN2#121b-e meeting and last RAN2#122 meeting, agreements regarding the RAT-dependent integrity have been made as follows:

Agreements:

- RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

- TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or PosSIB

- Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope. Other aspects of determining the TRP error sources are left to deployment and implementation.

- For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signaling

More agreements have been reached in the RAN2#121b-e meeting, as indicated as follows:

Agreements:

- LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

- LS to RAN1 to include the question of whether RAN1 identify a need for a DNU flag for measurements.

- For RAT-dependent integrity, the PL calculation is performed by the entity which also performs the position calculation for a location process.

- For UE-based integrity, the integrity parameters of error sources for RAT-dependent integrity are included in assistance data.

LPP Request/Provide Assistance Data are reused for retrieving the integrity parameters to the UE from the LMF. The request is per positioning method (as in legacy operation) and the provided integrity parameters are as appropriate for the selected positioning method.

Use of posSIBs for integrity parameters is not excluded.

- Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

- Working assumption:

For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

- Capture the stage 2 impact for RAT-dependent integrity in section 7 of 38.305.  Initial running CR to be seen at next meeting, using R2-2302504 and R2-2303682 as baseline.

Agreements:

- For stage-2 description of RAT-dependent integrity, move the section of “Integrity Principle of Operation” to a generic section that is not specific to positioning methods.

- Represent the TRP and ARP location errors by a Gaussian paired over-bounding

- Represent the RTD errors by a Gaussian paired over-bounding

In the last RAN2#123 meeting, agreements regarding the RAT-dependent integrity have been made as follows:

Agreements:

- The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc) in a new IE NR-Integrity-ServiceAlert.

- DNU flags for TRP/UE positioning measurements are not needed

- The ‘integrity Correlation Times’, defining the minimum timer interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data

- It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results provided to the LMF from UE and/or NG-RAN.

- The beam related information (Beam Bore-Sight direction/Beam Antenna Information) are error sources for DL-AOD positioning. FFS if RAN2 support signaling this information.

- For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

In this paper, we would like to further address our views on the RAT-independent integrity.

# Discussion

## 2.1 Residual risk for the RTD error source

According to the current positioning integrity CR R2-2309181, the residual risk related parameters, probOnsetTRPFault IE and meanTRPFaultDuration IE have been defined for the TRP/ARP location fault as follows:

NR-TRP-IntegrityParametersforResidualRisk

This field defines the and comprises the following sub-fields:

-	probOnsetTRPFault: This field specifies the Probability of Onset of TRP/ARP location Fault per Time Unit which is the probability of occurrence of TRP/ARP location error to exceed the residual error bound for more than the Time to Alert (TTA).

This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE NR-Integrity-ServiceParameters.

The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetTRPFault and the range is 10-10.2 to 1 per hour.

- meanTRPFaultDuration: This field specifies the Mean TRP/ARP location Fault Duration. Scale factor 1 s; range     1-3600 s.

We think that, similarly, the residual risk related parameters, probability of onset, and the mean fault duration should be defined for the RTD error source also.

Proposal 1: RAN2 to agree to capture the residual risk related parameters for the RTD error source in the TS 38.355.

## 2.2 Whether or not to support the mode 2 integrity reporting?

In the TR 38.859, the positioning integrity signalling impacts on the UE-based positioning signalling have been found as follows:

UE-based:

- UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure.

- LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

- LMF sends integrity requirement, e.g., TIR, to UE in LPP request location information message for integrity of UE-based mode.

-	UE sends integrity result to LMF using LPP location information Transfer message.

In the RAN2#121b-e meeting, it was agreed that the integrity parameters of error sources for RAT-dependent integrity are included in assistance data. With such information and the integrity parameters related to the measurement result (UE implementation to decide or is provided by the network), the UE could calculate the PL. However, it is FFS whether or not UE-based integrity should support mode 2 of integrity result report, i.e., UE compares the calculated PL with a given AL and indicates whether or not the positioning system is available for integrity operation. Note that in R17, the mode 2 is not supported for the GNSS positioning integrity, wherein the network does not need to send the AL and the TTA towards the UE, and the UE only needs to send the PL towards the network and subsequently the network could further identify the availability of the positioning integrity. We find there is no reason why we should not follow to use the mode 1 for reporting the GNSS positioning integrity result. Note that for the mode 2, the UE needs to consume additional computation resource for comparing the PL with the AL and determine if the event last for a time longer than the TTA. As a result, we propose RAN2 to agree that the only the mode 1 positioning integrity report should be supported and therefore only integrity parameter TargetIntegrityRisk should be provided towards the UE in the LPP RequestLocationInfomration msg for computation of the PL.

Proposal 2: RAN2 to agree that the only the mode 1 positioning integrity report needs to be supported for the UE-based positioning integrity, and therefore only integrity parameter TargetIntegrityRisk should be provided towards the UE in the LPP RequestLocationInfomration msg for computation of the PL.

## 2.3 the usage of the assistance data attached with the DNU flag

Regarding the DNU for the assistance data, in the last RAN2 meeting, it was agreed that the DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc) in a new IE NR-Integrity-ServiceAlert, and DNU flags for TRP/UE positioning measurements are not needed. And in the RAN2#120 meeting, a related agreement has been made as follows:

Agreement:

Proposal 1 (modified): Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Signaling details and relation to error sources can be determined in normative work. FFS which positioning methods are affected based on the progress in RAN1.

Firstly, we think we need to further discuss the scenario of when DNU flag is set, the concerned assistance data is not used for integrity calculation but maybe useable for positioning. As well known, the positioning result is derived based on the positioning measurement results and the TRP locations. For instance, for the DL-TDOA, the performance of the inter-TRP synchronization and the accuracy of the TRP location will contribute to not only the positioning accuracy but also the uncertainty of the positioning result. in our opinion, there are two kinds of interpreting upon reception of an assistance data with the DNU flag:

- Option 1: a positioning result with the positioning integrity result set to unavailable to be provided in the LPP ProvideLocationInformation msg to the LMF for further providing result to the LCS with positioning integrity requirement

- Option 2: no positioning result should be provided for the LPP session corresponding to the LCS with positioning integrity requirement, but positioning result is allowed to be provided for the LPP session corresponding to the LCS without integrity requirement

In our opinion, the Option 2 seems incorrect, since the assistance data received in the LPP ProvideAssistanceData msg corresponds to a particular LPP session for which session ID is indicated in the NAS layer. UE should not use the assistance data for positioning for other LPP session without integrity requirement.

Observation 1: the assistance data received in the LPP ProvideAssistanceData msg corresponds to a particular LPP session and should not be used for other LPP session without integrity requirement.

Proposal 3: RAN2 to confirm that in case that the assistance data attached with DNU flag is used for positioning, the provided positioning result sent to LMF should be attached with a flag indicating the positioning integrity result is unavailable.

## 2.3 positioning integrity for the DL-AOD

In the last RAN2 meeting, it was agreed that the beam related information (Beam Bore-Sight direction/Beam Antenna Information) are error sources for DL-AOD positioning. FFS if RAN2 supports signalling this information.

It could be found from the TS 37.355 that beam bore-sight directions of the DL-PRS resources are defined in the NR-DL-PRS-BeamInfo IE. Especially, each DL-PRS resource angle is represented with an azimuth and elevation angle, with the finest granularity 0.1 degree. Obviously, the integrity bounding related parameters could be defined in terms of the mean value and standard deviation of the error of the DL-PRS resource angle. In addition, the beam antenna information defines a more precise DL-PRS propagation pattern. The propagation power of each DL-PRS resource for each angle could be retrieved from the related NR-TRP-BeamAntennaInfo IE. The integrity bounding related parameters could be defined in terms of the mean value and standard deviation of the power error for each DL-PRS for each angle. Furthermore, the residual risk related parameters should be defined for the beam bore-sight direction and beam antenna information. Finally, how to set such mentioned integrity parameters should be up to network implementation, if RAN2 agrees to support signalling the integrity parameters for the DL-AOD positioning method.

Proposal 4: RAN2 to agree to support signalling the integrity bounding parameters related to NR-DL-PRS-BeamInfo and NR-TRP-BeamAntennaInfo IE and the residual risk related parameters in the assistance data for the DL-AOD positioning, if RAN2 agrees to support signalling the integrity parameters for the DL-AOD positioning method.

Proposal 5: RAN2 to agree it is up to network implementation how to set the integrity parameters related to the DL-AOD positioning method.

# Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following observations and proposals:

Proposal 1: RAN2 to agree to capture the residual risk related parameters for the RTD error source in the TS 38.355.

Proposal 2: RAN2 to agree that the only the mode 1 positioning integrity report needs to be supported for the UE-based positioning integrity, and therefore only integrity parameter TargetIntegrityRisk should be provided towards the UE in the LPP RequestLocationInfomration msg for computation of the PL.

Observation 1: the assistance data received in the LPP ProvideAssistanceData msg corresponds to a particular LPP session and should not be used for other LPP session without integrity requirement.

Proposal 3: RAN2 to confirm that in case that the assistance data attached with DNU flag is used for positioning, the provided positioning result sent to LMF should be attached with a flag indicating the positioning integrity result is unavailable.

Proposal 4: RAN2 to agree to support signalling the integrity bounding parameters related to NR-DL-PRS-BeamInfo and NR-TRP-BeamAntennaInfo IE and the residual risk related parameters in the assistance data for the DL-AOD positioning, if RAN2 agrees to support signalling the integrity parameters for the DL-AOD positioning method.

Proposal 5: RAN2 to agree it is up to network implementation how to set the integrity parameters related to the DL-AOD positioning method.