3GPP TSG-RAN WG2 Meeting #123bis	R2-2310633

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.4.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On Failure Handling for Rel-18 LTM

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

According to the agreements from the previous meeting [1]:

R2-123 Agreements: Upon an LTM cell switch failure (i.e., supervision timer expiry) or RLF, fast recovery similar to CHO: - a) UE performs cell selection. - b) If selected cell is an LTM candidate cell, UE performs RACH-based LTM cell switch on the selected cell (network-controlled). - c) If selected cell is not an LTM candidate cell, UE transmits RRC re-establishment request. UE shall release all LTM-related configurations upon going to RRC_IDLE. Upon RRC re-establishment, the UE handles the LTM related configuration similar to the CHO configurations. For the handling of LTM-related configurations in RRC_INACTIVE the UE applies the same principles as CHO ( = conditions/triggers to release configurations).

In this paper the above issues are discussed. Specifically, how the UE should behave in case of fast recovery (LTM Recovery). Additionally, further aspects on UE behavior in case of Beam failure are being discussed.

# 2	Failure Handling when the UE is configured with LTM

## 2.1	Failure handing when the UE is configured with LTM

As agreed in RAN2#123 if a failure occurs and the UE is configured accordingly the UE will perform cell selection and if the selected cell is configured for LTM, the UE will perform cell switch to the selected cell.

Regarding the TA value of the selected cell upon the cell switch two options exist:

- The UE has the TA value due to local measurements as possibly agreed in RAN1 (and subsequently supported by RAN4)

- The UE has not received the cell switch command this it does not have the TA value for any cell.

Observation 1: The UE may have the TA value of the target cell in case it calculates the TA by itself (the option currently being discussed in RAN1).

For the first case, the UE should perform RACH-less LTM to the selected cell since it has the TA value to the target. In case the UE does not have the TA value for the target cell it should perform RACH based access to the selected cell.

Proposal 1: If the UE has the TA value of the target cell it should perform RACH-less to that cell even in case of LTM execution in case of RLF to reduce interruption time at the cell switch. In case the UE does not have the TA value it should perform RACH based access.

Following the procedure described above, the UE will access the target cell using the prepared configurations as in normal LTM, with the only difference that this mobility process possibly is not triggered by the network. This will have the following impact: i) Serving DU will not send the cell change triggering notification to the CU; ii) The UE may not have the TA.

The impact of the above is that:

- path switch will take place with slight delay (if needed),

- target DU will not provide UL resources in case of RACH-less (assuming TA availability) with Dynamic Grant, and

- UE will need to perform RACH in case TA is not available.

These aspects do not have any impact neither in the UE operation nor in the network relevant to the LTM preparations and operation.

Observation 2: UE accessing the target cell in case of LTM recovery does not have any impact in the UE or network operation.

Thus, we can consider that the UE will maintain the LTM preparations for future use in order to reduce signalling for repreparations.

Proposal 2: Upon LTM recovery, the UE maintains the LTM preparations for future use in order to reduce signalling which would be needed for re-preparations.

## 2.2 	BF handling when the UE is configured with LTM

A similar procedure to the case of RLF of HOF could be followed in case of Beam Failure (BF). In that case, the UE would evaluate the beams, not only of the configured BFR resources but for all of the configured beams. If the configured beam belongs to a cell other than the serving, the UE can apply that candidate target cell LTM configuration. The benefit of this approach is that the UE does not desperately attempt to recover to the serving cell and just in case of RLF attempts to be handed over to a non-serving cell, but it evaluates the candidate cells during the beam selection procedure. The second benefit of such approach is that the UE does not remain in bad conditions for the beam selection to take place and in case BFR fails to try to recover to new cell (due to RLF) but it will be more flexible in the beam selection.

Observation 3: The UE should not be waiting for the BFR to fail and end up in RLF as this would lead to additional interruption time compared to recovering using LTM mobility directly to a non-serving cell as part of the BFR process.

The way to operate in case the UE is enabled to recover to non serving cell in case of BFR:

- On beam-failure detection in serving cell, after identifying the beam for recovery in a prepared cell, UE applies the new configuration

- If the UE does not have the TA of the target cell it will perform CFRA access to the prepared cell, similarly to the RLF case.

Proposal 3: In case of Beam Failure the UE evaluates selected beams from subset of candidate cells and if the selected beam for recovery belongs to a candidate LTM cell the UE performs LTM Recovery to that cell.

In order for the UE to be able to perform LTM mobility to non-serving cell in case of Beam Failure (simultaneously to the Beam Selection) using LTM preparations this should be configured in the RRC, similar to the RLF case. This will enable the UE to evaluate the non-serving cell beams at the time of the BFR. For the target cell to identify that this is part of a BFR process and not of an LTM cell switch the UE should indicate this. This can be indicated via MAC CE to the target cell or via RRC reconfiguration signalling. In case the UE indicates this using MAC CE the gNB should be informed respectively – this would require additional RAN3 signalling. I

Proposal 4: Mobility to non-serving cell using LTM preparations in case of Beam Failure is configured in RRC Reconfiguration at the time of LTM preparation.

Proposal 5: The UE will indicate the target cell that it accesses (using LTM mobility) it as a response to BFR process. This can be indicated via MAC CE to the target cell or via RRC reconfiguration signalling (similar to RLF/HOF). RAN2 decides which of the two options is preferred.

Proposal 6: If RAN2 decides that the UE informs the target DU about the Beam Failure using MAC CE, RAN3 should be informed to specify additional CU-DU interactions.

# 3	Conclusion

This document has made the following observations:

Observation 1: The UE may have the TA value of the target cell in case it calculates the TA by itself (the option currently being discussed in RAN1).

Observation 2: UE accessing the target cell in case of LTM recovery does not have any impact in the UE or network operation.

Observation 3: The UE should not be waiting for the BFR to fail and end up in RLF as this would lead to additional interruption time compared to recovering using LTM mobility directly to a non-serving cell as part of the BFR process.

The document has made the following proposals:

Proposal 1: If the UE has the TA value of the target cell it should perform RACHless to that cell even in case of LTM execution in case of RLF to reduce interruption time at the cell switch. In case the UE does not have the TA value it should perform RACH based access.

Proposal 2: Upon LTM recovery, the UE maintains the LTM preparations for future use in order to reduce signalling which would be needed for re-preparations.

Proposal 3: In case of Beam Failure the UE evaluates selected beams from subset of candidate cells and if the selected beam for recovery belongs to a candidate LTM cell the UE performs LTM Recovery to that cell.

Proposal 4: Mobility to non-serving cell using LTM preparations in case of Beam Failure is configured in RRC Reconfiguration at the time of LTM preparation.

Proposal 5: The UE will indicate the target cell that it accesses (using LTM mobility) it as a response to BFR process. This can be indicated via MAC CE to the target cell or via RRC reconfiguration signalling (similar to RLF/HOF). RAN2 decides which of the two options is preferred.

Proposal 6: If RAN2 decides that the UE informs the target DU about the Beam Failure using MAC CE, RAN3 should be informed to specify additional CU-DU interactions.

# References

[1] R2#123 Chairman Notes, R2_123_ChairNotes_EOM