3GPP TSG RAN WG2 Meeting #123bis               	R2-2310760

# Xiamen, China, 9th-13th of October 2023

Agenda Item: 	7.2.4

Source:		Sony

Title:			Considerations on Low Power High Accuracy Positioning

Document for:	Discussion & Decision

# Introduction

A new release-18 work item on Expanded and Improved NR positioning was approved at RANP#98e meeting in December 2022 [1]. The work is basically based on the study item, reported in TR 38.859 ]2].

For Low Power High Accuracy Positioning (LPHAP) the objectives were as follows.

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

At RAN2#122 meeting [9], a number of agreements were made:

Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state.

- The UE starts/restarts the area-specific TA timer when it receives the TA command.

- The UE stops the SRS transmission when the area-specific TA timer expires.

- The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area.

- Other stop/restart conditions can be discussed.

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters.  To be included in the LS from [AT122][415].

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

At RAN2#123 the following agreements were made [13]:

Agreements:

When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. The SRS configuration request is sent in the RRC message RRCResumeRequest (18/18).

WA: A new resume cause is introduced for the above use case.

Periodic SRS is supported to be configured with validity area.  This agreement does not affect preconfigured SRS.

Activation/deactivation is not required for periodic SRS.  This agreement does not affect preconfigured SRS.

Aperiodic SRS is not supported to be configured with validity area.

RAN2 do not further consider providing pre-configured SRS via system information in Rel-18.

For the activation indication and/or request for preconfigured SRSs, RRCResumeRequest message is used, and 1-bit indication in the RRCResumeRequest is introduced for this use.

WA: The resume cause introduced for the SRS configuration request can be reused for the activation indication of the pre-configuration SRS.

Sending the activation indication and/or requesting for preconfigured SRS using Msg1 is not supported.

Agreement:

Semi-persistent SRS is supported to be configured with validity area, and RAN2 agree to reuse legacy mechanism to deactivate the SP SRS

Agreements:

At least alignment of PRS to fixed (e)DRX is supported.

At least UE-initiated on-demand PRS request procedure is supported for the alignment of the PRS configuration to the fixed (e)DRX configuration.

UE may utilize the positioning assistance data through posSIB or assistance data received in RRC_CONNECTED when UE is to perform positioning in RRC_IDLE.  No stage 3 impact is foreseen.

The following criterion needs to be defined for the start/re-start of the area-specific TA timer:

	Reception of RRCRelease message containing the SRS configuration (excluding pre-configured SRS)

The following criteria need to be defined for the stop of the area-specific TA timer (FFS other conditions):

	Reception of RRCResume message

	Reception of RRCSetup message

	Reception of RRCRelease message without SRS configuration

RAN1 has concluded their work on positioning with additional agreements in last meeting, see [12].

In this contribution we continue discuss SRS configuration and alignment of eDRX and PRS configuration, and we also have a discussion point related to Validity area and Timing Advance.

# Discussion

## 2.1 Support for multiple SRS configurations

A UE supporting LPHAP is expected to reduce its power consumption significantly compared to the legacy UE in NR. Hence, the LPHAP target battery life as discussed during study item phase can be met. This, however, might lead to some limitations in the operation of a LPHAP UE. In many of the LPHAP use-cases, for example, on the operation of LPHAP UE in a factory / warehouse, there exists a certain number of base-stations. The position of the LPHAP UE needs to be tracked by the system for certain purposes, such as factory automation, tracking, logistic, etc. The LPHAP UE, in this use case, is expected to only have mobility function within the factory / warehouse. Hence, it will interact with those base-stations (gNB) as illustrated in Figure 1. Here, the LPHAP UE supports mobility with a limited number of gNBs. We assume once the LPHAP UE is outside the factory then the LPHAP UE can no longer utilize the low power capability or may not be operated at all.

In this illustrated indoor scenario, the LPHAP UE transmits uplink sounding reference signal (SRS) for positioning purposes to the gNBs. Each gNB performs positioning measurements, based on for instance UL-TDOA, or UL-AoA. The positioning measurements results are transmitted to the LMF. Furthermore, the LMF performs the legacy positioning estimation based on the obtained positioning measurements and the geographical position of the gNBs.

Figure 1: Illustration of LPHAP UE in an indoor factory

The location or the operation area of a few numbers of gNBs can also be called as validity area. Here, we can consider the operation of LPHAP UE is limited and only interact with those gNBs. Since the LPHAP UE operation is limited and can also be predicted, especially if we consider LPHAP UE operates with certain dedicated function, then the LPHAP UE can be preconfigured with UL SRS configuration. There are limited number of gNBs, so that there could be multiple UL SRS configuration that can be provided to the UE. In the legacy positioning, the UE retrieves the UL SRS configuration when the UE is in CONNECTED mode. When the UE moves to another cell then the UE would require to be in CONNECTED mode to receive the UL SRS configuration of a new cell. Transitioning to CONNECTED mode to only retrieve the UL SRS configuration leads to extra power consumption at the UE. It would therefore be beneficial if the UE receives the multiple UL SRS configuration for positioning in advance and the UE can use that when the UE operates in RRC_INACTIVE in that validity area, preventing it to transit to CONNECTED mode just to get an updated configuration.

Proposal 1: Support multiple UL SRS configuration supporting mobility between several gNBs within a validity area for positioning of the UE to operate UL and DL+UL positioning in RRC_INACTIVE state.

The configuration can be provided when the UE is in the connected mode via dedicated RRC signalling or it can also be provided using system information.

At RAN2#121bis e meeting it was agreed that RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE. It would be beneficial if the RRC signalling could contain multiple SRS configurations and potentially also multiple validity areas, which could be active at a later stage.

At RAN2#123 it was agreed to not support providing pre-configured SRS via system information in Rel-18

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode)

## 2.2 Network triggered activation of UL-SRS transmission

When the LPHAP UE operates in RRC_INACTIVE state, it would be beneficial if the LPHAP UE can be triggered to perform positioning procedure, such as UL positioning by transmitting UL SRS., based on previous configuration. In the case of the trigger comes from the network (e.g., LMF), the trigger can be included in downlink signal / channel in RRC_INACTIVE state, some wake-up mechanism would be suitable, e.g., using PEI, or paging DCI. For this likely legacy functionality can be used, but some positioning trigger indication would be needed.

Proposal 3: Support network to trigger activation of UL SRS transmission via downlink signal when the UE is in RRC_INACTIVE mode based on previous configuration.

At last meeting [13], it was agreed that activation/deactivation is not required for periodic SRS. Implicitly this means that the UE is configured in RRC Release message when to perform UL SRS transmission. Still there is no possibility to configure or activate the UE when being in RRC_Inactive.

Currently in Rel-18, there is an ongoing work item for MT-SDT [6], and agreements are made to support MT-SDT.

It would be relevant to discuss some aspect related to Low Power High Accuracy Positioning (LPHAP) related to MT-SDT.

We see e.g., about how to configure the UE to perform positioning measurements and reporting, even though the principles of “Low Power Periodic and Triggered 5GC-MT-LR Procedures” [5], should be considered. For this LCS procedure there are a number of variants like “deferred” or scheduled location time, where the UE has been configured/pre-configurated, while in  Connected mode (e.g., during Release), in order to at a later stage perform a positioning activity.

But any Network triggered positioning procedure while the UE is in Idle/Inactive mode is currently not standardized, though it is part of the objectives in the Positioning study, it would be more relevant to discuss in the SDT session for Rel-18.

Observation 1: Network triggered MT-LR procedures for UEs in Idle/Inactive is currently not supported.

Proposal 4: Network triggered MT-LR procedure using MT-SDT would preferably be discussed in conjunction with the SDT session.

The MT-SDT procedure would be suitable when network needs to trigger and potentially also configure a UE in RRC Idle/Inactive mode) to perform measurement and/or measurement reporting.

Further, in some of scenarios the UE has been configured when in RRC Connected mode, or when being released from Connected mode to Idle/Inactive mode. In some other scenarios the UE is not configured at all, and hence needs both being triggered as well as configured in order to perform the positioning measurements and/or reporting.

Observation 3. Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured while released from Connected mode.

Proposal 5: Both alternatives, configuration triggered via MT-SDT or configured via RRC release should be considered.

Proposal 6: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

Alignment of DRX for paging monitoring and occasions for UL SRS transmissions.

Alignment between DRX and DL-PRS is being discussed, but also for the case of UL-SRS transmission there could be a benefit to align these transmissions in Idle/Inactive mode, with the downlink paging monitoring, to save power and reduce the number of times the radio has been woken up.

The SRS should e.g., be transmitted in relatively short time after the reception of paging message, such as relative to the start of DRX active time. Furthermore, in case, after the paging reception, the UE performs RACH procedure in which the timing advance acquiring information can be used to validate the SRS configuration. If the SRS configuration is still valid, the UE continues to perform SRS for positioning.

Proposal 7: Support aligning the DRX on duration and UL SRS transmission for positioning and paging monitoring.

## 2.3 Alignment between eDRX and PRS configurations

In the RRC_IDLE/RRC_INACTIVE the operation of DRX and positioning procedure are performed independently from each other. This can result in an unnecessary UE power consumption as the UE needs to power on and off its receiver several times to perform both paging related and positioning related operations. To avoid such unnecessary wake-ups, there is therefore a need for positioning or DRX operation enhancements, where the operations of positioning and paging procedure can be aligned.

In the agreed work item one objective is related to alignment between eDRX and PRS. We consider this valid for any Idle/Inactive DRX scheme.

There can be two types of PRS transmissions, these are periodic and aperiodic (on-demand) PRS transmissions. Below, we describe an enhancement technique for each type.

- In case of aperiodic/event triggered PRS transmission, the PRS transmission (by gNB) is initiated by LMF, but triggered by a defined event such as movement, change of temperature etc. In this case, LMF is not aware of DRX occasion configuration, e.g., occasion time, periodicity, etc. We propose LMF to assign and optionally configure a PRS pattern to relevant gNB(s), and UE, that is aligned with the DRX pattern.

- In order to make the LMF aware of UE’s DRX pattern, UE reports to LMF, where the report contains the UE-ID, and/or its calculated DRX cycle/Paging Frame, i.e., in which SFN it will wake up to monitor Paging Occasion (PO). This report can be included in a new information Element IE, in the On-demand PRS request message. The LMF will then assign and configure a PRS pattern to relevant gNB and UE that is aligned with the configured DRX pattern

In case of periodic PRS transmission, we propose the gNB to adjust UE’s DRX occasion based on UE request so that it is aligned with PRS transmission. To facilitate this, we propose the UE calculates the distance in time between PRS occasion and DRX occasion. Then, the UE proposes a DRX offset to be used by the gNB (or AMF). Once, it is granted (by gNB of AMF), the UE is provided with the new (adjusted) DRX,

Case a) has been agreed, but the alignment of of DRX cycle to the fixed PRS transmission has not been concluded.

Proposal 8: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation.

## 2.4 Validity area and TA value

At RAN2#121 meeting it was discussed and proposed in R2-2300115 [4], to wait RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc, including the proposal below.

In RAN1#112bis, no further agreements were made related to TA [8]. And at RAN2#122 some agreements were made to define a validity area specific TA-timer for a UE in RRC.Inactive state, and which can be started/restarted with TA command, stopped when timer expires or if the UE reselects outside the validity area.

Observation 4: Depending in the size of the validity area, the TA value may have to be adjusted depending on UE location.

In legacy NR, timing advance plays important role for uplink transmission, including UL SRS transmission. The TA value is obtained during random access procedure. The UE transmits preamble (Msg1), gNB calculates the TA, then gNB provides the TA value to the UE via Msg2 during RACH procedure. The UE uses the TA for subsequent transmissions. The TA value is related to the serving cell it was released from.

A UE subject for LPHAP, can in some cases be considered as a moving device, including moving within the validity area. Depending on distance to base station, the UE may need to re acquire a new TA value. But re-obtaining a new TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption and signaling. On the other hand, not re-obtaining TA value could lead the LPHAP UE to use inaccurate TA and resulting the unsynchronized UL SRS reception at the gNB side.

Observation 5: Reobtaining the TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption and signaling.

Hence, it would be beneficial for the UE to update the TA value without performing RACH procedure.

At RAN1#113 [11], it was agreed that,

If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens.

But also when the UE is moving within the cell, the TA value may have to be adjusted.

At RAN1#114 [12], the following was agreed.

With regards to the reference RS for the RSRP change for TA validation:

- Alt1: The downlink pathloss reference for TA validation (stored RSRP) is derived from the cell where UE determines the latest valid TA:

- If UE maintains the TA from the last serving cell, the stored RSRP of the downlink pathloss reference is derived from SSBs of the last serving cell.

- otherwise when UE determines to autonomously adjust TA when enabled by the network and when cell re-selection occurs, if confirmed by RAN4, the stored RSRP of the downlink pathloss reference is updated with a new value derived from SSBs of the current camping cell.

- Send LS to RAN2 stating that there is no consensus in RAN1 regarding the reference RS for the current RSRP derivation for TA validation but it can be further discussed in RAN2.

The UE can adjust the TA based on downlink measurements, for example, based on SSB, DL-PRS reception. DL-PRS is designed for positioning purpose and typically transmitted in a wider bandwidth. The autonomous TA adjustment based on DL-PRS reception is expected to improve the accuracy of the new TA value. Hence, it improves the positioning estimation calculation. One can consider the adjustment of TA value using DL-PRS can be up to UE implementation. However, we consider the adjustment of TA value based on DL-PRS is expected to be better than SSB-based measurement, particularly when DL-PRS is transmitted with wider bandwidth. We consider it would be beneficial if the gNB provides indication which reference signal to be used for UE autonomous TA value adjustment.

Proposal 9: Support the UE to adjust the TA value based on the downlink timing measurement.

# Summary

In this contribution, we have discussed our view on potential positioning enhancements in NR. Our observations and proposals are listed below:

Proposal 1: Support multiple UL SRS configuration supporting mobility between several gNBs within an validity area for positioning of the UE to operate UL and DL+UL positioning in RRC_INACTIVE state.

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode).

Proposal 3: Support network to trigger activation of UL SRS transmission via downlink signal when the UE is in RRC_INACTIVE mode based on previous configuration.

Observation 1: Network triggered MT-LR procedures for UEs in Idle/Inactive is currently not supported.

Proposal 4: Network triggered MT-LR procedure using MT-SDT would preferably be discussed in the SDT session.

Observation 3. Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured while released from Connected mode.

Proposal 5: Both alternatives, configuration triggered via MT-SDT or configured via RRC release should be considered.

Proposal 6: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

Proposal 7: Support aligning the DRX on duration and UL SRS transmission for positioning and paging monitoring.

Proposal 8: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation.

Observation 4: Depending in the size of the validity area, the TA value may have to be adjusted depending on UE location.

Observation 5: Reobtaining the TA value may require the LPHAP UE to perform RACH procedure, in which it may cost a non-negligible power consumption and signaling.

Proposal 9: Support the UE to adjust the TA value based on the downlink timing measurement.

# References

- RP-223549, New WID on Expanded and Improved NR Positioning, RANP #98e, December 2022

- TR 38.859, “Study on expanded and improved NR position (Release 18)”, December 2022

- Minutes RAN2#121, R2-2301904, Report from session on positioning and sidelink relay

- R2-2300115, Discussion on LPHAP

- Draft_Minutes_report_RAN1#121_v040

- RP-213583, “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR,” RAN#94e.

- Minutes RAN2#121bis-e

- Minutes RAN1#112bis-e

- Minutes RAN2#122

- [Post122][401][POS] SRS configuration and activation in LPHAP (CATT)

- Minutes RAN1#113

- Draft Minutes report RAN1#114_v020

- Minutes RAN2#123