3GPP TSG-RAN WG2 #123-bis	R2-2310833

Xiamen, CN, 9 September – 13 September, 2023

Agenda item:	7.2.2

Source:	Robert Bosch GmbH

Title:	Further discussion on sidelink positioning

Document for:	Agreement

# Introduction

In this contribution, we discuss remaining sidelink positioning issues with respect to priority, discovery, cross-PLMN, handover, delays and relative positioning.

At the RAN#101 meeting, the WID of Expanded and Improved NR Positioning has been updated in [1] as follows (changes highlighted in Yellow):

- Specify unicast session-based signalling and procedures to facilitate support of SL positioning for single target UE (it is not precluded to apply the procedures to multiple target UEs but no signaling optimizations will be considered for this case) [RAN2, RAN3]:

  - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

  - Specify the protocol and procedures for SL positioning between UEs and a single LMF for in coverage scenario only, including joint PC5-Uu scenarios.

    - NOTE: Assumes all involved UEs are served by same LMF.

  - For SL-TDOA, RAN2 will not work on procedures for synchronization of the anchor UEs. RAN2 can discuss and implement agreed RAN1 parameters related to synchronization.

Additionally, the following were agreed in RAN2#123 [2]

Agreements:

For LMF involved SL based positioning, follow SA2 on how to handle LMF involved SL based positioning between UE (who has connection with network), LMF and AMF. FFS on how to handle session for UEs involved in the same LMF involved SL based positioning and the relationship between routing ID/correlation ID and session ID.

At least for UE-only operation, introduce explicit field “sessionID” in SLPP, and put it under message header of SLPP message. FFS how session ID is defined.

At least for UE-only operation, the UE who receives the LCS request at least needs to:

-	Initiate the first SLPP procedure;

-	Assign the sessionID, and include it in the SLPP messages (Rx side should use the received sessionID for messages in the same positioning session).

FFS within what scope the session ID is unique.

At least for UE-only operation, if the UE who receives the LCS request can act as the SL Positioning Server UE, then the UE shall trigger following procedures with each of UEs (UE2-UEn in the figure) in the SLPP session:

-	SL Positioning Capability Transfer procedure,

-	SL Location Information Transfer (FFS on who decide positioning method) and

-	SL Positioning Assistance Data exchange (depends on RAN1 discussion on how to select the SL-PRS resources)

In stage 3 specification, use "Endpoint A" and "Endpoint B” to describe the procedure instead of target UE, anchor UE and server UE concept, e.g. [figure omitted]

Agreement:

RAN2 to apply terms of “UE-only Operation” and “Network-based Operation” defined in TS 23.586 by SA2 for SLPP procedures.

Agreements:

Delivery by an IC UE to the LMF via SLPP of information received from an OOC UE via SLPP (UE2 => UE1 => LMF), and the reverse operation LMF => UE1 => UE2, are needed at least for partial coverage scenarios.

FFS if this involves single or separate SLPP sessions (LMF  UE1 and UE1  UE2).

FFS if the same functionality is needed for IC scenarios (depending on whether the LMF communicates with each UE or always through the target).

RAN2 see risk to completion of sidelink positioning with the current scope.

Agreement:

FFS which (if any) additional parameters can be included (as optional or mandatory) in the metadata in the discovery message for anchor and server UE selection; it should be based on technical requirements for the fields and how they will be used.

Agreements:

Define 8 priority levels for SL-PRS priority, same as the number of priority levels for SL-SCH. Send a LS to RAN1 and SA2 on RAN2 agreement with the understanding that the SL-PRS priority levels are mapped from sidelink positioning/ranging QoS. (14/14)

The SL-PRS priority can be provided by the UE’s own high layer when it triggers the SL-PRS transmission. (14/14) The following issues are open and can be raised in the LS for RAN1 input:

Whether the UE’s higher layer can provide SL-PRS priority for the SL-PRS triggered by peer UE

Whether the peer UE triggers the SL-PRS transmission can provide the SL-PRS priority

When aperiodic/one-shot SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, design a new MAC CE for the UE to send to the gNB for SL-PRS resource request. (12/14) FFS when LMF is involved.

At least when periodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, the UE sends an RRC message to the gNB for providing the assistance information for CG configuration. (13/14) FFS when the LMF is involved.

Support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission. (14/14)

Additionally, in RAN1#114, the following relevant agreements were made [2]:

Agreement

For a slot, a single priority value is provided by higher layers to the physical layer and is used at least to determine the PSSCH and/or SL-PRS transmission power via the value of PMAX,CBR.

- For dedicated resource pool, this corresponds to the priority level of SL PRS.

- Send an LS to RAN2 requesting them to take the above into consideration when defining priority levels for SL PRS and PSSCH that are multiplexed in the same slot of a shared resource pool.

Agreement

Endorse the draft LS in R1-2308558 with the following modification:

RAN1 also made the following conclusion related to priority and congestion control, and RAN1 expects the same handling of priorities for shared resource pool as the above agreement.

ACTION: RAN1 respectfully asks RAN WG2 to take the above agreement and conclusion into account when defining priority levels for SL PRS and PSSCH that are multiplexed in the same slot of a shared resource pool.

Final LS in R1-2308559.

Agreement

To mitigate the impact of synchronization errors between anchor UEs for SL-PRS based measurement, the exchanged synchronization information of anchor UEs between a UE and LMF or another UE includes the following:

- [The synchronization source type (GNSS, gNB/eNB, and UE) of anchor UEs,

  - If the synchronization source of an anchor UE is SyncRef UE, the anchor UE can optionally indicate the coverage status and synchronization connection status (whether the SyncRef UE is directly or indirectly synchronized to GNSS/gNB, or other SyncRef UE) of the SyncRef UE

  - If the synchronization source of an anchor UE is gNB, the anchor UE can further provide cell identity information]

- [Synchronization quality/accuracy information]

- The RTD between anchor UEs

Agreement

For SL Positioning measurement report content, the following can be included:

- [SL PRS resource ID]

- ARP ID used for reception

- Measurement results

  - Rx-Tx timing difference and quality

  - RSTD measurement and quality

  - RTOA measurement and quality

  - AoA measurement and quality

- RSRP, RSRPP measurement Time stamp

  - Rx timestamp

  - Tx timestamp

- LoS/NLOS indicator

- [UE identity information or information related UE identity information]

Note1: unified or separate report for different SL positioning methods is up to other WGs (e.g., RAN2)

Note2: whether to include UE identity information or information related UE identity information is up to RAN2, including whether this is optional in the report.

Agreement

The following quality information can be reported in a similar way as in legacy Uu positioning:

- timing quality corresponding to the timing related measurements

- angle quality corresponding to the AoA measurement

No specification impact on how to set the quality information and from RAN1 perspective, no performance requirements are expected to be defined for the quality information in Rel-18.

It is up to RAN2 whether location quality information can be reported when location information is reported.

Finally, the following important remaining open issues in RAN2 was identified in [2]:

- Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA.

- Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

# Discussion

## SL positioning transport

### SL positioning PC5 QoS flow mapping

In RAN2#123 meeting, 8 levels of priority have been agreed (i.e., for 3-bit priority field in SCI). According to RAN2 agreement, SL-PRS priority can be provided by the UE’s own high layer when it triggers the SL-PRS transmission. However, higher layers may not confine itself to the same 8-levels for each UE, or each UE may have different mapping between positioning QoS and associated priority level. Thus, if this is left purely to UE/application implement, this may create non-uniform performance among UEs and may also impact the resource allocation and congestion.

In a shared resource pool, both SL-PRS priority and SL traffic priority need also to be harmonized, i.e., at least when there is a comparison between the two priorities for LCH mapping and/or resource allocation.

In this case, higher layer can also indicate the required accuracy or convey the specified positioning QoS. Herewith, lower layers (of server UE or LMF) can perform a mapping (either many-to-one or based on a selection criterion, e.g., priority thresholds) between the values coming from higher layers and a specified 8-level priorities in RAN2 specs.

Observation 1: In a shared or dedicated resource pool, leaving the mapping of SL-PRS 8 priority level only to higher layer may result in unharmonized performance among the UEs.

Proposal 1: RAN2 to specify mapping of 8-level SL-PRS priorities to values coming from higher layer. RAN2 to inform other WGs about its decision.

- FFS: whether the mapping is done in server UE or LMF only.

### SL-PRS periodicity

In the case of periodic PRS, the server UE (in UE-only operation) or LMF are the responsible entities for setting the frequency to be selected from (pre-)configured periodicities. In the current RAN1 discussions, it is mentioned that the PRS frequency is set by higher layers without giving more details. Additionally, for SL communications, the periodicity is set mainly based on the (remaining) PDB. However, for SL positioning procedure, the selection of periodicities should be different, e.g., based on the required accuracy and/or other QoS requirements.

Different applications may have different positioning requirements depending on external parameters (e.g., UE speed, the application nature, etc.), i.e., setting different QoS requirements for positioning. Depending on MO-LR and MT-LR sidelink positioning procedures, the LMF/server UE could translate Application layer requirements, e.g., update rate, accuracy, and precision, into an appropriate PRS period (i.e., similar to periodicity).

Observation 2: SL-PRS reservation period can be related to the QoS requirements of applications triggering positioning procedure.

Proposal 2: Discuss possible RAN2 impact for mapping between positioning QoS requirements and PRS period.

- FFS: whether the mapping between positioning QoS and periodicity is done in server UE or LMF only.

## Remaining details on discovery procedure and anchor / server UE selection

RAN2 has been discussing the development of procedures for SL positioning including anchor and target UE roles and possible discovery procedure.

### UE-Only SL Operation (for SL positioning)

During RAN2#123, the rapporteur reports the following options:

For SLPP procedures, there were two options on how to capture the SLPP procedures in stage 2 specification:

- Option 1: capture the entire procedure of SL positioning in the stage 2 specification;

- Option 2: capture the separate procedures of each SLPP transfer as well as legacy Uu positioning.

In our understanding, it is important to capture the entire procedure of SL positioning in the stage 2 specification. Additionally, and according to WID update in RAN#101, the UE only procedure need to be considered if the target UE or any of this anchor UEs are out of coverage, i.e., this covers also partial coverage scenario.

Based on the agreed series of steps for SL positioning procedure, Figure 1 suggests a simplistic out-of-coverage SL procedure introducing Server UE, which can be separated or co-located with target UE.

In the signal flow in Figure 1, and after location service request is triggered, SL positioning capability is exchanged between server UE/Target UE and Target UE/Anchor UE(s). In this proposal, the Anchor UE(s) selection can be either in server UE or Target UE. Additionally, the SL-PRS is exchanged between Anchor UE(s) and target UE. Finally, the measurement data and positioning information are provided from Anchor UE(s) to target UE/Server UE and from Target UE to Server UE, where the final positioning calculation is conducted in the server UE.

Figure 1 UE sidelink positioning for out of coverage scenario with server UE (Optional)

Observation 3: In case of out of coverage, server UE can be captured as an optional entity in stage 2, where possible co-location with target UE should be noted.

Proposal 3: RAN2 to confirm the signaling flow for UE based sidelink positioning for out of coverage with server UE (optional, and possibly co-located with target UE) as captured in Figures 1.

### UE-Only SL Operation for partial-coverage

It is clear that the WID updates assumes that for LMF operation, all UEs are assumed to be under coverage of same LMF (in in-coverage scenario). Where is it also easy to conduct the out of coverage is considering UE operation only. However, for partial coverage it is still not clear how to support SL positioning under the current down-scoping, i.e., when forwarding information of LMF to out of coverage UEs are not going to be specified. Therefore, we propose to switch always to out of coverage operation when at least one anchor UE is in out of coverage of the LMF.

Observation 4: The current down-scoping does not indicate the case of partial-coverage.

Proposal 4: RAN2 to confirm that the out-of-coverage scenario is when at least one UE (involved in SLPP operation) is not in coverage of the LMF.

### Details on server UE and its selection

RAN2 agreed in #121 [5] meeting on the following functionalities of server UE:

Agreement:

RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

Additionally, section 6.8 in TS 23.586 [6] dictates the following:

SL Positioning Server UE: A UE offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to determine Ranging/SL Position method, distribute assistant data and calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if any of the functionalities is supported.

In RAN2#122, two different understanding were considered for this agreement (which have been captured in the Summary of AI 7.2.2):

- Understanding 1: different functionalities of server UE can be distributed over different UEs, all these UEs are considered as location server UE.

- Understanding 2: all the functionalities of server UE are located in the same UE, which is known as server UE.

In our view, understanding 2 is easier to consider in Rel-18 given that a server UE is considered for (at least) out-of-coverage and partial coverage scenarios. In this case, a separate entity that hold all functionalities of the server UE need to be defined (also in stage 2 specifications). Additionally, the second understanding is not misaligned with SA2 definition above so far. Wherein this server UE entity can be, at any time, the target UE itself or any of the Anchor UE(s) or a third-party UE (not directly involved in SL-PRS measurements, e.g., a RSU in V2X use case). Additionally, RAN2 may discuss the case when server UE is not selected initially and whether it is the target UE or Anchor UE based on the location service request, e.g., based on MO-RT or MT-RT, etc..

Thereafter, for selection of server UE, at least when the target UE is not capable of being server UE and the coverage scenario is either out of coverage or partial coverage,

Observation 5: understanding 2 (all the functionalities of server UE are located in the same UE) is easier to conclude in Rel-18

Proposal 5: RAN2 to confirm that all the functionalities of server UE are located in the same UE, which can be the Targe UE, Anchor UE, or a third-party UE.

### Sidelink positioning during handover

A case similar as cross-PLMN that needs to be handled in RAN2 is SL positioning during a handover situation. In this, due to high mobility of target or anchor UE(s), there is always a possibility that one UE will be out coverage during SL procedure. Figure 3 depicts the situation of anchor UE performing handover during the SL positioning procedure.

Figure 2: a) one target UE and two anchor UEs in the coverage of the same LMF during an SL positioning procedure; b) one anchor UE is out-of-coverage the initial LMFs after HO took place during the SL positioning procedure.

In RAN#101, it was decided for in coverage scenario to have all UEs under the same LMF. In this case, the partial coverage has to be treated differently.

In our understanding, forwarding LMF information (to UEs and back to LMF) shall be also down scoped based on the WID update. Therefore, we propose to switch all UEs to out-of-coverage during SL positioning procedure when one (or more) UE is (are) performing handover. FFS how to handle the switching time between LMF based and UE-only operation.

Observation 6: the current procedure does not cover the handover case and does not discuss whether the WID down scoping impacts mobility.

Proposal 6: RAN2 to discuss in the SL positioning procedure the impact of switching all UEs to out of coverage mode during handover procedure.

- FFS: how to handle the switching time between LMF based and UE-only operation

### Further SLPP signalling related timing and delays for location quality information

RAN1#114 Agreements:

Agreement

The following quality information can be reported in a similar way as in legacy Uu positioning:

- timing quality corresponding to the timing related measurements

- angle quality corresponding to the AoA measurement

No specification impact on how to set the quality information and from RAN1 perspective, no performance requirements are expected to be defined for the quality information in Rel-18.

It is up to RAN2 whether location quality information can be reported when location information is reported.

There are some timing-related factors that affect the relevance of the estimation, e.g., when the target UE has moved between the positioning request and the positioning provision. This is especially true for high mobility UEs such as cars. RAN2 already mentioned to SA2 that relative velocity between two UEs can be estimated; however, it is not yet decided how it will be exactly specified in Rel-18.

The mobility affecting the measurements is exactly the displacement of the UEs (e.g., relative to each other) during the measurement/positioning procedure delays. Knowing/exchanging the exact timestamps should support compensating such a displacement error in positioning. Therefore, RAN2 need to discuss exchanging other assistance timing information for increasing positioning accuracy, e.g., timestamping of SLPP measurements and additional reporting delays.

Observation 7: the accuracy of the estimated position may drastically be affected by delays for high mobility nodes.

Observation 8: Exchanging of time-stamping and additional reporting delay should be used to compensate any uncounted displacement due to mobility

Proposal 7: RAN2 to consider supporting the exchange of assistance timing information, e.g., precise timestamping of SLPP measurements, additional delays (e.g., network delays).

In addition to timing, the relative velocity and the direction of the mobility could also impact the measurements. Herewith, RAN2 should consider supporting the exchange of UE mobility characteristics (i.e. UE speed, direction). These high mobility nodes also have the advantage of having known speed and heading thanks to cooperative awareness, and sometimes known predicted trajectories. Combined with precise timestamping of measurements and a priori estimation of delays, the LMF or server UE may be able to compensate the mobility of nodes during the SLPP.

Proposal 8: RAN2 to consider supporting the exchange of assistance mobility information for the target and anchor UEs, e.g., direction of heading, velocity, etc..

## Open topics in sidelink relative positioning

### Further assistance data related to per panel reporting

RAN1#114 Agreements

Agreement

For provision of the ARP location information in assistance data for sidelink positioning, support the following:

- The ARP location information can be a position relative to a ‘reference point’.

- Note: RAN1 will not define “reference point”. The “reference point” definition can be up to other WGs

In the assistance data, the reference point (which could resemble the aperture geometry indication of each ARP in the UE) and, additionally, further related information for every ARP need to be conveyed to LMF or UEs, e.g., ARP ID, position relative to a ‘reference point’, the associated SL-PRS, etc..

For the reference point, RAN1 decided that the definition should be left to other WGs. In our understanding, RAN2 should convey the reference point definition from higher layers. The reference point can be identified based on the shape, size and geometry of the UE, which is left to the UE. However, RAN2 need to have a limited options for the reference point to be able to conduct information about the ARP.

Figure 3: Vehicle bounding box with ARP (orange) and ARP reference point (black)

Observation 9: RAN1 did not identify the definition of reference point of ARP location information in the assistance information.

Proposal 9: RAN2 to define the reference point and configure few options for the ARP location in the device to be conveyed in ARPs location information in assistance data.

The problem of the existing assumptions is that the orientation of the ARPs may not be easily considered if the pose of the and the orientation of the ARPs are not among the assistance data/information. With the relative position of the ARPs with respect to the reference point, the ARPs are located on circles around the reference point, although with the appropriate position relative to each other. This is solved with the orientation of at least one ARP with respect to the reference point. Therefore, antenna and/or ARP orientation can be very beneficial if shared as an assistance information (e.g., leaving further utilization to the UE implementation).

Observation 10: in the case of vehicle devices, distances between panels can be of up to few meters, and the ARP orientation can highly impact the estimation of the distance if not taken into account.

Proposal 10: RAN2 to consider ARP orientation in the assistance information report for sidelink positioning measurements.

### Positioning Filtering of Higher Accuracy

Multilateration (spatial) and multi-iteration (temporal) are two related concepts that are used in positioning systems to determine the location of an object or device and are still open for Rel-18. Multilateration refers to the process of determining the location of an object or device based on the distance between it and multiple reference points or sensors. This distance can be measured using various techniques, such as time-of-flight, angle-of-arrival, or phase differences. By combining the distances from multiple reference points, the position of the object or device can be triangulated. Multi-iteration, on the other hand, refers to the process of iteratively refining the position estimate of an object or device based on the measurement data from multiple reference points. This process involves iteratively adjusting the position estimate until it converges to the true position of the object or device.

Multilateration and mult-iteration enable accurate and efficient determination of the location of objects or devices. However, it’s important to note that both methods have limitations and can be affected by various factors, such as signal strength, multipath effects, and sensor noise.

Observation 11: multilateration and multi-iteration can be used to enhance the UE positioning accuracy.

Multilateration and/or multi-iteration can be done when Server UE requests relative positioning between target and one or more anchor UEs, especially using located anchor UEs. They both yield multiple measurements that need to be processed in order produce on estimate. To do so, higher layer filtering of measurements can be done at server UE.

Proposal 11: RAN2 to discuss whether a protocol of multiple measurements should be specified or left to the UE implementation. FFS: if specified, whether the higher layer filtering should be discussed in RAN2 or other WGs.

# Conclusion

Based on the discussion in this paper, we recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: RAN2 to specify mapping of 8-level SL-PRS priorities to values coming from higher layer. RAN2 to inform other WGs about its decision.

- FFS: whether the mapping is done in server UE or LMF only.

Proposal 2: Discuss possible RAN2 impact for mapping between positioning QoS requirements and PRS period.

- FFS: whether the mapping between positioning QoS and periodicity is done in server UE or LMF only.

Proposal 3: RAN2 to confirm the signaling flow for UE based sidelink positioning for out-of-coverage with server UE (optional, and possibly co-located with target UE) as captured in Figures 1.

Proposal 4: RAN2 to confirm that the out of coverage scenario is when at least one UE (involved in SLPP operation) is not in coverage of the LMF.

Proposal 5: RAN2 to confirm that all the functionalities of server UE are located in the same UE, which can be the Targe UE, Anchor UE, or a third-party UE.

Proposal 6: RAN2 to discuss in the SL positioning procedure the impact of switching all UEs to out of coverage mode during handover procedure.

- FFS: how to handle the switching time between LMF based and UE-only operation

Proposal 7: RAN2 to consider supporting the exchange of assistance timing information, e.g., precise timestamping of SLPP measurements, additional delays (e.g., network delays).

Proposal 8: RAN2 to consider supporting the exchange of assistance mobility information for the target and anchor UEs, e.g., direction of heading, velocity, etc..

Proposal 9: RAN2 to define the reference point and configure few options for the ARP location in the device to be conveyed in ARPs location information in assistance data.

Proposal 10: RAN2 to consider ARP orientation in the assistance information report for sidelink positioning measurements.

Proposal 11: RAN2 to discuss whether a protocol of multiple measurements should be specified or left to the UE implementation. FFS: if specified, whether the higher layer filtering should be discussed in RAN2 or other WGs.

# References

[1] RP-232670, “Expanded and Improved NR Positioning,” Revised WID, Intel Corporation, CATT, MediaTek, RAN#101, Bangalore, September 2023.

[2] RP-232041, “SR of Work Item on expanded and improved NR positioning,” Intel Corporation, CATT, MediaTek, RAN#101, Bangalore, September 2023.

[3] R2-2309644, “Summary of [Post123][403][POS] Sidelink positioning MAC issues,” Huawei, HiSilicon, Xiamen, China, October, 2023

[4] TR 23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning.

[5] TS 23.586 Architectural Enhancements to support Ranging based services and Sidelink Positioning.