# 3GPP TSG RAN WG2 Meeting #123bis                           	    R2-2310903
Xiamen, China, October 9-13, 2023

Agenda item:	7.15.3

Source:	Qualcomm Incorporated

Title:	Discussion on SL FR2

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss way forward for sidelink FR2.

# Discussion

It’s scoped that sidelink FR2 SI will be on sidelink beam management for unicast by using existing sidelink CSI-RS framework and Uu beam management concepts if applicable.

For sidelink beam management, the following subjects have been outlined in the SI objective.

- Initial beam-pairing

- Beam maintenance

- Beam failure recovery

## Sidelink initial beam-pairing

The following agreements were made on initial beam pairing at RAN1 #112-bis e-meeting [1].

Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, including at least the following steps and how to determine UE2: - UE1 sends reference signals via different transmit beams - Note: multiple reference signals transmissions (e.g., repetitions) from each of the beams can be studied - FFS when reference signals are sent - FFS applicable reference signal - UE2 measures the reference signals and determines a UE1 transmit beam and/or a UE2 receive beam - FFS:whether/how to determine a UE2 transmit beam - UE2 indicates to UE1 the determined UE1 transmit beam - FFS how to indicate the determined transmit beam, including its feasibility - UE1 and UE2 set up sidelink unicast link using the determined beam, following existing link establishment procedure. Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed during sidelink unicast link establishment - UE1 sends PSCCH/PSSCH that carries unicast link establishment message (e.g., DCR message) via different transmit beams - Note: multiple PSCCH/PSSCH transmissions (e.g., repetitions) from each of the beams can be studied. - FFS: applicable reference signals which are transmitted together with unicast link establishment message. - if UE2 successfully decodes one (or more) of the PSCCH/PSSCH(s) and UE2 determines to establish a unicast link with UE1, it indicates to UE1 one (or more) UE1 transmit beam(s) of PSCCH/PSSCH(s) which is successfully received - FFS details (e.g., implicit or explicit indication) - FFS: how to map between each PSCCH/PSSCH and UE1 transmit beam - FFS: how UE2 determines UE1 transmit beam(s) and/or UE2 transmit/receive beam(s) - UE1 uses one of the indicated beam(s) to finish the remaining sidelink unicast link establishment procedure with UE2 - FFS: how UE1 determines one of the indicated beam(s) - FFS: use of additional reference signal or additional messages or additional measurement for efficient beam pairing. Agreement RAN1 can study the following candidate procedure where initial beam pairing starts after sidelink unicast link establishment between UE1 and UE2, including studying whether and in which cases initial beam pairing after sidelink unicast link establishment is feasible. - UE1 and UE2 set up sidelink unicast link, following existing link establishment procedure - FFS the beams used for unicast link establishment. - UE1 and/or UE2 configure the resources for beam sweeping and/or beam reporting - FFS details of resources configuration - UE1 and/or UE2 use the configured resources to transmit reference signals and determine a pair of transmit beam and receive beam based on beam sweeping. - FFS applicable reference signal(s) - FFS whether/how to indicate the determined beams between UE1 and UE2 FFS difference between initial beam pairing (after sidelink unicast link establishment) and beam maintenance

RAN1 closed the SI with all above three schemes included [3]. It’s expected that down selection will be conducted in WI phase if the WI is approved for Release 19.

## Sidelink beam failure recovery

The following agreements were made on beam failure recovery at RAN2 #121-bis e-meeting [2].

Agreement:

For beam failure detection, reuse Uu design of timer + counter based mechanism as baseline, and R2 further study how SL beam failure is detected. FFS on Tx or Rx UE based manner.

Agreement:

Upon beam failure is detection, support BFR signaling exchange between peer UEs, and further study e.g., RLF declaration due to beam failure.

How to detect sidelink beam failure is currently under discussion in RAN1, which includes two detection schemes 1) Rx UE detects beam failure based on beam measurements and 2) Tx UE detects beam failure based on DTX of HARQ feedback.

Rx UE based detection is more aligned with Uu UE based beam failure detection but requires periodic or semi-periodic sidelink CSI-RS transmissions for beam measurements.

Tx UE based detection doesn’t require extra sidelink CSI-RS transmissions for beam measurements but cannot provide candidate beams for timely beam recovery.

Even though initial beam pairing, beam maintenance and beam failure recovery are separate design aspects of sidelink beam management to be addressed for sidelink FR2, but they are all tied with the signal design for beam measurements. For example, a best Tx beam selection for initial beam pairing may be based on beam measurements, beam maintenance depends on beam monitoring which may be based on beam measurements, beam failure recovery with candidate beam(s) selection may also require beam measurements. Therefore, it’s preferable to have a unified framework for different design aspects of sidelink beam management for sidelink FR2.

RAN1 closed the SI with both stand-alone CSI-RS and non-standalone CSI-RS included for beam maintenance [3]. It’s not clear if down selection will be conducted in WI phase, if the WI is approved for Release 19.

Proposal 1. Based on RAN1's closure with the SI, no more study needed at RAN2.

Proposal 2. RAN2 may start WI based on RAN1's progress in Release 19 if approved.

# Conclusion

In this contribution, we discussed way forward for sidelink FR2 and concluded with the following proposals.

Proposal 1. Based on RAN1's closure with the SI, no more study needed at RAN2.

Proposal 2. RAN2 may start WI based on RAN1's progress in Release 19 if approved.

# References

[1] Session chair meeting note, “Chair's notes RAN1#112bis-e 9.4 R18 NR SL v09”, 3GPP TSG RAN1 #112-bis e-meeting, April 17 – 26, 2023.

[2] Session chair meeting note, “RAN2-121bis-e_R16_NR SL_R17&R18_NR SL enh_20230425-EOM_r1”, 3GPP TSG RAN2 #121-bis e-meeting, April 17 – 26, 2023.

[3] RP-231583 “Status Report to TSG”, RAN #101 meeting, Bangalore, India, September 11 – 15, 2023.