3GPP TSG-RAN WG1 Meeting #123bis	R2-2310983

Xiamen, China, 9th – 13th October, 2023

Agenda Item:	7.4.2.1

Source:	Panasonic

Title: 	Fast cell recovery aspects for LTM failures

Document for:	Discussion and Decision

# Introduction

As per the discussion in RAN2#123 meeting, the following agreements regarding LTM fast recovery are made [1].

Upon an LTM cell switch failure (i.e., supervision timer expiry) or RLF, fast recovery similar to CHO: - a) UE performs cell selection. - b) If selected cell is an LTM candidate cell, UE performs RACH-based LTM cell switch on the selected cell (network-controlled). - c) If selected cell is not an LTM candidate cell, UE transmits RRC re-establishment request.

Also, from the running CR 38.300, post-RAN2#123 email discussions [2], the following procedure for LTM fast recovery is captured,

- in case of LTM, for RLF in the source cell: - selects a suitable cell and if the selected cell is a LTM candidate and if network configured the UE to try LTM after RLF then the UE attempts LTM execution once, otherwise re-establishment is performed; - enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

This document discusses few aspects on LTM fast recovery, providing more details to the above procedure.

# Discussion

From RAN2 agreements the support for fast cell recovery using the LTM candidate configurations are intended. When encountering RLF with the source cell or LTM cell switch failure, i.e. when LTM supervisor timer expires or upon detecting RLF at the target cell, UE is expected to perform LTM execution to one among the candidate cells. But the intuition behind the network configuration for the purpose of LTM recovery and its detailed procedure are not yet discussed in detail.

In this document the network controlled LTM recovery mechanism is proposed, where the network controls the configuration towards potential candidate cells for UE cell recovery.

## Network configuration to benefit fast cell recovery process.

During LTM procedure, one of the following conditions could be applied by the gNB to initiate or update the list of LTM candidate cells configured at the UE, and later command LTM execution towards the identified target cell.

- The radio quality of the source cell at UE gets poor.

- L1 measurement results reported to evaluate the radio link status of the candidate cells with the source cell.

- Other handover conditions which may not involve radio link aspects such as, cell congestion or load balancing of the current serving cell, or when specific services are supported by a specific cell and requires cell change.

Since the network is in control of cell switch decision to the target cell during LTM execution, the choice of the target cell could be the changing channel conditions or any other handover criteria which does not involve radio link aspects at all. Such "light" operation of cell switch is one of the merits of LTM, since prolonged data loss could be reduced until the service is resumed over the new cell.

Eventually, when UE encounters cell switch failure either upon supervisor timer expiry or RLF at the target cell, also when RLF is detected at the source cell before cell switch command, in either of these cases UE autonomously deciding the best candidate cell for recovery based on only the channel conditions among the configured LTM candidate cells may not always result in the best services provided to the UE. At times when network decides that the recovered cell is not suitable to continue, it could trigger another cell switch towards a better target cell resulting in prolonged delay before normal services are resumed at that UE. Overall, since LTM procedure focusses mainly on reducing the handover latency to provide fast cell switch or recovery to the best cell at the earliest, giving more control to the network for both cell switch and cell recovery achieves the main purpose.

Observation 1: Configuring recovery candidate cells at UE provides faster recovery to the best candidate cell, both in the view of radio condition based cell selection and non-radio condition based cell selection criteria.

Proposal 1: UE performs cell selection to one of the LTM candidate cells configured  for recovery. And the LTM recovery cell configuration process must be possible to re-configure according to the dynamic network conditions and channel variations.

## Fast recovery operations for LTM failures.

When the target cell SINR/ signal quality is better than the source cell SINR/ signal quality, the cell switch execution is most likely to succeed but not always guaranteed. Moreover, when UE moves to the boundary of two candidate cells, then the cell switch execution to the target cell may not always succeed (or not always guaranteed), but instead the cell switch to the other candidate cell towards this boundary may succeed.

In the process of cell selection for LTM cell switch, network may perform early DL and UL synchronisation to one or multiple candidate cells once the best set of cells are identified, preparing for the LTM cell switch operation which might be intended later during the UE mobility. In such cases the UE could use this candidate cell for recovery with a highest priority among all the other configured candidate cells. Based on the simple reason that, the network has already identified the next best cell for cell switch or recovery and started LTM execution process early. This becomes an important approach for the network to explicitly indicate the prioritised candidate cells to UE for cell switch and recovery, without having to specify it through RRCReconfiguration message.

Proposal 2: When early DL or UL synchronization to one or more candidate cells are performed by the UE upon network instruction, then this set of cells are considered as high priority while performing cell selection for fast cell recovery.

Observation 2: Early UL synchronisation depends on the TA value sent to UE before cell switch command. In case RAN2 decides to send the TA value only along with cell switch command, then UE starts RACH-based recovery procedure to the early DL synchronised candidate cell.

Further, when multiple candidate cells are available for recovery, but early synchronisation is not performed, then to assist UE for cell selection according to the identified best set of cells, the list of candidate cells for recovery are configured at UE using a separate RRC re-configuration message or along with the LTM configuration RRC message. And by configuring the recovery candidate cell at UE, the network is expected to prepare these recovery candidate cells with the current UE context information in advance, i.e. by transferring to the corresponding DU in case of inter-DU candidate cells.

Proposal 3: RRCReconfiguration message with a list of (at least one) candidate cells is configured to UE to enable fast cell recovery process, when recovery to the LTM candidate cells is intended by the network.

When multiple candidate cells are intended for recovery and configured at UE, then this implies UE could perform recovery process with any of the candidate cells based on the latest L1 measurements available with the UE after cell switch failure detection or RLF. But, since the channel status using L1 measurement result is not the only metric to determine the best candidate cell for cell switch or recovery, a priority order of the candidate cells is useful for UE to choose the preferred first cell (among LTM candidate cells) for cell selection and RACH transmission. Which means, it enables UE to recover to the best identified cell at the earliest, for example, when candidate cell-1 is having a good radio condition but due to higher traffic congestion with other users it may not be able to provide expected service to the recovered UE, while the candidate cell-2 with slightly bad channel condition having very low traffic congestion can provide higher throughput as compared to cell-1. Conclusively, if cell-2 is to be given more priority than cell-1 during UE cell selection for LTM recovery, the ‘order of preference’ or the ‘priority order’ is also indicated by the network.

Also, since the network conditions are dynamic in nature and also may not always be tracked in real time, it is best to introduce two approaches for indicating the priority order of candidate cells, first using the RRC re-configuration before the cell switch decision and second along with the cell switch MAC CE.

For the case of RRC re-configuration message, the same list of candidate cell configuration which is sent to UE to configure the possible candidate cells for recovery could also incorporate the priority order by arranging the list of candidate cells with the necessary order, within the same IE structure.

Proposal 4: When multiple LTM candidate cells are intended for recovery process, then RRCReconfiguration message with a list of candidate cells are arranged according to the preferred priority order as intended by the network. Based on this prioritized list of candidate cells UE performs cell selection operation for LTM recovery.

Proposal 5: When all the candidate cells in LTM config list are intended for fast recovery process, it is still required to configure all the recovery candidate cells at UE, which provides the priority order among the candidate cells.

Since the non-radio conditions tend to vary dynamically (i.e. the load balancing conditions vary due to the changing number of users serviced by a node at a given time), it is possible that the RRC configuration for candidate cell recovery which was configured when the LTM configuration was initially sent to UE, is not valid by the time the cell switch command is sent. For this reason, it must be flexible for the priority order to be updated even during the time when cell switch is triggered, hence the cell switch MAC CE must have a field to include the recovery candidate cell along with the candidate cell indicated for cell switch. When UE finds recovery candidate cell within the cell switch command then the RRC configuration for cell recovery is overruled and applies recovery process over the indicated recovery cell. And when the recovery process to this recovery candidate cell also fails, then UE enters RRC connection re-establishment as per legacy for cell (re)selection.

Proposal 6: Cell switch MAC CE includes the recovery candidate cell along with the cell switch candidate cell when RRC configured priority list is not valid at the time of cell switch decision, and UE applies recovery configuration to this indicated recovery candidate cell.

During RACH-based cell recovery procedure with one of the LTM candidate cells, and after successful reception of RAR message from the recovery candidate cell, UE could send RRCReconfiguration complete message as the UL transmission to identify the UE arrival after recovering from the cell switch failure or RLF. Because the RRC re-configuration massage from the source cell configures the LTM candidate cells for both LTM cell switch and LTM recovery, and accordingly UE is performing cell recovery with one of the LTM candidate cells configured for LTM recovery. Therefore, it is possible to utilise RRCReconfiguration for UL transmission to the recovered candidate cell, as similar to cell switch operation with the LTM target cell.

Proposal 7: RRCReconfiguration complete message could be sent as the first UL data to the recovered candidate cell, as part of LTM recovery operation.

# Conclusions

Observation 1: Configuring recovery candidate cells at UE provides faster recovery to the best candidate cell, both in the view of radio condition based cell selection and non-radio condition based cell selection criteria.

Proposal 1: UE performs cell selection to one of the LTM candidate cells configured  for recovery. And the LTM recovery cell configuration process must be possible to re-configure according to the dynamic network conditions and channel variations.

Proposal 2: When early DL or UL synchronization to one or more candidate cells are performed by the UE upon network instruction, then this set of cells are considered as high priority while performing cell selection for fast cell recovery.

Observation 2: Early UL synchronisation depends on the TA value sent to UE before cell switch command. In case RAN2 decides to send the TA value only along with cell switch command, then UE starts RACH-based recovery procedure to the early DL synchronised candidate cell.

Proposal 3: RRCReconfiguration message with a list of (at least one) candidate cells is configured to UE to enable fast cell recovery process, when recovery to the LTM candidate cells is intended by the network.

Proposal 4: When multiple LTM candidate cells are intended for recovery process, then RRCReconfiguration message with a list of candidate cells are arranged according to the preferred priority order as intended by the network. Based on this prioritized list of candidate cells UE performs cell selection operation for LTM recovery.

Proposal 5: When all the candidate cells in LTM config list are intended for fast recovery process, it is still required to configure all the recovery candidate cells at UE, which provides the priority order among the candidate cells.

Proposal 6: Cell switch MAC CE includes the recovery candidate cell along with the cell switch candidate cell when RRC configured priority list is not valid at the time of cell switch decision, and UE applies recovery configuration to this indicated recovery candidate cell.

Proposal 7: RRCReconfiguration complete message could be sent as the first UL data to the recovered candidate cell, as part of LTM recovery operation.

# References

[1]		Report of 3GPP TSG RAN WG2 meeting #123, Toulouse, France.

[2]		R2-2309335 - 38.300 running CR for introduction of NR further mobility enhancements.