3GPP TSG-RAN WG2 Meeting #123bis	R2-2311044

Xiamen 9th – 13th Oct, 2023

Source: 		   ZTE Corporation, Sanechips

Title:            Consideration on the Lower MSD Capability Signaling

Agenda item:    7.25.1

Document for:   Discussion and Decision

- Introduction

About the MSD signaling, in the last meeting, RAN2 has agreed that

Lower MSD cap is reported outside BC list Filtering FFS (discussion postponed until more mature) In the signaling, victim / aggressor need to be identified

In this paper, we will share our further thinking on the signaling design based both the RAN2 and RAN4’s progress.

- Discussion

2.1 General

To report the MSD capability, two options are mentioned in the previous submitted papers as shown in Fig 1:

- Option 1：Report MSD capability per Aggressor Band Pair [1];

- Option 2：Report MSD capability per Victim Band [2].

Proposal 1: To report the MSD capability, two options can be considered:

- Option 1：Report MSD capability per Aggressor Band Pair;

- Option 2：Report MSD capability per Victim Band.

2.2 Detail Description on Each Option

Before we give more detail description, we’d like to define some common structure as below:

FreqBandIndicatorNRPair       ::=		SEQUENCE {

band1    FreqBandIndicatorNR,

band2    FreqBandIndicatorNR

}    ----2x bits

MSD1 ::=		SEQUENCE {

MSD1-r18		ENUMERATED {harmonic, harmonicmix, crossBandIso,ALL},

msdPowerClass-r18	ENUMERATED {pc1dot5, pc2, pc3},

msdThreshold-r18	ENUMERATED {xx}

}    ---- y1 bits

MSD2  ::=		SEQUENCE {

MSD2-r18			ENUMERATED {IMD2, IMD3, IMD4, IMD5,ALL ...},

msdPowerClass-r18     	ENUMERATED {pc1dot5, pc2, pc3},

msdThreshold-r18	     ENUMERATED {xx}

}    ----y2 bits

In this common structure, the MSD1 is the non-IMD MSD type that caused by a single band, while the MSD2 is the IMD type that caused by the band pair.

Detail for the Option 1

In the option 1: the MSD was reported per aggressor band pair, and the Asn.1 coding can be generally as below:

lowerMSD-r18	SEQUENCE (SIZE (1..maxLowerMSD)) OF LowerMSD-r18

LowerMSD-r18 ::=		SEQUENCE {

aggressorbandPair                FreqBandIndicatorNRPair,

internalVictimBandMSD1           MSD1  	OPTIONAL,

internalVictimBandMSD2           MSD1     OPTIONAL,

externalVictimbandMSDList-r18  ExternalVictimbandMSDList-r18				OPTIONAL

}

ExternalVictimbandMSDList-r18	SEQUENCE (SIZE (1..FFS)) OF  ExternalVictimbandMSD        OPTIONAL

ExternalVictimbandMSD ::=		SEQUENCE {

victimBand                FreqBandIndicatorNR,

msd2                        MDS2	}

In which

- internalVictimBandMSD1/ internalVictimBandMSD2: indicates the internal MSD between the band pair, for example the band pair = band 1+band2, internalVictimBandMSD1 indicates the MSD on the band 1 that caused by the band 2, while the internalVictimBandMSD2 indicates the MSD on the band 2 that caused by the band 1;

- externalVictimbandMSDList-r18: indicates the victim bands that aggressed by this band pair.

Detail for the Option 2

In the option 2: the MSD was reported per victim band, and the Asn.1 coding can be generally as below:

lowerMSD-r18	SEQUENCE (SIZE (1..maxLowerMSD)) OF LowerMSD-r18

LowerMSD-r18 ::=		SEQUENCE {

victimBand                     FreqBandIndicatorNR,

aggressorbandList-r18	     AggressorbandList-r18				OPTIONAL,

aggressorbandPairList-r18	AggressorbandPairList-r18		     OPTIONAL,

}

AggressorbandList-r18        SEQUENCE (SIZE (1..FFS)) OF Aggressorband

Aggressorband ::=		SEQUENCE {

victimBand            FreqBandIndicatorNR,

msd1                   MDS1

}

AggressorbandPairList-r18	SEQUENCE (SIZE (1..FFS)) OF Aggressorbandpair

Aggressorbandpair ::=		SEQUENCE {

bandPair                FreqBandIndicatorNRPair,

msd2                        MDS2

}

In which

- aggressorbandList-r18: indicates the aggressor band list that has non-IMD MSD impact on the victimBand

- aggressorbandPairList-r18: indicates the aggressor band pair list that has IMD MSD impact on the victimBand

2.3 Option 1 vs Option

In this chapter, these 2 options would be compared from the signaling overhead perspective. We assume that the UE would report the lower MSD capability for the band A/B/C/D. Each band has harmonic MSD on the other Band, and each band pair has IMD MSD on rest bands. The UE would report the MSD as in the Table 1 and Table 2 for the option 1 and option 2 respectively.

Table 1: UE report MSD with Option 1

|  | Aggressor bandPair | internalVictimBandMSD1/MSD2 | MSD Type | ExternalVictimbandMSD | MsdType |
| --- | --- | --- | --- | --- | --- |
| 1 | A+B | A/B | MSD1 | A/B/C/D | MSD2 |
| 2 | A+C | A/C | MSD1 | A/B/C/D | MSD2 |
| 3 | A+D | A/D | MSD1 | A/B/C/D | MSD2 |
| 4 | B+C | B/C | MSD1 | A/B/C/D | MSD2 |
| 5 | B+D | B/D | MSD1 | A/B/C/D | MSD2 |
| 6 | C+D | C/D | MSD1 | A/B/C/D | MSD2 |

Table 2: UE report MSD with Option 2

|  | Victim | AggressorbandList | MSD Type | AggressorbandPairList | MSD Type |
| --- | --- | --- | --- | --- | --- |
| 1 | A | B/C/D | MSD1 | A+B/A+C/A+D/B+C/B+D/C+D | MSD2 |
| 2 | B | A/C/D | MSD1 | A+B/A+C/A+D/B+C/B+D/C+D | MSD2 |
| 3 | C | A/B/D | MSD1 | A+B/A+C/A+D/B+C/B+D/C+D | MSD2 |
| 4 | D | A/B/C | MSD1 | A+B/A+C/A+D/B+C/B+D/C+D | MSD2 |

From this table, we can see that compared with option 1, option 2 needs more bits to indicate the different band pairs for each victim band. If we give a quantitative analysis, the overhead of each option is as below:

- Option 1: 6*[(2x+2y1+m+4*(x+y2)]        = 36x+12y1+24y2+6m

- Option 2: 4*[(x+2m+3*(x+y1)+6*(2x+y2)]   =64x+12y1+24y2+8m

In which the “x” indicates the bits of one band, “y1” indicates the bits the MSD1, while the “y2” indicate the bits of MSD2 as described in the commons structure of chapter 2.2. “m” indicates the bits that used to describe the size of the sequence.

Proposal 2: The Option 1 (Report MSD capability per Aggressor Band Pair) can be taken as a baseline from the signaling overhead perspective.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: To report the MSD capability, two options can be considered:

- Option 1：Report MSD capability per Aggressor Band Pair;

- Option 2：Report MSD capability per Victim Band.

Proposal 2: The Option 1 (Report MSD capability per Aggressor Band Pair) can be taken as a baseline from the signaling overhead perspective.

- Reference

- R2-2307543	Consideration on Lower MSD Capability Signaling	ZTE Corporation, Sanechips	discussion	Rel-18	NR_ENDC_RF_FR1_enh2

- R2-2308863	Discussion on lower MSD capability	Huawei, HiSilicon	discussion	Rel-18	NR_ENDC_RF_FR1_enh2

- R2-2308864	Introduction of lower MSD capability	Huawei, HiSilicon	CR	Rel-18	38.331	17.5.0	4292	-	B	NR_ENDC_RF_FR1_enh2

- R2-2308865	Introduction of lower MSD capability	Huawei, HiSilicon	CR	Rel-18	38.306	17.5.0	0950	-	B	NR_ENDC_RF_FR1_enh2

- R4-2310276        LS on lower MSD capability (Contact Huawei)  Ran4