3GPP TSG-RAN WG2 Meeting #123-bis                                                      	  R2-2311093

Xiamen, China, October 9 – 13, 2023

Agenda item: 		7.4.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on early TA acquisition and RACH-less LTM

Document for:	Discussion

# 1	Introduction

In RAN2#123 the following agreements were achieved [1]:

- Define the association between CG occasion and beam in RRC and specify that the UE uses a CG occasion associated with the indicated beam in MAC - Observation: In cases for which it is desired that CG used for LTM is not used further once the UE has made the cell its new serving cell, it is assumed that the network could release Type1 CG resource on LTM completion (existing functionality) - Before RACH-less LTM procedure completion, the UE shall not trigger RACH (when the UE has no valid PUCCH resource for triggered SRs), as in LTE RACH-less. - RAN2 assumes: For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, (FFS if specified contents should be transmitted with this transmission, e.g., as LTE MAC CE). - FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure. - Confirm that the RACH procedure toward a candidate cell is considered as complete once the preamble transmission is instructed to the lower layer. - Automatic retransmission by timer with CG (similar to NR-U, SDT) is supported for the first UL data transmission with CG. - All the RRC configurations related to early RACH are specific per LTM candidate cell and signalled separately from the candidate cell configuration (i.e. LTM Candidate configuration). - The early RACH procedure share a same MAC entity with the legacy RACH procedure. (e.g. no extra MAC entity is needed for early RACH) - It is up to UE implementation to handle the RACH initiation collisions where the early RACH is getting involved. No specification change can be foreseen. - R2 assumes For counting the power ramping step for early RACH, Reuse PREAMBLE_POWER_RAMPING_COUNTER

In RAN2#122 the following agreements were achieved [2]:

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. - Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

In RAN2#121bis-e the following agreement was achieved [3]:

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

In RAN3#121 the following working assumptions were made [4]:

- Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2. - WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU

This contribution discusses aspects of early TA acquisition and RACH-less LTM.

# 2	Early TA acquisition

## 2.1	Power ramping for PDCCH-ordered RACH during LBT failure

RAN2#123 had the following agreement: “FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure.”

The power is ramped up for PRACH retransmissions based on the assumption that the network failed to receive the prior PRACH transmission due to transmitting the PRACH with low power value by the UE. In the case of LBT failure, the UE did not transmit PRACH at all. Thus, power ramping in this case should be skipped.

Proposal 1: UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if prior preamble transmission encounters LBT failure.

## 2.2	Sharing of PRACH resources for early TA acquisition among UEs

RAN2#121bis-e agreed that CFRA resources for early TA acquisition can be shared among UEs based on the following agreement:

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

In the inter-DU scenario, the candidate DU cannot tell which UE used the shared RACH resource, so the candidate DU cannot associate the computed TA value to a particular UE. To resolve this issue, there are two solutions:

- Solution 1: the serving DU and the candidate DU coordinate before the UE is ordered to send the PRACH. The candidate DU ensures no two UEs send PRACH using the same CFRA resource at any time.

- Solution 2: CFRA resource sharing is only allowed among UEs of the same serving DU, which implies the candidate DU can determine which serving DU ordered the PRACH transmission. Then, the candidate DU forwards the TA result to the serving DU, which associates the received TA value to the right UE.

RAN3#121 already made working assumptions along the lines of Solution 2:

- Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

- WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU

Furthermore, Solution 1 introduces extra backhaul latency and signaling overhead. RAN3 is further calling to align with RAN2 on this issue. Based on this, we propose the following:

Proposal 2: Confirm RAN3’s WA that only UEs of the same gNB-DU may share RACH resources for early TA acquisition.

# 3	Aspects of RACH-less LTM

## 3.1	Grant for first UL transmission

RAN2#122 agreed two methods to provide an UL grant to the UE for the first UL data transmission to the target cell:

- Method 1: based on dynamic grant

- Method 2: based on configured grant

Which method the UE applies should be based on configuration:

- If the UE is configured with PDCCH search space of the target cell, Method 1 applies.

- If the UE is configured with a configured grant for the target cell, Method 2 applies.

- If the UE is configured with PDCCH search space of the target cell and a configured grant for the target cell, both methods apply.

Proposal 3: At LTM triggering, the UE may receive a dynamic grant and/or configured grant for the first UL data transmission based on whether the RRC configuration of the target cell includes a PDCCH search space of the target cell and/or a configured grant for the target cell respectively. The two configurations are not exclusive.

## 3.2	Beam transfer from source DU to target DU

RAN2#122 assumed that the source DU informs the target DU about the selected beam for RACH-less LTM execution for the case that the UE’s first UL transmission on the target cell uses a dynamic grant. Since the source DU neither knows whether the UE is configured with a configured grant nor can tell the type of grant that will be used on the target cell (since dynamic grant and configured grant can co-exist as per the previous section), the source DU should always transfer the selected beam to the target DU.

Proposal 4: For RACH-less LTM, RAN2 assumes the source DU always informs the target DU about the selected beam to give the target DU the possibility to use a dynamic grant even if CG is configured.

## 3.3	Determination of successful LTM completion

RAN2#123 assumed that:

- For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, (FFS if specified contents should be transmitted with this transmission, e.g., as LTE MAC CE).

Several companies propose that the PDCCH addressing the UE’s C-RNTI in the target cell also schedules the UE Contention Resolution MAC CE as in LTE RACH-less scheme. However, this is not needed for NR LTM based on the following observation:

Observation 1: In LTE, the CRC for PDCCH is only 16 bits, whereas in NR, the CRC for PDCCH is 24 bits. While the UE contention resolution MAC CE is needed for LTE RACH-less scheme to minimize the false alarm rate for determining successful handover completion, it is redundant for NR.

We propose the following:

Proposal 5: The UE determines successful RACH-less LTM execution upon being scheduled by the target cell for a new transmission (as agreed). No content of a PDSCH payload is specified.

## 3.4	Resume of DL scheduling by the target cell

The UE monitors for PDCCH from the target cell if the UE is configured with PDCCH search space of the target cell. PDCCH may schedule the UE in either UL or DL. It is up to the network whether PDCCH from the target cell schedules DL first before the UE sends its first UL data transmission.

Proposal 6: At LTM triggering, the UE may receive a dynamic grant from the target cell for DL scheduling prior to the UE’s first UL transmission to minimize DL interruption time. This is based on network implementation.

# Conclusion

This contribution discussed aspects of early TA acquisition and RACH-less LTM. The following proposals and observation have been made:

Early TA acquisition:

Proposal 1: UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if prior preamble transmission encounters LBT failure.

Proposal 2: Confirm RAN3’s WA that only UEs of the same gNB-DU may share RACH resources for early TA acquisition.

RACH-less LTM:

Proposal 3: At LTM triggering, the UE may receive a dynamic grant and/or configured grant for the first UL data transmission based on whether the RRC configuration of the target cell includes a PDCCH search space of the target cell and/or a configured grant for the target cell respectively. The two configurations are not exclusive.

Proposal 4: For RACH-less LTM, RAN2 assumes the source DU always informs the target DU about the selected beam to give the target DU the possibility to use a dynamic grant even if CG is configured.

Observation 1: In LTE, the CRC for PDCCH is only 16 bits, whereas in NR, the CRC for PDCCH is 24 bits. While the UE contention resolution MAC CE is needed for LTE RACH-less scheme to minimize the false alarm rate for determining successful handover completion, it is redundant for NR.

Proposal 5: The UE determines successful RACH-less LTM execution upon being scheduled by the target cell for a new transmission (as agreed). No content of a PDSCH payload is specified.

Proposal 6: At LTM triggering, the UE may receive a dynamic grant from the target cell for DL scheduling prior to the UE’s first UL transmission to minimize DL interruption time. This is based on network implementation.

# References

- Chairman Notes; TSG RAN WG2 Meeting #123

- Chairman Notes; TSG RAN WG2 Meeting #122

- Chairman Notes; TSG RAN WG2 Meeting #121bis-e

- Chairman Notes; TSG RAN WG3 Meeting #121