3GPP TSG-RAN WG2 Meeting #123bis		R2-2311097

Xiamen, China, Oct 9 – 13, 2023

Agenda Item:	7.4.4

Source: 	LG Electronics

Title:  	Simultaneous Execution of CHO and CPAC

Document for:	Discussion

Introduction

In this contribution, we discuss the leftover issues for simultaneous execution of CHO and CPA/CPC.

Discussion

CHO Failure Recovery

In the previous RAN2 meeting, RAN2 left the below leftover issues [1]:

CHO recovery details to handle the additions brought by this feature is FFS

According to the 37.340 specifications, CHO can already include SCG configuration when performing handovers [2]:

## 10.1 General Similar procedures as defined under clause 10.1.2.8 (Dual Connectivity operation) in TS 36.300 [2] apply for MR-DC. Similar CHO principles as defined in TS 36.300 [2] and TS 38.300 [3] apply for the Conditional PSCell Change and Conditional PSCell Addition in MR-DC. Conditional PSCell Change and conditional PSCell addition are not supported for the MR-DC options NE-DC and NGEN-DC. Configuration of a deactivated SCG in a conditional configuration, configuration of CPC while the SCG is deactivated and SCG deactivation while CPC is configured are not supported. In MR-DC, CHO is supported in Master Node to eNB/gNB Change procedure and Conditional Handover with Secondary Node procedure.

CHO configuration already supports handover including SCG configuration. Based on the CHO+SCG configuration, Rel-17 UEs can performs the CHO based recovery. In other words, the UE performs a cell selection procedure, and if the selected cell is one of the candidate PCells of CHO+SCG, the UE initiates the CHO-based recovery based on CHO+SCG configuration.

Observation 1.	According to the current specification, the CHO recovery procedure with CHO+SCG can be initiated by only cell selection procedure for PCell.

The current design for CHO+CPC is structurally identical to the legacy case of CHO+SCG, with the exception of additional execution conditions for CPC. If the UE performs the cell selection procedure, and if the selected cell is one of the candidate PCells of CHO+CPC, the UE can perform the CHO-based recovery without any additional issues. This is because the recovery is initiated by the cell selection procedure of RRC Re-establishment, not by the provided execution conditions for handover.

While RAN2 has discussed CHO+CPC for SN mobility robustness increase when performing handover, the primary objective of the CHO-based recovery is to support fast recovery of the MCG. Thus, we believe that the existing CHO-based recovery should also be supported for CHO+CPC without any additional UE behaviour.

Proposal 1:	The CHO recovery procedure with CHO+CPC/CPA, is initiated by the existing cell selection procedure without any new UE behaviour.

  - Candidates Management

The candidate PSCell list prepared for CHO+CPC can have different cell lists depending on the target PCell for each CHO. Thus, candidate PSCell lists unrelated to the target PCell of CHO when executing CHO can be regarded as not valid configurations. However, when executing CHO, the candidate PSCell list related to the related target PCell may still be a valid configuration even if CHO is executed because this candidate PSCell list can be still working with the target PCell after CHO.

Observation 2.	Each candidate PSCell for CHO+CPC can become a candidate PSCell that cannot be supported depending on the target PCell for CHO. But, each candidate PSCell for CHO+CPC can continue to be used after CHO+CPC if the target PCell for CHO is the same cell.

According to the current principle, the UE removes all configured conditional reconfigurations after performing CHO, CPC, or CPA. However, it is not helpful for the UE not to maintain the list of candidate PSCells that can still be valid after CHO is complete. RAN2 does not yet have an exact agreement but knows that it is beneficial for the UE to keep the list of candidate PSCells provided for CHO+CPC even when only CHO has been executed. In the same context, even when CHO+CPC is simultaneously executed, if a valid candidate PSCell list other than the selected candidate PSCell as the target PSCell can be a beneficial configuration for the subsequent CPC in terms of mobility robustness.

Proposal 2.	Upon successful completion of CHO+CPC with other candidate PSCells, these candidate PSCells associated with the target PCell are maintained for the next CPC.

Conclusion

It is proposed to discuss and decide on the following proposals:

* CHO Failure Recovery

Observation 1.	According to the current specification, the CHO recovery procedure with CHO+SCG can be initiated by only cell selection procedure for PCell.

Proposal 1:	The CHO recovery procedure with CHO+CPC/CPA, is initiated by the existing cell selection procedure without any new UE behaviour.

* Candidate Management

Observation 2.	Each candidate PSCell for CHO+CPC can become a candidate PSCell that cannot be supported depending on the target PCell for CHO. But, each candidate PSCell for CHO+CPC can continue to be used after CHO+CPC if the target PCell for CHO is the same cell.

Proposal 2.	Upon successful completion of CHO+CPC with other candidate PSCells, these candidate PSCells associated with the target PCell are maintained for the next CPC.

Reference

- R2-23xxxx, “RAN2 Chairman Notes,” MediaTek, Aug 2023

- 3GPP 37.340 v17.5.0 (2023-06)