3GPP TSG-RAN WG2 #123bis	Tdoc R2-2311180

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.20.1

Source:	Ericsson

Title:	Response to LS on Dual TCI state switching in mDCI

Document for:	Discussion

# 1	LS on Dual TCI state switching in mDCI

RAN2 has received LS in R4-2314479 “LS on Dual TCI state switching in mDCI” with the below content:

# 1	Overall description

RAN4 is discussing requirements for DCI based TCI state switching in mDCI scenario for UE capable of multi-rx operation in FR2-1 and have following questions.

Q1 to RAN1: Based on the illustration of figure 1,

- RAN4 understands that, minimum duration between point A and C should not be smaller than timeDurationForQCL, which is already defined in RAN1 specification.

- RAN4 understands that, minimum duration between point B and D should not be smaller than timeDurationForQCL, which is already defined in RAN1 specification.

- Based on RAN4 discussion, it is identified that when the duration between point B and C is smaller than timeDurationForQCL, some UE implementations may not be able to perform dual TCI state switching for simultaneous PDSCH reception with different QCL type-D. RAN4 would like to check whether there is any minimum duration defined in RAN1 specifications for duration between point B and C.

  - If No,

    - when duration between point B and C is smaller than timeDurationForQCL, what is the expected UE behaviour after point C., e.g., what are TCI states assumptions after point C to perform simultaneous PDSCH reception with different QCL type-D till the NW provides UE with new TCI state indication?

    - Does RAN1 sees the need to define such minimum duration between B and C to address potential UE implementation complexity for some UE implementations. If RAN1 sees the necessity, RAN4 kindly requests RAN1 to introduce such restriction in RAN1 specification as DCI based TCI state switching requirements in RAN4 specification refers to RAN1 specification.

Figure 1: Example mDCI scenario

Q2 to RAN1: In mDCI scenario, can network configure two PDCCH transmission simultaneously with different QCL type D which are associated with different CoresetPoolIndex to UE?

- If yes, can UE receive two PDCCHs simultaneously with different QCL type D which are associated with different CoresetPoolIndex?

Q3 to RAN1 and RAN2: Can RAN1 and RAN2 confirm if the RRC based TCI state switch (without MAC CE) is supported for the following scenario.

- Two TCI states are configured in the RRC configured TCI state list. Can UE perform PDCCH TCI state switch for individual TCI states without waiting for MAC CE command (i.e., RRC reconfiguration directly triggering TCI state switch for PDCCH for mDCI).

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to provide answers to above questions Q1-Q3 to progress further RAN4 work.

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to provide answers to the above question Q3 to progress further RAN4 work.

3. Date of Next RAN4 Meeting:

TSG-RAN WG4#108bis	                October 9 - 13, 2023		Xiamen, China

TSG-RAN WG4#109	                November 13 - 17, 2023	Chicago, US

# 2	TCI state switching for mTRP mDCI

The TCI state is configured for the PDCCH in IE ControlResourceSet, where each coreset is also assigned a coresetPoolID in case of mDCI mTRP operation. All coresets assigned with same corestePoolID belong to same TRP.

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see TS 38.213 [13], clause 10.1). For the UE not supporting multipleCORESET in FR1, in order to receive MBS multicast in CFR within the UE's active BWP, if a CORESET is not configured within the PDCCH-ConfigMulticast, the CORESET other than CORESET#0 configured within the UE's active BWP for scheduling unicast can be used for scheduling MBS multicast, and the CORESET is expected to be included completely within the CFR and the parameters configured in the CORESET are expected to be supported by the UE for MBS multicast.

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=              SEQUENCE {

controlResourceSetId                ControlResourceSetId,

frequencyDomainResources            BIT STRING (SIZE (45)),

duration                            INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                 CHOICE {

interleaved                         SEQUENCE {

reg-BundleSize                      ENUMERATED {n2, n3, n6},

interleaverSize                     ENUMERATED {n2, n3, n6},

shiftIndex                          INTEGER(0..maxNrofPhysicalResourceBlocks-1)       OPTIONAL -- Need S

},

nonInterleaved                      NULL

},

precoderGranularity                 ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-StatesPDCCH-ToReleaseList       SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-PresentInDCI                        ENUMERATED {enabled}                                  OPTIONAL, -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                                    OPTIONAL, -- Need S

...,

[[

rb-Offset-r16                       INTEGER (0..5)                                            OPTIONAL, -- Need S

tci-PresentDCI-1-2-r16              INTEGER (1..3)                                            OPTIONAL, -- Need S

coresetPoolIndex-r16                INTEGER (0..1)                                            OPTIONAL, -- Need S

controlResourceSetId-v1610          ControlResourceSetId-v1610                                OPTIONAL  -- Need S

]],

[[

followUnifiedTCI-State-r17           ENUMERATED {enabled}                                      OPTIONAL  -- Need R

]]

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG) (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Identifies the instance of the ControlResourceSet IE. Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Other values identify CORESETs configured by dedicated signalling or in SIB1 or SIB20. The controlResourceSetId is unique among the BWPs of a serving cell. If the field controlResourceSetId-v1610 is present, the UE shall ignore the controlResourceSetId field (without suffix).

coresetPoolIndex The index of the CORESET pool for this CORESET as specified in TS 38.213 [13] (clauses 9 and 10) and TS 38.214 [19] (clauses 5.1 and 6.1). If the field is absent, the UE applies the value 0.

duration Contiguous time duration of the CORESET in number of symbols (see TS 38.211 [16], clause 7.3.2.2).

followUnifiedTCI-State When set to enabled, for PDCCH reception on this CORESET, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group in the BWP or MBS CFR where the CORESET is configured. When at least one search space is configured with freqMonitorLocation-r16, only the first NRBG,set0size bits are valid (see TS 38.213 [13], clause 10.1). The first (left-most / most significant) bit corresponds to the first RB group in the BWP or MBS CFR where the CORESET is configured, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero (see TS 38.211 [16], clause 7.3.2.2).

interleaverSize Interleaver-size (see TS 38.211 [16], clause 7.3.2.2).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.3.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles (see TS 38.211 [16], clause 7.3.2.2).

shiftIndex When the field is absent the UE applies the value of the physCellIdconfigured for this serving cell (see TS 38.211 [16], clause 7.3.2.2).

tci-PresentInDCI This field indicates if TCI field is present or absent in DCI format 1_1 and DCI format 4_2. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in DCI format 1_1 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-PresentDCI-1-2 Configures the number of bits for "Transmission configuration indicator" in DCI format 1_2. When the field is absent the UE applies the value of 0 bit for the "Transmission configuration indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.5). In case of cross carrier scheduling, the network configures this field for the ControlResourceSet used for cross carrier scheduling in DCI format 1_2 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-StatesPDCCH-ToAddList A subset of the TCI states defined in pdsch-Config, either with tci-StatesToAddModList or dl-OrJointTCI-StateList, included in the BWP-DownlinkDedicated corresponding to the serving cell and to the DL BWP to which the ControlResourceSet belong to. They are used for providing QCL relationships between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports (see TS 38.213 [13], clause 6.). The network configures at most maxNrofTCI-StatesPDCCH entries. The QCL relationships defined herein do not apply to MBS broadcast.

| Conditional Presence | Explanation |
| --- | --- |
| NotSIB-initialBWP | The field is absent in SIB1/SIB20 and in the PDCCH-ConfigCommon of the initial BWP in ServingCellConfigCommon, if SIB1/SIB20 is broadcasted. Otherwise, it is optionally present, Need N. |

TS 38.321 specifies a MAC CE which can be used in indicating the TCI state for a specific coreset:

#### 6.1.3.15	TCI State Indication for UE-specific PDCCH MAC CE

The TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-StatesToAddModList and tci-StatesToReleaseList in the PDSCH-Config in the active BWP or by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE

Regarding scenario in Question 3, the operation is the same also for single TRP Release-15 operation. For example network may have configured the UE with number of coresets and the TCI states for each coreset. After this, the network sends the MAC CE to indicate one TCI state out of the list. Thereafter, network may either send another MAC CE to switch the indicated TCI state. Alternatively, network can send RRC reconfiguration to the UE and configure new TCI states for the UE. However, in order for the UE to have one TCI state per coreset, the list size needs to be 1. Otherwise a MAC CE is also needed to indicate a TCI state from the list. The described operation is the same regardless of whether UE is configured with coresetPoolID to group the coresets for mTRP mDCI operation or not.

RAN2 to send LS to RAN4 to explain that TCI switching via RRC works in the same manner for sTRP, and mTRP mDCI, that is regardless of whether  parameter coresetPoolIndex is configured or not.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to send LS to RAN4 to explain that TCI switching via RRC works in the same manner for sTRP, and mTRP mDCI, that is regardless of whether  parameter coresetPoolIndex is configured or not.

# Draft LS to RAN4

3GPP TSG-RAN WG2 Meeting #124bis 	R2-22110xx

# Electronic April 2023

Title:	DRAFT Response LS to LS on Dual TCI state switching in mDCI

Response to: 	R2-XXXXXX

Release:	Rel-15

Work Item:	NR_newRAT-Perf

Source:	Ericsson (to be replaced with 3GPP TSG-RAN WG2)

To:	3GPP TSG-RAN WG4

Contact Person:

Name:	Helka-Liina Määttänen

Email Address:	Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

RAN2 would like to inform RAN4 that TCI switching via RRC works in the same manner for sTRP, and mTRP mDCI, that is regardless of whether  parameter coresetPoolIndex is configured or not.

3. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN4 to take the response into account in their work.

4. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #124 	November 2023    Chicago

TSG-RAN WG2 Meeting #125	February 2024      Athens