3GPP TSG-RAN WG2 #123bis	R2-2311193

# Xiamen, China, 9 – 13 Oct 2023

Agenda Item:	5.1.3.1

Source:	Ericsson

Title:	Discussion on lowest subband for CSI reporting

Document for:	Discussion

# 1	Introduction

RAN1 has identified that there were ambiguities of how the field csi-ReportingBand was interpreted which can result in that CSI report may be decodable incorrectly.

# 2	Discussion

## 2.1	Correcting the csi-ReportingBand field description

csi-ReportingBand is a bitstring of size 3 – 19 bits. The field description incorrectly captures that the right-most bit in the bit string represents the lowest subband in the BWP:

#### – CSI-ReportConfig The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1. CSI-ReportConfig information element -- ASN1START -- TAG-CSI-REPORTCONFIG-START CSI-ReportConfig ::= SEQUENCE { ... reportFreqConfiguration SEQUENCE { csi-ReportingBand CHOICE { subbands3 BIT STRING(SIZE(3)), subbands4 BIT STRING(SIZE(4)), subbands5 BIT STRING(SIZE(5)), subbands6 BIT STRING(SIZE(6)), subbands7 BIT STRING(SIZE(7)), subbands8 BIT STRING(SIZE(8)), subbands9 BIT STRING(SIZE(9)), subbands10 BIT STRING(SIZE(10)), subbands11 BIT STRING(SIZE(11)), subbands12 BIT STRING(SIZE(12)), subbands13 BIT STRING(SIZE(13)), subbands14 BIT STRING(SIZE(14)), subbands15 BIT STRING(SIZE(15)), subbands16 BIT STRING(SIZE(16)), subbands17 BIT STRING(SIZE(17)), subbands18 BIT STRING(SIZE(18)), ..., subbands19-v1530 BIT STRING(SIZE(19)) } OPTIONAL -- Need S } ... } -- TAG-CSI-REPORTCONFIG-STOP -- ASN1STOP CSI-ReportConfig field descriptions. csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4)..

RAN1 endorsed a correction to Release-17 version of 38.212 (see R1-2212995):

Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to ‘1’ are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0.

RAN1 also endorsed a correction to Release-17 version of 38.214 (see R1-2212996):

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand with value ‘1’ are numbered continuously in increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

RAN2 should update the field description to not have contradicting wording compared to the RAN1 changes. The best approach in our view is to remove the ambiguous wording in RRC and instead refer to RAN1 specifications, for example:

CSI-ReportConfig field descriptions

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband as described in TS 38.212 [17] and TS 38.214 [19]. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4).

Even it would be possible to add same clarification to TS 38.331 than it was done for both TS 38.212 and TS 38.214, it would mean the same item is described in three different specification.

Remove “The right-most bit in the bit string represents the lowest subband in the BWP.” from the field description of csi-ReportingBand.  And add references to 38.212 and 38.214 in the field description of csi-ReportingBand.

## 2.2	Capability indication

The impact analysis in the endorsed RAN1 CRs state:

“When network and UE have different understanding of the interpretation the CSI report may not be decodable correctly.”.

To ensure that the gNB knows that the UE and gNB has the same interpretation of this field, a UE capability needs to be added which indicates that UE has implemented the corrected interpretation.

A capability indication needs to be added to indicate if the UE has implemented the corrected interpretation.

## 2.3	From which release?

RAN1 discussed from which release this should be corrected. RAN1’s endorsed CRs are only from Rel-17.

If the change is done only from Rel-17, it means that any UE chipset vendor who wants to be able to use correct CSI reporting cannot do so for their Rel-16 or Rel-15 UEs since the feature needs to be considered broken. We think this would impose an unnecessary restriction for the UE’s. Instead, we suggest that we can do this change from Rel-15. Then, RAN2 needs to inform RAN2 about RAN2 decicion to make the correction from Release-15.

Correct the field description and add a UE capability from Rel-15.

Inform RAN1 about RAN2 decision.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove “The right-most bit in the bit string represents the lowest subband in the BWP.” from the field description of csi-ReportingBand.  And add references to 38.212 and 38.214 in the field description of csi-ReportingBand.

Proposal 2	A capability indication needs to be added to indicate if the UE has implemented the corrected interpretation.

Proposal 3	Correct the field description and add a UE capability from Rel-15.

Proposal 4	Inform RAN1 about RAN2 decision.

# 6	Annex – Draft LS

3GPP TSG-RAN2 Meeting #123bis R2-23xxxxx # Xiamen, P.R. China, 9-13 October 2023 Title: LS on clarification of csi-ReportingBand handling Response to: - Release: Rel-15 Work Item: - Source: Ericsson (to be RAN WG2) To: RAN WG1 Cc: - Contact person: Helka-Liina Määttänen helka-liina.maattanen@ericsson.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: - # 1 Overall description RAN2 noticed that RAN1 has endorsed a correction to 38.212 in R1-2212995, and a correction to 38.214 in R1-2212996 to address an ambiguity regarding the field csi-ReportingBand. RAN2 discussed the issue and updated the field description as follows: csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband as described in TS 38.212 [X] and TS 38.214 [Y]. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4). Further, RAN2 understands that it is important for the network to know which interpretation the UE has regarding this field. RAN2 will add a UE capability indication which indicates, if present, that the UE applies the corrected interpretation captured by the RAN1 CRs R1-2212995 and R1-2212996. RAN2 did this change from Rel-15, and encourages RAN1 to also adopt this change from Rel-15. # 2 Actions To RAN WG4 ACTION: RAN2 asks RAN1 to take the above into account in their future work. # 3 Dates of next TSG RAN WG2 meetings RAN2#124 November 13-17, 2023 Chicago, USA RAN2#125 February 26 – March 1, 2024 Athens, Greece