3GPP TSG-RAN WG2 Meeting #123bis	R2-2311246

# Xiamen, China, Oct. 9 – 13, 2023

Agenda Item:	7.5.4.3

Source:	Ericsson

Title:	Configured Grant enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

Here we discuss the remaining issues for configured grants.

# 2	Configured Grant enhancement

## 2.1	RAN1 request for higher layer parameters

RAN1 has provided the needed RRC parameters in [1].

Regarding the number of slots in one periodicity RAN1 asks for:

| nrofSlots_InCGperiod | new |  | Number of consecutive slots for CG PUSCH transmission occasions in a period of a single CG PUSCH configuration | INTEGER(2..32) |  | Per BWP, in ConfiguredGrantConfig |
| --- | --- | --- | --- | --- | --- | --- |

In ConfiguredGrantConfig, introduce a new field cg-Slots-r18 with range INTEGER(2..32), need code R, and field description “Indicates the number of consecutive slots for CG PUSCH transmission in one periodicity.”

Replace the numberOfPUSCH-PerPeriod (and the inconsistently used numberOfPUSCH_PerPeriod) in the MAC running CR with cg-Slots.

Regarding UTO-UCI, RAN1 ask for:

| nrof_UTO_UCI | new |  | Indication of UTO-UCI is enabled if nrof_UTO_UCI is configured. nrof_UTO_UCI is the number of bits in the UTO-UCI bitmap. | INTEGER(3..8) |  | Per BWP, in ConfiguredGrantConfig |
| --- | --- | --- | --- | --- | --- | --- |
| Beta-offset_UTO_UCI | new |  | Beta offset value for UTO-UCI multiplexing on CG PUSCH. Beta-offset_UTO_UCI is required to be present when nrof_UTO_UCI field is configured. | INTEGER (0..31) |  | Per BWP, in ConfiguredGrantConfig |

In ConfiguredGrantConfig, introduce a new field cg-UTO-UCI-r18 with range INTEGER(3..8), need code R, and field description “Enables UTO-UCI and sets the number of bits in the UTO-UCI bitmap.”

In ConfiguredGrantConfig, introduce a new field betaOffset-UTO-UCI-r18 with range INTEGER(0..31), need code R, and field description “Beta offset for UTO-UCI. The ASN-1 shall be written such that the field is always configured if cg-UTO-UCI is configured.

## 2.2	The running MAC CR

MAC shall be aware of TOs that will not be used such that MAC PDUs obtained are not dropped in the PHY layer.

This was the reason to include the question sent in the LS to RAN1 [2] asking for a reference where invalidity is described in the RAN1 specification.

The current running MAC CR has this:

And

To determine the HPID, the running MAC CR only consider valid grants, thus a grant that is invalid will not have a HPID. There is ambiguity in the line “2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;” when the grant is invalid.

- If the procedural text “2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;” is reached for invalid grants, there is ambiguity in UE behaviour, some UEs may drop the grant before obtaining an MAC PDU and some UEs may not.

To avoid this ambiguity, we propose:

The reference to transmission opportunities that PHY determines to be invalid shall be added in running MAC spec in the sentence “For each Serving Cell and each configured uplink grant, if configured and activated and has not been indicated by the MAC entity to the lower layers as to be unused for PUSCH transmission, the MAC entity shall:”.

## 2.3	Dynamic indication of unused CG

The scheduling and the decision of whether to use or not use a CG resource depends on available data status which is today available in MAC for buffer status reporting.

The selection of unused TOs must be specified and controllable by the gNB.

The MAC selection algorithm for UTO-UCI is described in a new section, for example 5.8.2x or 5.4.x.

MAC layer indicates, to lower layers, the future transmission opportunities that shall be unused and that shall be indicated as unused in the UTO-UCI.

The network shall be able to adapt unused TO selection depending on the network load, the QoS requirements of the UEs, the traffic behaviour for a UE, or depending on the mix of UEs in the system. Otherwise, the feature may not show any gain on the system level or there may only be gain in certain scenarios. There is also a risk that UEs becomes greedy and indicates all future TOs as “used” to achieve better QoS which would be detrimental to network capacity.

- The best selection algorithm for unused TOs may be affected by the system load, the QOS requirements, traffic behaviour, mix of UEs or on the priorities of different UEs, that is, on information that is not available to an individual UE.

Therefore, the selection cannot be left to UE implementation, the network must be in control of the UEs selection of “unused” TOs.

The network controls the UE algorithm for unused TO selection that are indicated in the UTO-UCI.

### 2.3.1	Data volume available for transmission

The selection algorithm will at least need to be aware of how much data that is currently available for transmission.

Therefore, we need to update the purpose of data volume reporting in RLC and PDCP from “for MAC buffer status reporting” to include “for the purpose of decision on future unused transmission opportunities” or change to “for the purpose of informing MAC entity of data volume available in PDCP/RLC. We propose:

The purpose of data volume calculation functionality in RLC is updated to say “for the purpose of informing the MAC entity of data volume available in RLC”.

The purpose of data volume calculation functionality in PDCP is updated to say “for the purpose of informing the MAC entity of data volume available in PDCP”.

### 2.3.2	Selection algorithm for UTO-UCI indication

RAN1 have agreed that a TO that has been indicated as “unused” TO in an UTO-UCI at a later occasion cannot be used for transmission. While for TOs that has been indicated as “NOT unused” in a UTO-UCI can later be indicated as “unused” TO (which is similar to skip uplink functionality, while the UTO-UCI is sent earlier than the actual TO).

MAC entity shall consider all CG TOs indicated as “unused” as unavailable for transmissions. That is, no overriding for TOs that have been indicated as “unused”.

MAC entity is allowed to indicate a TO as “unused” even if it has previous been indicated as “used”.

There are many possible algorithms, one useful may be:

- The simplest useful algorithm may be that the UE indicate as “used” the TOs needed to empty the current buffer, and future TOs are indicated as “unused”. This can be further improved by complementing with a timer:

  - Alt-1: A timer is started/re-started for each arrival of data that can be mapped on the CG TOs. While the timer is running the UE shall indicate a TO as “unused” if the earlier “used” TOs are enough to empty the buffer, otherwise the UE may indicate the TO as “used”. When the timer expires, a TO not previously indicated as “unused” the UE itself may choose between “used” or “unused”.

  - Alt-2: A timer is started/re-started for each arrival of data that can be mapped on the CG TOs. While the timer is running the UE itself may choose between “used” or “unused”. When the timer expires,

    - If the TO was previously referenced by a UTO-UCI, the UE shall indicate the TO as “unused” if the earlier “used” TOs are enough to empty the buffer, otherwise the UE may indicate the TO as “used”.

    - If the TO was not previously referenced by a UTO-UCI the UE itself may choose between “used” or “unused”.

For algorithm A-Alt1, the timer and the number of TOs referenced by UTO-UCI has the purpose to capture expected arrival of next data burst. The time the timer runs plus the time duration defined by UTO-UCI should preferable be less than or equal to the time between data arrivals.

For algorithm A-Alt2, the timer serves the purpose to be able to wait for potential new arrival of data. Again, the time the timer runs plus the time duration defined by UTO-UCI should preferable be less than or equal to the time between data arrivals.

The algorithm A-Alt1 is good if data arrives semi-periodically in bursts and data in a burst arrive at the same time, while algorithm A-Alt2 is good if data arrives semi-periodically in bursts but some data in a burst may be delayed compared to first data in the burst. Clearly, algorithm A-Alt2 is slightly more complex than algorithm A-Alt1 since UE need to remember which TOs that have previously been referenced by a UTO-UCI. However, since UE is not allowed to change a TO previously indicated as “unused” to “used”, we believe it is a minor additional complexity for algorithm A-Alt2. Figure 1 illustrates how the two alternatives (Alt1 and Alt2) operates.

Figure 1: Illustration of algorithm A-Alt1 (left figure) and A-Alt2 (right figure).

We think we need the extra control from adding a timer, and Alt-1 is a little simpler.

The algorithm for selecting “used” or “unused” TOs in controlled by a timer CG_DATA_TIMER that is started/restarted for each time new data, that can be mapped to the CG TOs, becomes available for transmission in the UE buffer. While CG_DATA_TIMER is running the UE shall indicate a TO as “unused” if the earlier “used” TOs are enough to empty the buffer, otherwise the UE may indicate the TO as “used”. When the timer expires, a TO not previously indicated as “unused” the UE itself may choose between “used” or “unused”.

### 2.3.3	Interaction between uplink skipping and UTO-UCI

In the running CR [3], a CG grant is only indicated to the HARQ entity for TOs that have not previously indicated as “unused”:

For each Serving Cell and each configured uplink grant, if configured and activated and has not been indicated by the MAC entity to the lower layers as to be unused for PUSCH transmission, the MAC entity shall:

However, for TOs previously indicated as “used” or not previously referenced by UTO-UCI the grant is indicated to the HARQ entity. The UTO-UCI is a new UCI that should be excluded in MAC procedure when UE is configured with enhancedSkipUplinkTxConfigured. That is, if UE is configured with UTO-UCI and is configured with enhancedSkipUplinkTxConfigured, the UE skips the grant if UE would skip the grant according to legacy procedure if UTO-UCI would not be present.

For enhancedSkipUplinkTxConfigured, the UE skips the grant according to legacy proceedure considering UCI with UTO-UCI excluded.

Furthermore, we think that a TO should be indicated as “unused” if the TO would be skipped according to legacy procedure if UTO-UCI would not be present in UCI when UE is configured with enhancedSkipUplinkTxConfigured. That is, if UE is configured with enhancedSkipUplinkTxConfigured and the PUSCH on the TO would include UCI other than UTO-UCI then the UE indicate the TO as “used”.

For enhancedSkipUplinkTxConfigured, the UE indicate a TO as

- “unused” if the TO would be skipped when considering UCI with UTO-UCI excluded and

- “used” if the UCI comprises UCI other than UTO-UCI.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If the procedural text “2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;” is reached for invalid grants, there is ambiguity in UE behaviour, some UEs may drop the grant before obtaining an MAC PDU and some UEs may not.

Observation 2	The best selection algorithm for unused TOs may be affected by the system load, the QOS requirements, traffic behaviour, mix of UEs or on the priorities of different UEs, that is, on information that is not available to an individual UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In ConfiguredGrantConfig, introduce a new field cg-Slots-r18 with range INTEGER(2..32), need code R, and field description “Indicates the number of consecutive slots for CG PUSCH transmission in one periodicity.”

Proposal 2	Replace the numberOfPUSCH-PerPeriod (and the inconsistently used numberOfPUSCH_PerPeriod) in the MAC running CR with cg-Slots.

Proposal 3	In ConfiguredGrantConfig, introduce a new field cg-UTO-UCI-r18 with range INTEGER(3..8), need code R, and field description “Enables UTO-UCI and sets the number of bits in the UTO-UCI bitmap.”

Proposal 4	In ConfiguredGrantConfig, introduce a new field betaOffset-UTO-UCI-r18 with range INTEGER(0..31), need code R, and field description “Beta offset for UTO-UCI. The ASN-1 shall be written such that the field is always configured if cg-UTO-UCI is configured.

Proposal 5	The reference to transmission opportunities that PHY determines to be invalid shall be added in running MAC spec in the sentence “For each Serving Cell and each configured uplink grant, if configured and activated and has not been indicated by the MAC entity to the lower layers as to be unused for PUSCH transmission, the MAC entity shall:”.

Proposal 6	The MAC selection algorithm for UTO-UCI is described in a new section, for example 5.8.2x or 5.4.x.

Proposal 7	MAC layer indicates, to lower layers, the future transmission opportunities that shall be unused and that shall be indicated as unused in the UTO-UCI.

Proposal 8	The network controls the UE algorithm for unused TO selection that are indicated in the UTO-UCI.

Proposal 9	The purpose of data volume calculation functionality in RLC is updated to say “for the purpose of informing the MAC entity of data volume available in RLC”.

Proposal 10	The purpose of data volume calculation functionality in PDCP is updated to say “for the purpose of informing the MAC entity of data volume available in PDCP”.

Proposal 11	MAC entity shall consider all CG TOs indicated as “unused” as unavailable for transmissions. That is, no overriding for TOs that have been indicated as “unused”.

Proposal 12	MAC entity is allowed to indicate a TO as “unused” even if it has previous been indicated as “used”.

Proposal 13	The algorithm for selecting “used” or “unused” TOs in controlled by a timer CG_DATA_TIMER that is started/restarted for each time new data, that can be mapped to the CG TOs, becomes available for transmission in the UE buffer. While CG_DATA_TIMER is running the UE shall indicate a TO as “unused” if the earlier “used” TOs are enough to empty the buffer, otherwise the UE may indicate the TO as “used”. When the timer expires, a TO not previously indicated as “unused” the UE itself may choose between “used” or “unused”.

Proposal 14	For enhancedSkipUplinkTxConfigured, the UE skips the grant according to legacy proceedure considering UCI with UTO-UCI excluded.

Proposal 15	For enhancedSkipUplinkTxConfigured, the UE indicate a TO as

-	“unused” if the TO would be skipped when considering UCI with UTO-UCI excluded and

-	“used” if the UCI comprises UCI other than UTO-UCI.

# 4 References

- R1-2308674, “LS on Rel-18 higher-layers parameter list”, LS to RAN2, RAN3 from RAN1, RAN1#114, Toulouse, France, Aug. 21-25, 2023.

- R2-2309007, “Reply LS on XR capacity enhancements”, LS to RAN1 from RAN2, RAN2#123, Toulouse, France, Aug. 21-25, 2023.

- R2-2309316, “Introduction of XR enhancements to TS 38.321”, Qualcomm, RAN2#123, Toulouse, France, Aug. 21-25, 2023.