3GPP TSG-RAN WG2 Meeting #123bis	R2-2311537

Xiamen, China, 9 - 13 Oct, 2023

Title: 	Summary of the BAP discussion of [AT123bis][506][mIAB]

Source: 	Huawei, HiSilicon

Agenda item:	7.12.1

Document Type:	Discussion and Decision

- Introduction

[AT123bis][506][mIAB] BAP (HW)

Scope: progress based on proposals to this meeting and comments

Intended outcome: Endorsable running CR

Deadline: CB acc to Meeting schedule

- Discussion

Companies provide comments to the non-F1 traffic via reflector or ftp.

Rapporteur:

Whether non-F1 traffic belongs to a certain DU, i.e. whether IAB-node implementation can determine the logical DU for each non-F1 data. -If yes, there is no different handling between non-F1 and F1 traffic in BAP procedure. -if no, we may need have different CR implementation for non-F1 traffic in BAP procedure.

LGE

For non-F1 traffic, of course, non-F1 traffic is not from a specific DU, but anyway non-F1 traffic should use one of mapping tables and default configurations for transmission and if there are two logical DUs, we think that it should be up to IAB-node implementation which mapping table/default configuration between two logical DUs is used/associated. So, it is preferred to have a simple note to clarify this rather than having different CR implementation with normative text.

Samsung

As far as we can see, as long as the two logical DUs are configured with different IP addresses, mIAB node can differentiate non-F1 traffic based on the IP address. This is the same as the F1 traffic. (Is the concern here that non-F1 traffic is actually not at all associated with a specific DU but rather a different entity/application (as per Qualcomm’s comments)? In any case, our current understanding is as per above.)

ZTE

I share a similar view with Samsung that the non-F1 traffic is associated with DU since non-F1 traffic use the IP address associated with specific logical DU. The IP address can be used to identify which logical DU it belongs to, which is similar to the handling of F1-traffic.

Intel

Thanks for handling this email discussion. We share the same understanding that there is no different handling between non-F1 and F1 traffic in BAP procedure.

QC

Just to clarify: Non-F1 traffic does not belong to any DU. The DU is assumed to only support F1. Non-F1 traffic refers to, e.g., OAM traffic which applies to the entire IAB-node and is not DU-specific. Please consider this in the BAP CR.

- Conclusion

In this paper, rapporteur propose following proposals for the MAC open issues:

Proposal: RAN2 understands that mobile IAB-node implementation can decide/consider the logical DU for non-F1 data as needed (even though the non-F1 traffic is not technically specified as per logical DU)

- Reference