3GPP TSG-RAN WG2 Meeting #124                                R2-2313250

Chicago, USA, 13th November - 17th November, 2023

Agenda item:		7.2.5

Source:				Samsung

Title:					Remaining issues on BW aggregation

Document for:		Discussion & Decision

1	Introduction

This contribution provides our views on some remaining open issues on BW aggregation.

2	Discussion

2.1 On-demand DL-PRS aggregation

For On-demand DL-PRS aggregation, the following agreement is made in RAN1.

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

According to RAN1 agreement, we should support the LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation. Since RAN1 leaves FFS on details, RAN2 should discuss how to support that feature.

In general, there are two cases of the on-demand PRS request feature (i.e., request for pre-defined configuration and request for explicit parameter). So we would like to discuss how to support each case separately.

- Request for pre-defined configuration

For the case of request for pre-defined configuration, we see the need of enhancement on both NR-On-Demand-DL-PRS-Configuration and NR-On-Demand-DL-PRS-Request in LPP. First, the LMF should be able to indicate which On-Demand-DL-PRS-Configurations can be aggregated for BW aggregation. For that, in NR-On-Demand-DL-PRS-Configuration-r17 IE, we can introduce a new field to indicate one or more sets of aggregated On-Demand-DL-PRS-Configurations. Next the UE can request one or more sets of aggregated On-Demand-DL-PRS-Configurations among the pre-defined sets. Thus, in NR-On-Demand-DL-PRS-Request, we can introduce a new field indicate the list of the index associated with the pre-configured sets in order of preference. Based on the discussion above, we would like to propose the following.

Proposal 1: In NR-On-Demand-DL-PRS-Configuration IE, introduce a new field to indicate one or more sets of aggregated NR-On-Demand-DL-PRS-Configurations.

Proposal 2: In NR-On-Demand-DL-PRS-Request IE, introduce a new field to indicate the list of the index associated with the pre-configured sets in order of preference.

- Request for explicit parameter

For the case of request for explicit parameter, we see the need of enhancement only on NR-On-Demand-DL-PRS-Request in LPP. In the case without pre-defined configuration, it doesn’t make sense for the UE to request specific sets of DL-PRS configuration or sets of PFLs to use for BW aggregation because the UE is not aware of gNB/TRP’s configuration/capability. On the other hand, it seems reasonable to allow the UE to request a number of PFLs to use for BW aggregation or bandwidth of aggregated DL-PRS considering its positioning QoS and radio capability. For that, in NR-On-Demand-DL-PRS-Request-r17 IE, we can introduce a new field to indicate the requested number of PLFs or the requested bandwidth of aggregated DL-PRS.

Proposal 3: In NR-On-Demand-DL-PRS-Request IE, introduce a new field to indicate the requested number of PLFs or the requested bandwidth of aggregated DL-PRS.

2.2 UL-SRS aggregation

Linked SRS-PosResourceSet configuration in RRC

Regarding positioning SRS configuration, the following agreements are made in RAN1.

Agreement For SRS bandwidth aggregation across two or three carriers, support - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied.

Also, we can also find the following in the RAN1 higher layer parameter list [1].

| linked-SRSPosResourceSetIdList-SrsAggregation | New | Indication of the SRS for positioning resource sets in the indicated two or three carriers that are linked for SRS for positioning BW aggregation from the gNB to the UE | Indicatation of SRS for positioning resource sets in each of the Indicated 2 or 3 carriers. | In SRS-Config Exact parent IE up to RAN2. |
| --- | --- | --- | --- | --- |

Regarding configuration of the linked SRS-PosResourceSet, we find some FFSs that we need to check with RAN1.

First, with the current RRC running CR, only one set (or combination) of linked SRS-PosResourceSets can be configured per UE. However, when we check the higher layer parameter list and the UE feature list from RAN1, it is unclear whether the configuration of multiple sets of linked SRS-PosResourceSets is supported or not. Also, there have been no RAN1 discussion on this. Thus, we need to check this with RAN1 as in DL-PRS aggregation case.

Second, in the current RRC running CR, the srs-PosResourceSetLinkedForAggBWList-r18 is included in RRCReconfiguration-v1700-IEs. Here, the issue is whether SRS-PosResourceSets can be linked across different cell groups (i.e., MCG or SCG) or not. If it can be supported, we should keep the position of srs-PosResourceSetLinkedForAggBWList-r18 in RRCReconfiguration IE as it is. However, in that case, we need to add a new field (e.g., cellGroupInfo-r18) to indicate the applicable cell group in SRS-PosResourceSetLinkedForAggBW-r18 IE because the servingCellIndex is unique per cell group (not per UE). On the other hand, if the linking across different cell group is not supported, we can move the srs-PosResourceSetLinkedForAggBWList-r18 from RRCReconfiguration IE to CellGroupConfig IE. Thus, we need to check this with RAN1.

Proposal 4: For configuration of linked SRS-PosResourceSets, RAN2 send LS to RAN1 on the following FFS.

- whether more than one sets of aggregated SRS-PosResourceSets can be configured or not.

- whether SRS-PosResourceSets can be linked across different cell groups (i.e., MCG and SCG).

MAC CE to activate/deactivate linked SRS-PosResourceSet

Regarding the MAC CE to activate/deactivate linked SRS-PosResourceSet, the following agreements are made in RAN1.

For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

According to the RAN1 agreement above, there are two requirements on the MAC CE for activation/deactivation of the aggregated SRS-PosResourceSets as below.

- The MAC CE should be able to indicate activation/deactivation of one or two or three of the aggregated/linked SRS-PosResourceSets. For example, when three SRS-PosResourceSets (e.g., Set X/Y/Z) are linked via RRC signalling, the (de)activation of various combination (e.g., {SetX}, {SetY}, {SetZ}, {SetX&Y}, {SetY&Z}, {SetX&Y&Z}} should be supported by the MAC CE.

- The MAC CE should be able to indicate the single spatial relation for each combination of aggregated SRS-PosResources. When there are multiple combinations of aggregated SRS-PosResources within the aggregated SRS-PosResourceSets, the spatial relation information for each combination of aggregated SRS-PosResources should be indicated by the MAC CE.

Please, find the figure 1 below for the example.

Figure 1. Example of the combination of linked SRS-PosResources and the spatial relation for each combination.

From our understanding, the aforementioned requirements can not be supported by the legacy MAC CE even in the case of using the reserved 1 bit. Thus, RAN2 needs to design a new MAC CE.

Proposal 5: Introduce a new MAC CE to activate/deactivate the linked SRS-PosResourceSets.

For the detailed design of the new MAC CE, it would be related to the configuration of linked SRS-PosResourceSet in RRC. Since we still have some FFSs in the RRC configuration, we can postpone design of the new MAC CE until the related FFS (e.g., whether more than one sets of aggregated SRS-PosResourceSets can be configured or not) is confirmed by RAN1.

Proposal 6: Postpone the design of a new MAC CE to activate/deactivate the linked SRS-PosResourceSets and wait for RAN1’s conclusion on the related FFS (e.g., whether more than one sets of aggregated SRS-PosResourceSets can be configured or not).

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

On-demand DL-PRS aggregation:

Proposal 1: The TAT (i.e., srs-ValidityAreaTimeAlignmentTimer) is restarted when the UE autonomously adjust the TA.

Proposal 2: When the UE performs cell reselection within the validity area, if autonomousTA-AdjustmentEnabled is configured, RRC can indicate MAC to trigger autonomous TA adjustment.

Proposal 3: When the upper layer indicates the MAC to trigger autonomous TA adjustment, MAC can perform the following procedure if there is ongoing SRS transmission.
1) instruct PHY to adjust TA, 2) restart TAT timer (i.e., srs-ValidityAreaTimeAlignmentTimer), and 3) update the stored RSRP.

Linked SRS-PosResourceSet configuration in RRC:

Proposal 4: For configuration of linked SRS-PosResourceSets, RAN2 send LS to RAN1 on the following FFS.

- whether more than one sets of aggregated SRS-PosResourceSets can be configured or not.

- whether SRS-PosResourceSets can be linked across different cell groups (i.e., MCG and SCG).

MAC CE to activate/deactivate linked SRS-PosResourceSet:

Proposal 5: Introduce a new MAC CE to activate/deactivate the linked SRS-PosResourceSets.

Proposal 6: Postpone the design of a new MAC CE to activate/deactivate the linked SRS-PosResourceSets and wait for RAN1’s conclusion on the related FFS (e.g., whether more than one sets of aggregated SRS-PosResourceSets can be configured or not).

4	Reference

- R1- 2310692, ‘Consolidated Rel-18 higher layers parameter list' , Ericsson