3GPP TSG-RAN WG2 Meeting #126                               R2-2405777

Fukuoka, Japan, 20th May – 24th May, 2024

Agenda item:		7.14.2

Source:				Samsung

Title:					Report of [AT126][604][QoE] Remaining QoE issues (Samsung)

Document for:		Discussion & Decision

- Introduction

During the Wed. QoE session, RAN2 discussed R2-2404604 [1].

R2-2404604 Discussion on remaining QoE issues Samsung Shenzhen discussion Rel-18 NR_QoE_enh-Core Proposal 1. UE suspends sending MBS QoE reports to gNB, when QoE reporting is paused. Adopt the following proposed text. Proposal 2. UE sets each entry of measReportAppLayerContainerList in the order of time. Adopt the following proposed text. Proposal 3. UE uses measReportAppLayerContainerList-r18 only to include QoE reports for Rel-18 MBS QoE measurement. Adopt the following proposed text. DISCUSSION on P1: - Ericsson does not think the scenario can happen. P1 is postponed to clarify whether the described scenario can happen. DISCUSSION on P2: - Ericsson indicates the time stamp is in the container (as opposed to RVQoE). - Huawei agrees with Ericsson, the time stamp in container is enough. QCM agrees. - Samsung indicates the timestamp is needed for MDT alignment. P2 is not agreed unless critical issues are found DISCUSSION on P3: - Ericsson thinks it is not a problem as R17 gNB will just not read this IE. Samsung indicates that then some reports will be lost. - Huawei thinks there should be no problem. Samsung clarifies that Offline Samsung to discuss P1, P2 and P3. If companies think the issues exist, they solutions can be captured during the RRC CR review. [AT126][604][QoE] Remaining QoE issues (Samsung) Scope: Discuss P1, P2, P3 from R2-2404604 Intended outcome: Report with agreeable proposals to be approved offline (report in R2-2405777) Deadline: Friday 2024-05-24 0900

This paper is to discuss the following offline discussion and report the outcome.

[AT126][604][QoE] Remaining QoE issues (Samsung)

Scope: Discuss P1, P2, P3 from R2-2404604

Intended outcome: Report with agreeable proposals to be approved offline (report in R2-2405777)

Deadline:  Friday 2024-05-24 0900

2	Discussion

2.1 P2 and P3

P2 and P3 in R2-2404604 [1] are captured below.

Proposal 2. UE sets each entry of measReportAppLayerContainerList in the order of time. Adopt the following proposed text. Proposal 3. UE uses measReportAppLayerContainerList-r18 only to include QoE reports for Rel-18 MBS QoE measurement. Adopt the following proposed text.

Companies think P2 is not needed since there is time information in each report container. They also understand, without P3, UE can choose whether to use measReportAppLayerContainerList-r18 based on the indication (i.e., idleInactiveReportAllowed) from gNB. Therefore, we propose P2 and P3 are not agreed.

Proposal 1. P2 and P3 in R2-2404604 are not agreed.

2.2. P1

P1 in R2-2404604 [1] is captured below.

Proposal 1. UE suspends sending MBS QoE reports to gNB, when QoE reporting is paused. Adopt the following proposed text. #### 5.7.16.2 Initiation A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement and reporting, i.e. when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network. Upon initiating the procedure, the UE shall: (…) 1> for each measConfigAppLayerId with an application layer measurement report container which has not been transmitted since the UE received the RRCSetup message: 2> if the application layer measurement reporting has not been suspended for the measConfigAppLayerId associated with the application layer measurement report container according to clause 5.3.5.13d: 3> set the measConfigAppLayerId in a MeasReportAppLayer in the MeasurementReportAppLayer message(s) to the value of the measConfigAppLayerId received together with the application layer measurement report container; 3> for each application measurement report container stored for that measConfigAppLayerId: 4> set the measReportAppLayerContainerList in the MeasurementReportAppLayer message(s) to the stored value of the application layer measurement report container; NOTE 0: If the application layer measurement configurations and reports exceed the size of an RRC message, the UE includes the appLayerIdleInactiveConfig and corresponding appLayerSessionStatus in the first MeasurementReportAppLayer message when the UE enters RRC_CONNECTED, and transmits application layer measurement report containers in subsequent MeasurementReportAppLayer messages. 1> for each measConfigAppLayerId received from upper layers: 2> if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and 2> if the application layer measurement reporting has not been suspended for the measConfigAppLayerId associated with the application layer measurement report container according to clause 5.3.5.13d: 3> set the measReportAppLayerContainer in the MeasurementReportAppLayer message to the received value in the application layer measurement report container; (…)

Simply speaking, the current spec applies pause/resume feature only to Rel-17 QoE measurement reporting (i.e., measReportAppLayerContainer), and this proposal is to apply pause/resume also for Rel-18 MBS QoE report (i.e., measReportAppLayerContainerList).

Let’s assume the following scenario:

1) UE is configured with MBS QoE configuration.

2) UE goes to RRC_INACTIVE or RRC_IDLE, and performs MBS QoE measurement and logs MBS QoE reports (i.e., QoE report 1)

3) UE transits to RRC_CONNECTED, and continuously performs QoE measurement in RRC_CONNECTED as well, and generates QoE report (i.e., QoE report 2) in RRC_CONNECTED

4) gNB retrieves the QoE configuration.

5) In case of RAN overload, gNB pauses QoE reporting (by configuring pauseReporting as true)

6) According to current spec, UE suspends sending QoE report 2,. However UE still sends QoE report 1 to gNB.

P1 proposes UE should suspend sending QoE report 1 as well, considering gNB is in RAN overload and QoE report 1 may be way-too large size (ex. logged during 48 hours).

During the offline discussion, all companies supported P1. Therefore, we propose P1 is agreed.

Proposal 2. Adopt the proposed text in P1 in R2-2404604.

3	Conclusion

Based on the discussion above, we propose:

Proposal 1. P2 and P3 in R2-2404604 are not agreed.

Proposal 2. Adopt the proposed text in P1 in R2-2404604.

4	Reference

[1] 	R2-2404604, Discussion on remaining QoE issues, Samsung Shenzhen