3GPP TSG-RAN2#127 	R2-2406913

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:		8.7.1.1 (NR_XR_Ph3-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on SA2 and RAN2 LSs for XR

Document for:	Discussion and Decision

1.	Introduction

In RAN2#126, RAN2 received two LSs [1][2] from SA2 and RAN3 for XR.

In this contribution, we present our view to answer questions of two LSs.

2.	Discussion

RAN2 has received LS [1] from SA2, and there are two question for RAN2 as yellow highlighted.

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

According to SA2 discussion in SA2#163, the intention of these questions is to ask whether the provided information, i.e., the dynamic change of periodicity and the time to next burst, can be used to optimize DL scheduling in RAN2 perspective.

# Wednesday Q3 session # KI#5: Dynamic traffic characteristics - It is assumed that the information is provided by the application server to optimize scheduling for DL. What kind of traffic characteristics are provided by the UPF to the NG-RAN? - Burst Size (as part of the first PDU of the burst) - YES: 14 - NO: 0 - Time to next burst (5 objections: Tencent, Vivo, Oppo, CT, CMCC) - YES: 5 - NO: 6 - Periodicity (2 objections: Tencent, Vivo) - YES: 4 - NO: 8 - Should the packet detection rules be extended to support detection of data boost indication carried in N6 DL packet header (ref# sol#16. There is a dependency on Key Issue 2)? - YES: 8 - NO: 1

For the first question regarding the dynamic change of periodicity, we think that the information would not be well used for DL scheduling because the current SPS mechanism, which is used for the periodic DL data, does not support dynamic switching.

In current SPS mechanism, the periodicity of SPS can be changed by RRC configuration. But, as well known, RRC configuration cannot be changed dynamically. If multiple SPSs are configured, SPS configuration can be changed by DCI indicating SPS activation/deactivation, but DCI can indicate SPS activation/deactivation once at a time, i.e., one DCI indicates SPS deactivation and the next DCI indicates SPS activation.

In addition, DRX onDuration may be configured to align with the periodic DL data for receiving DCI for DL assignment. Based on the dynamic change of periodicity, DRX configuration may need to be changed. However, currently, DRX configuration can be changed via RRC, so DRX onDuration cannot be changed dynamically.

Therefore, in order for the dynamic periodicity change information to be used well for DL scheduling, RAN2 should consider to enhance the dynamic switching of SPS and DRX.

With the above reasoning, we think that RAN2 should answer as

- Even if the information for the dynamic change of periodicity is provided to NG-RAN, there is currently no mechanism to optimize DL scheduling.

- In order for the information for the dynamic change of periodicity to be used to optimize DL scheduling, the SPS enhancement and the DRX enhancement need to be considered in RAN2.

Proposal 1. For the first question in SA2 LS, RAN2 answers as:

- Even if the information for the dynamic change of periodicity is provided to NG-RAN, there is currently no mechanism to optimize DL scheduling.

- In order for the information for the dynamic change of periodicity to be used to optimize DL scheduling, the SPS enhancement and the DRX enhancement need to be considered in RAN2.

For the second question regarding the time to next burst (TTNB), we think that the information may be useful from the network point of view.

If TTNB is provided to NG-RAN considering jitter, NG-RAN can schedule DL resource efficiently considering all UEs in the NG-RAN. For example, if the next data burst for UE-A is expected soon but for UE-B it is not, the NG-RAN can schedule DL resource enough to UE-A. Thus, TTNB can be considered useful information for the network.

However, we think that TTNB is not useful information from the UE point of view. Considering TTNB provided to NG-RAN for DL scheduling and DL scheduling is determined by NG-RAN, we don’t see any benefit/impact on the UE.

Therefore, we think that RAN2 should answer as

- From the network point of view, the information may be useful for DL resource scheduling, if the information for TTNB is provided to NG-RAN,

- From the UE point of view, the information does not have any benefit/impact on the UE.

Proposal 2. For the second question in SA2 LS, RAN2 answers as:

- From the network point of view, the information may be useful for DL resource scheduling, if the information for TTNB is provided to NG-RAN,

- From the UE point of view, the information does not have any benefit/impact on the UE.

In addition, RAN2 has received LS [2], and RAN3 asked as follows:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

Regarding the question from RAN3, the UE behaviour related to PSI-based PDU discarding is very clearly specified in the current specification.

[38.321 S5.18.34] The MAC entity shall for each DRB configured with discardTimerForLowImportance: 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB: 2> indicate the activation of the PSI-based SDU discard for the DRB to upper-layers. 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB: 2> indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers. [38.323 S5.2.1] - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured). [38.323 S5.3] When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU.

In the MAC layer, regardless of which MAC entity receives PSI-based SDU Discard A/D MAC CE, the MAC entity indicates the activation/deactivation of the PSI-based SDU discard for the DRB to PDCP layer, i.e. even if only one MAC entity receives the MAC CE while the other MAC entity does not, PSI-based SDU discard activation/deactivation is indicated to PDCP layer.

Upon activating PSI-based SDU discard, the PDCP layer applies PSI-based SDU discard mechanism, i.e., starts the discardTimerForLowImportance and discard PDU set if discardTimerForLowImportance expires.

In other words, as long as PSI-based SDU Discard A/D MAC CE is received regardless of which MAC entity receives, PSI-based PDU set discard is applied to all bearers even in split bearer case.

In RAN2#125bis, the optimization for PSI-based discard was proposed that PSI-based discard is activated if PSI-based SDU Discard A/D MAC CE is received in both MAC entities, but RAN2 agreed that no further optimizations will be supported in Rel-18 for UL split-bearer.

In our view, the optimization considering split bearer is still not needed in Rel-19. In split bearer case, MN and SN would coordinate with each other. If one node suffers in congestion and the other node does not, PSI-based discard is not activated for a UE. The UE implemented well may send UL traffic to the node without congestion based on ul-DataSplitThreshold. If both nodes suffer in congestion, PSI-based discard is activated and PSI-based discard mechanism is applied for the UE. Thus, we think the optimization considering split bearer is still not needed in Rel-19 XR.

Therefore, we think that RAN2 should answer as

- Regardless of which node sends PSI-based SDU Discard A/D MAC CE, the UE applies PSI-based PDU set discard to all bearers even in split bearer case, i.e., there is no special handling of PSI-based discard for split bearer.

Proposal 3. For the question in RAN3 LS, RAN2 answers as:

- Regardless of which node sends PSI-based SDU Discard A/D MAC CE, the UE applies PSI-based PDU set discard to all bearers even in split bearer case, i.e., there is no special handling of PSI-based discard for split bearer.

3.	Conclusion

In this document, we discuss on SA2 LS and RAN3 LS, and made proposals as follows.

Proposal 1. For the first question in SA2 LS, RAN2 answers as:

- Even if the information for the dynamic change of periodicity is provided to NG-RAN, there is currently no mechanism to optimize DL scheduling.

- In order for the information for the dynamic change of periodicity to be used to optimize DL scheduling, the SPS enhancement and the DRX enhancement need to be considered in RAN2.

Proposal 2. For the second question in SA2 LS, RAN2 answers as:

- From the network point of view, the information may be useful for DL resource scheduling, if the information for TTNB is provided to NG-RAN,

- From the UE point of view, the information does not have any benefit/impact on the UE.

Proposal 3. For the question in RAN3 LS, RAN2 answers as:

- Regardless of which node sends PSI-based SDU Discard A/D MAC CE, the UE applies PSI-based PDU set discard to all bearers even in split bearer case, i.e., there is no special handling of PSI-based discard for split bearer.

4.	Reference

- R2-2406241	LS on FS_XRM Ph2	SA2

- R2-2406221	LS on UL PSI based PDU discarding in NR-DC	RAN3