3GPP TSG-RAN WG2 Meeting #127	R2- 2407058

Maastricht, Netherlands, 19-23 August, 2024

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we investigate connection scenarios, architecture, and connection establishment procedures for a multi-hop relay operation. In addition, we would like to discuss how to perform the paging monitoring and SIB delivery for the remote UE in multi-hop relay operation.

2.	Discussion

2.1 RRC state of the last/intermediate (or first) Relay UE

As the legacy Rel-17 U2N Relay operation, any RRC state of the last L2 U2N Relay UE can be supported for the multi-hop relay operation. The intermediate L2 U2N relay UE can have RRC_IDLE/INACTIVE/CONNECTED, however, the CONNECTED state means only indirect U2N connection. When the last L2 U2N Relay UE is in RRC_CONNECTED, the intermediate L2 U2N Relay UE can be in RRC_CONNECTED, IDLE or INACTIVE state. When the last L2 U2N Relay UE is in RRC_INACTIVE, the intermediate L2 U2N Relay UE can be in RRC_INACTIVE or IDLE. When the last L2 U2N Relay UE is in RRC_IDLE, the intermediate L2 U2N Relay UE can be only in RRC_IDLE.

Proposal 1: Any RRC state (i.e., IDLE, INACTIVE, or CONNECTED) can be supported for the last L2 U2N Relay UE.

Proposal 2: IDLE, INACTIVE or “indirect U2N” CONNECTED state can be supported for the intermediate L2 U2N Relay UE.

2.2 Connection establishment for multi-hop relay operation

2.2.1 Connection configuration

The purpose of the relay operation is coverage extension or power saving for the U2N Remote UE. The U2N Remote UE should have one multi-hop relay path as the normal Uu connected UE has one Uu connection. The intermediate U2N Relay UE also has one indirect U2N connection with the last U2N Relay UE. Inversely, one last U2N Relay UE can have multiple intermediate U2N Relay UE and one intermediate U2N Relay UE can have multiple U2N Remote UE.

Proposal 3.1: One U2N Remote UE has only one multi-hop relay path.

Proposal 3.2: One intermediate U2N Relay UE has one U2N connection with the last U2N Relay UE.

Proposal 3.3: One last U2N Relay UE has multiple connected intermediate U2N Relay UEs.

Proposal 3.4: One intermediate U2N Relay UE has multiple connected U2N Remote UEs.

2.2.2 Protocol stacks

Protocol stacks for the user plane and control plane of multi-hop L2 U2N relay architecture are shown in Figures 1 and 2. The figures are basically combined the structure of the legacy rel-17 U2N Relay and rel-18 U2U Relay. The intermediate U2N Relay UE has the SL connection for the multi-hop U2N relay connection. The intermediate U2N Relay can have a SL connection with Remote UE, the other intermediate U2N Relay UE, or the last U2N Relay UE. The intermediate U2N Relay UE has indirect U2N connection with a gNB via the last U2N Relay UE. The last U2N Relay UE has a Uu direct connection with a gNB and SL connection with an intermediate Relay UE for supporting multi-hop U2N relay.

Proposal 4: The proposed figure 1 and 2 can be considered as a protocol stack for multi-hop L2 U2N relay operation.

Figure 1 user plane protocol stack for multi-hop L2 U2N Relay

Figure 2 Control plane protocol stack for multi-hop L2 U2N Relay

2.2.3 Connection establishment procedure

The legacy Rel-17 L2 U2N initial RRC connection establishment procedure can be considered as a baseline for supporting the initial multi-hop connection establishment setup. After the discovery procedure for multi-hop relay operation, the Remote UE makes a PC5 RRC connection with the selected intermediate L2 U2N Relay UE. While the PC5 RRC connection between the L2 U2N intermediate Relay UE and the L2 U2N Remote UE, the intermediate L2 U2N Relay UE makes a PC5 RRC connection with the last L2 U2N Relay UE. The PC5 RRC connection has a PC5 RLC channel configuration. Considering the latency for the multi-hop L2 U2N connection, some schemes may be needed to reduce the latency of the multi-hop. For example, the PC5 RRC connection requirement from L2 U2N Remote UE to the L2 U2N intermediate Relay UE triggers the PC5 RRC connection between the L2 U2N intermediate Relay UE and the L2 U2N last Relay UE even the connection between L2 U2N Remote UE and intermediate L2 U2N Relay UE is accomplished.

Proposal 5: Consider Rel-17 Procedure for L2 U2N Remote UE connection establishment as a baseline for the initial setup of the multi-hop L2 U2N indirect path.

Proposal 6: For the initial setup of a multi-hop indirect path, the L2 U2N remote UE and the intermediate L2 U2N Relay UE are required to have a PC5-RRC connection while the intermediate L2 U2N Relay UE and the last L2 U2N Relay UE are required to have a PC5-RRC connection with PC5 Relay RLC channel configuration.

TS38.300 Figure 16.12.5.1-1: Procedure for L2 U2N Remote UE connection establishment

- Step 1) For the multi-hop L2 U2N initial connection setup, L2 U2N Remote UE sends the RRC setup request message (i.e., RRCSetupRequest) to the gNB via intermediate L2 U2N Relay UE and last L2 U2N Relay UE. The initial RRC setup request message is transmitted by using the specified SL_RLC channel configured at the last L2 U2N Relay UE for the connection of the L2 U2N Remote UE.

- Step 2) When the intermediate L2 U2N Relay UE receives the initial RRC setup request message from the L2 U2N Remote UE if the intermediate L2 U2N Relay UE is not in RRC_CONNECTED state, the intermediate L2 U2N Relay UE sends the first RRC message (i.e., RRCSetupRequest) to the gNB via the last L2 U2N Relay UE for RRC connection of itself. The first RRC message is transmitted via a specified PC5 Relay RLC channel.

- Step 3) When the last L2 U2N Relay UE receives the initial RRC setup request message from the L2 U2N intermediate Relay UE if the last L2 U2N Relay UE is not in RRC_CONNECTED state, the last L2 U2N Relay UE sends the first RRC message (i.e., RRCSetupRequest) to the gNB directly for the RRC connection of itself. After the last L2 U2N Relay UE enters RRC_CONNECTED, the last L2 U2N Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support multi-hop relay operation for the L2 U2N Remote UE.

- Step 4) The last L2 U2N Relay UE in RRC_CONNECTED sends the first RRC message of the intermediate L2 U2N Relay UE to the gNB. Upon reception of the first RRC message of the intermediate L2 U2N Relay UE, the gNB responds with an RRCSetup message to the intermediate L2 U2N Relay UE via the last L2 U2N Relay UE.

- Step 5) Upon receiving the RRCSetup message, the intermediate L2 U2N Relay UE enters RRC_CONNECTED and sends the SidelinkUEInformationNR message to request the dedicated configurations required to support multi-hop relay operation for the L2 U2N Remote UE.

- Step 6) The intermediate L2 U2N Relay UE in RRC_CONNECTED sends the first RRC message of the L2 U2N Remote UE to the gNB via the last L2 U2N Relay UE. Upon reception of the first RRC message of the L2 U2N Remote UE from the intermediate L2 U2N Relay UE, the gNB responds with an RRCSetup message to the L2 U2N Remote UE via the last/intermediate L2 U2N Relay UE.

The local ID is assigned to the intermediate L2 U2N Relay UE and the L2 U2N Remote UE via the RRCSetup message from the gNB. The local ID of the intermediate L2 U2N Relay UE is used to deliver the RRC configuration message for the multi-hop relay from the gNB to the intermediate L2 U2N Relay UE.

Proposal 7: The initial setup of a multi-hop indirect path is triggered by the transmission of the first RRC message (i.e., RRCSetupRequest) from the L2 U2N Remote UE.

Proposal 8: The RRC connection establishment of the intermediate L2 U2N Relay UE not in RRC_CONNECTED is triggered by the reception of the first RRC message (i.e., RRCSetupRequest) of the L2 U2N Remote UE.

Proposal 9: The RRC connection establishment of the last L2 U2N Relay UE not in RRC_CONNECTED is triggered by the reception of the first RRC message (i.e., RRCSetupRequest) of the intermediate (/first) L2 U2N Relay UE.

Proposal 10: If the first RRC message (i.e., RRCSetupRequest) is received from the L2 U2N Remote UE, the intermediate (/first) L2 U2N Relay UE in RRC_CONNECTED sends the SidelinkUEInformationNR message via the last L2 U2N Relay UE to request for the dedicated configurations required to support multi-hop relay operation, before sending the first RRC message of the L2 U2N Remote UE to gNB.

Proposal 11: If all L2 U2N Relay UEs on the multi-path indirect path are not in RRC_CONNECTED, gNB firstly responds to the first RRC message of the last L2 U2N Relay UE, secondly responds to the first RRC message of the intermediate (/first) L2 U2N Relay UE, and finally responds to the first RRC message of the L2 U2N Remote UE.

2.2.4 Paging monitoring/SIB delivery

It’s not certain the intermediate L2 U2N Relay UE has a reliable Uu link quality to monitor the paging or receive SIB information for the L2 U2N Remote UE. But the last L2 U2N Relay UE has to have reliable Uu link quality. Therefore, the last L2 U2N Relay UE should have the responsibility of paging monitoring and SIB delivery for the L2 U2N Remote UE and the intermediate L2 U2N Relay UE(s). The intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE may have to know the direction of the message when receiving a request message for paging monitoring or SIB delivery from the L2 U2N Remote UE. The intermediate L2 U2N Relay UE has to know which SL connection is toward to the last L2 U2N Relay UE.

Proposal 12: the last L2 U2N Relay UE has the responsibility of paging and SIB monitoring for the intermediate L2 U2N Relay UEs and L2 U2N Remote UEs.

Proposal 13: When the intermediate L2 U2N Relay UE receives a request message for paging or SIB delivery from the L2 U2N Remote UE, the intermediate L2 U2N Relay UE forwards it to the last L2 U2N Relay UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the last L2 U2N Relay UE.

If the intermediate L2 U2N Relay UE receives SIB request message from several L2 U2N Remote UE, the intermediate L2 U2N Relay UE can combine the received requested SIB types. The intermediate Relay UE sends the combined SIB request messages to the last L2 U2N Relay UE. The last L2 U2N Relay UE in RRC_CONNECTED requests the SIBs to the gNB. The last L2 U2N Relay UE in RRC_IDLE/INACTIVE monitors the SIBs and then delivers them to the intermediate L2 U2N Relay UE. When the intermediate L2 U2N Relay UE receives the SIBs from the last L2 U2N Relay UE, the intermediate L2 U2N Relay UE delivers them to the L2 U2N Remote UE which had requested. If the intermediate L2 U2N Relay UE already has stored the latest updated SIBs, the intermediate L2 U2N Relay UE can deliver them to the L2 U2N Remote UE without requesting SIBs to the last L2 U2N Relay UE.

Proposal 14: The intermediate L2 U2N Relay UE can combine the types of request SIBs received from several U2N Remote UE(s), and deliver them to the last U2N Relay UE.

Proposal 15: If the intermediate U2N Relay UE already has the latest updated SIBs which are the same as the L2 U2N Remote UE is requesting, the intermediate U2N Relay UE can deliver the stored SIBs to the L2 U2N Remote UE without forwarding the SIB request message to the last U2N Relay UE.

3.	Conclusion

Proposal 1: Any RRC state (i.e., IDLE, INACTIVE, or CONNECTED) can be supported for the last L2 U2N Relay UE.

Proposal 2: IDLE, INACTIVE or “indirect U2N” CONNECTED state can be supported for the intermediate L2 U2N Relay UE.

Proposal 3.1: One U2N Remote UE has only one multi-hop relay path.

Proposal 3.2: One intermediate U2N Relay UE has one U2N connection with the last U2N Relay UE.

Proposal 3.3: One last U2N Relay UE has multiple connected intermediate U2N Relay UEs.

Proposal 3.4: One intermediate U2N Relay UE has multiple connected U2N Remote UEs.

Proposal 4: The proposed figure 1 and 2 can be considered as a protocol stack for multi-hop L2 U2N relay operation.

Proposal 7: The initial setup of a multi-hop indirect path is triggered by the transmission of the first RRC message (i.e., RRCSetupRequest) from the L2 U2N Remote UE.

Proposal 8: The RRC connection establishment of the intermediate L2 U2N Relay UE not in RRC_CONNECTED is triggered by the reception of the first RRC message (i.e., RRCSetupRequest) of the L2 U2N Remote UE.

Proposal 9: The RRC connection establishment of the last L2 U2N Relay UE not in RRC_CONNECTED is triggered by the reception of the first RRC message (i.e., RRCSetupRequest) of the intermediate (/first) L2 U2N Relay UE.

Proposal 10: If the first RRC message (i.e., RRCSetupRequest) is received from the L2 U2N Remote UE, the intermediate (/first) L2 U2N Relay UE in RRC_CONNECTED sends the SidelinkUEInformationNR message via the last L2 U2N Relay UE to request for the dedicated configurations required to support multi-hop relay operation, before sending the first RRC message of the L2 U2N Remote UE to gNB.

Proposal 11: If all L2 U2N Relay UEs on the multi-path indirect path are not in RRC_CONNECTED, gNB firstly responds to the first RRC message of the last L2 U2N Relay UE, secondly responds to the first RRC message of the intermediate (/first) L2 U2N Relay UE, and finally responds to the first RRC message of the L2 U2N Remote UE.

Proposal 12: Paging monitoring and SIB forwarding for intermediate L2 U2N Relay UEs and L2 U2N Remote UEs are the responsibility of the last L2 U2N relay UE.

Proposal 13: When the intermediate L2 U2N Relay UE receives a message for paging monitoring or SIB delivery request from the L2 U2N Remote UE, the intermediate L2 U2N Relay UE forwards it to the last L2 U2N Relay UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the last L2 U2N Relay UE.

Proposal 14: The intermediate L2 U2N Relay UE can combine the types of request SIBs received from several U2N Remote UE(s), and deliver them to the last U2N Relay UE.

Proposal 15: If the intermediate U2N Relay UE already has the latest updated SIBs which are the same as the L2 U2N Remote UE is requesting, the intermediate U2N Relay UE can deliver the stored SIBs to the L2 U2N Remote UE without forwarding the SIB request message to the last U2N Relay UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.