3GPP TSG-RAN WG2 Meeting # 127 	                       R2-2406630

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.11.2

Source: 	Sony

Title:	SBFD configuration and supporting Random access

Document for:	Discussion and Decision

# Introduction

3GPP has been working for enhancing duplexing operation for TDD based on subband duplex (SBFD) operation at gNB side within a TDD carrier [1-2]. The motivations of enhancing duplexing operation at least for TDD are for improving system capacity, reducing delays, and improving uplink coverage.

A Rel-19 Work Item (WI) on Duplex Evolution is therefore agreed to specify the requirements for FD-TDD where FD-TDD is performed at the gNB, i.e., the gNB can transmit and receive data/signals to/from the UEs at the same time on the same frequency band, whilst the UE is maintained as HD-TDD. That is, full duplex TDD is achieved at the gNB by scheduling a UE in the DL and scheduling another UE in the UL within the same OFDM symbol.

The objectives are as follows [2]:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of time location of SBFD subbands in SIB is not precluded

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of frequency domain location of SBFD subbands in SIB is not precluded

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

…………………….

RAN1#116 Agreement:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier.​

RAN1#117 Agreement:

- For RRC connected mode UEs, SBFD subband time period:

  - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- For cell-specific indication of SBFD subband frequency location, adopt the following option:

  - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#116bis Agreement:

- Confirm the working assumption:​

  - Working assumption​

    - For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.​

RAN1#117 Agreement:

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

RAN1#117 Agreement:

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

In this contribution, we discuss the time-frequency configuration of SBFD subbands and random access in SBFD operation.

## SBFD Subband Configuration

RAN1 made some progress with several agreements for RRC connected mode UEs: the time and frequency locations of UL subband and DL subband(s) are explicitly configured in a cell specific manner. In other words, the time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs in advance.

For SBFD time location, either one or two patterns can be configured:

- When only one TDD-UL-DL pattern is configured as an example in Figure 1, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- When two TDD-UL-DL patterns are configured as an example in Figure 2, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Figure 1. SBFD time location with only one TDD-UL-DL pattern configured.

Figure 2. SBFD time location with two TDD-UL-DL patterns configured.

Based on RAN1 earlier agreements, one issue is that the whether the cell specific SBFD time configuration is carried in SIB or in UE dedicated RRC message. Since random access will be supported for both CBRA and CFRA for Idle and connected mode UEs, the SBFD time configuration should at least be indicated/configured in SIB1.

Proposal 1: SBFD time location configurations is configured in SIB1.

Another issue is how to model the frequency location of the UL SBFD subband. RAN1 seems to have ruled out to model as a bandwidth part (BWP) because the assumption is that the center frequencies should be aligned for configured DL and UL BWP pair. But there could be two options:

Option 1. Common frequency resource within a DL BWP: It may be possible to configure the UL SBFD frequency location as “common frequency resource” associated with DL BWP similar to the frequency resource configuration for MBS multicast in the earlier releases (Rel-17/18).

TS 38.211:

### 4.4.6	Common MBS frequency resource

A common MBS frequency resource is a contiguous set of common resource blocks. The starting position NMBS,istart,μ of the common MBS frequency resource i is defined relative to point A and the size of the common MBS frequency resource is given by NMBS,isize,μ. Resource blocks in a common MBS frequency resource are numbered in the same way as resource blocks in clause 4.4.4.4 with NBWP,istart,μ and NBWP,isize,μ replaced by NMBS,istart,μ and NMBS,isize,μ, respectively.

TS 38.331:

#### –	CFR-ConfigMulticast

The IE CFR-ConfigMulticast indicates UE specific common frequency resource configuration for multicast for one dedicated BWP.

CFR-ConfigMulticast information element

-- ASN1START

-- TAG-CFR-CONFIGMULTICAST-START

CFR-ConfigMulticast-r17::= SEQUENCE {

locationAndBandwidthMulticast-r17              INTEGER (0..37949)       OPTIONAL,    -- Need S

pdcch-ConfigMulticast-r17                      PDCCH-Config                             OPTIONAL,    -- Need M

pdsch-ConfigMulticast-r17                      PDSCH-Config                             OPTIONAL,    -- Need M

sps-ConfigMulticastToAddModList-r17            SPS-ConfigMulticastToAddModList-r17      OPTIONAL,    -- Need N

sps-ConfigMulticastToReleaseList-r17           SPS-ConfigMulticastToReleaseList-r17     OPTIONAL     -- Need N

}

SPS-ConfigMulticastToAddModList-r17 ::=	SEQUENCE (SIZE (1..8)) OF SPS-Config

SPS-ConfigMulticastToReleaseList-r17 ::= SEQUENCE (SIZE (1..8)) OF SPS-ConfigIndex-r16

-- TAG-CFR-CONFIGMULTICAST-STOP

-- ASN1STOP

CFR-ConfigMulticast field descriptions

locationAndBandwidthMulticast Frequency domain location and bandwidth for MBS multicast. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting N^size_BWP=275. The first PRB is a PRB determined by subcarrierSpacing of the associated BWP and offsetToCarrier corresponding to this subcarrier spacing. If not configured, the UE applies the value of locationAndBandwidth of the DL BWP in which the cfr-ConfigMulticast is configured.

For connected mode UEs, since a UE is switching between different BWPs (note that for TDD DL and UL are pair with same index with aligned center frequency), it may be reasonable to configure the UL SBFD “common” frequency resource associated with a DL BWP, hence it becomes a BWP specific. Examples of this is shown on Figure 3 in which the UL SBFD “common” frequency resource can be added when a UE activates DL BWP#1, and in later time if the UE switches to DL BWP#2, it releases the first the UL SBFD “common” frequency resource and adds a new UL SBFD “common” frequency resource (using add and release function).

Figure 3. Frequency location of the UL SBFD subband is modelled as UL SBFD “common” frequency resource associated with DL BWP.

Option 2. Intersection between cell-specific UL subband and active DL BWP in SBFD symbols: Another way is to fix the frequency location of the UL SBFD subband in the system carrier bandwidth as shown on Figure 4. The UL SBFD is determined by the intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. For example, a UE has an active BWP#1 where the intersection of f2 to f3 defines the UL subband#2 in the current active BWP1. However, if the UE switches to BWP2 (activated), the UE has another intersection of f1 to f2 that defines the UL subband#3 in the new active BWP#2.

Figure 4. Frequency location of the UL SBFD subband is modelled as intersection between cell-specific UL subband and active DL BWP in SBFD symbols.

Comparing these two options, when frequency location of the UL SBFD subband is modelled as intersection between cell-specific UL subband and active DL BWP in SBFD symbols, then this may limit the number of BWPs that can be configured for the UE. However, the benefit may be that Idle and connected UEs can share the same “single” UL SBFD subband which can be configured in SIB1. On the other hand, it may be simpler (less complex) to model the UL SBFD frequency location as “common frequency resource” associated with DL BWP as spec already supports similar configurations for MBS.

Proposal 2: For connected mode UEs, SBFD frequency location is configured within the associated DL BWP named as an UL SBFD “common” frequency resource (i.e., BWP specific configuration).

## RACH configuration for SBFD Subband

Work Item (WI) captures to specify SBFD operation to support random access in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage. Hence, it is reasonable to support both 4-step RACH and 2-step RACH in UL SBFD symbols.

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

In addition, RAN1 agreed that CBRA and CFRA in SBFD symbols is supported for SBFD aware UEs in RRC CONNECTED state. RAN2 can confirm RAN1 agreement as it is beneficial to support CBRA and CFRA in SBFD symbols with the same motivation as above (i.e., increasing RACH capacity, reducing delays, and improving the uplink coverage).

Proposal 3: RAN2 confirms to support both 4-step RACH and 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Proposal 4: RAN2 confirms to support CBRA and CFRA in SBFD symbols for SBFD aware UEs in RRC CONNECTED state.​

Furthermore, RAN1 made some progress and agreed two different configuration options to support for SBFD-aware UEs in RRC CONNECTED state. Option 1 is to use one single RACH configuration which is based on the existing RACH parameters. Option 2 is to support two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration. However, a UE is not required to support both options. RAN1 is still discussing the details for both options such as RO validation rules and SSB-RO mapping rules, reinterpretation msg1-FrequencyStart etc., so RAN2 should wait further progress in RAN1.

## Conclusions

In this contribution, we have discussed the time-frequency configuration of UL SBFD subbands and random access in SBFD operation. We have the following observation and proposals:

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 1: SBFD time location configurations is configured in SIB1.

Proposal 2: For connected mode UEs, SBFD frequency location is configured within the associated DL BWP named as an UL SBFD “common” frequency resource (i.e., BWP specific configuration).

Proposal 3: RAN2 confirms to support both 4-step RACH and 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Proposal 4: RAN2 confirms to support CBRA and CFRA in SBFD symbols for SBFD aware UEs in RRC CONNECTED state.​

## References

# [1] RP-232763, 3GPP TR 38.858 V2.0.0, Study on Evolution of NR Duplex Operation.

# [2] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104.