3GPP TSG-RAN WG2 Meeting #R2-127		R2-2406820 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	Initial considerations for inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective mentions that for inter-CU LTM Rel-18 intra-CU LTM is the baseline. This implies that any enhancements for the inter-CU LTM use cases are going to be on top of the Rel 18 framework.

In this contribution, we describe and discuss some aspects involved in extending LTM to support inter-CU use-cases. We also propose a way forward for some of these.

# Discussion

## Simultaneous LTM in MN and SN

The WID [1], has a NOTE “The case that LTM is configured in both MCG and SCG is excluded” which is highlighted above in sec 1.

In RAN2#125, for Rel 18 LTM, during the discussion on coexistence of LTM and other features (Dual Connectivity for eg), the following agreement was made.

No restriction of configuring MCG LTM and SCG LTM. No intention to further work in R2 on network interaction to better enable this.

In our understanding, this implies the following (in scope of R18) - Configuring LTM independently at the same time at MN and SN is permitted, but no MN-SN co-ordination will be further discussed in scope of Rel18 work.

In our view, the above two are contradictory to each other. While the Rel 18 agreement imposes no restriction, the Rel 19 scope excludes such a configuration in the first place.

Moreover, even if such a simultaneous configuration of LTM at MN and SN is excluded, it has to be well co-ordinated between MN and SN. There also needs to be a mechanism to address race conditions when both MN and SN configure LTM at the same time.

The Rel 18 agreement and Rel 19 WI scope are not consistent with each other.

RAN2 discusses and agrees a way-forward to configure/not-configure simultaneous LTM at MN and SN and aligns the way-forward in both Rel 18 and Rel 19.

If simultaneous LTM at MN and SN is excluded, RAN2 sends an LS to RAN3 to address the issue and prevent such occurrences using a coordinated solution.

## Timing Advance acquisition in inter-CU LTM

Agreements on early sync phase:

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Timing Advance acquisition is a key step in LTM. This enables the UE to perform RACH-less LTM to the identified target cell. This is true with inter-CU LTM candidate cells as well. In RAN2 #126, the above agreement was made where the below described Rel18 solution was agreed as the baseline.

- UE performs RACH to the candidate gNB-DU based on a PDCCH order from the serving gNB-DU to acquire TA.

- The candidate gNB-DU sends the UE’s TA to the serving gNB-DU via the gNB-CU.

- The serving gNB-DU delivers the TA to the UE in the DL MAC CE used for sending the LTM cell switch command.

In this paper we discuss the RAR based option which is about delivering the UE’s candidate cell TA in the Random-Access Response directly to the UE instead of delivering over the backhaul. This provides an optimization over the baseline solution as the backhaul latency (avg 6-8ms over two F1 interfaces in R18) in delivering the TA is avoided. In case of inter-CU LTM, this backhaul latency to deliver the candidate cell TA to the serving gNB-DU will have an additional increase because of the Xn interface between the two gNBs. Therefore, we think that it is beneficial to deliver the TA directly to the UE, especially in the inter-CU LTM scenario.

During LTM, it is advantageous to deliver the LTM candidate cell TA directly to the UE.

Moreover, as mentioned in the highlighted agreement, providing TA in the RAR is probably inevitable in case of conditional LTM, as the gNB-DU will not issue a LTM serving cell switch command to the UE. In a scenario where RAR based TA acquisition is supported for C-LTM, we think it is too limiting and inflexible to not allow use of RAR for TA delivery for non-conditional LTM.

Therefore, we propose that the agreement from RAN2#126 is re-considered and both options i.e RAR based option and the Rel 18 baseline option are supported, and the usage can be decided based on implementation. The serving gNB-DU may also be informed whether the TA has been delivered to the UE in the RAR message.

Delivery of LTM candidate cell TA using the RAR based solution directly to the UE is supported, in addition to the Rel 18 baseline solution for non-conditional LTM also.

Delivering the LTM candidate cell TA using the RAR can also be useful in a multi-TRP scenario where TAs for two TRPs are required to be delivered to the UE. The TRP ID or the Cell ID may be used to identify and associate the TA.

Delivery of candidate cell TA using the Random-Access Response (RAR) can be performed for a UE in multi-TRP configuration also.

## Reference configuration for LTM

Rel 18 LTM describes how a reference configuration is to be used by candidate/target gNB-DU to provide a delta configuration. They also describe how a UE can derive full candidate/target cell configuration using the reference and delta configurations.

It is possible that a UE may have to undergo LTM cell switch to different cells with different configurations i.e a gNB may host gNB-DUs with different characteristics whose cell configurations are vastly different and therefore reference configurations cannot be used to determine the full candidate cell configuration.

In such scenarios, it is imperative that the candidate/target gNB-DU provides a full candidate cell configuration. But during subsequent LTM cell switches, there may be adjacent cells having similar characteristics which effectively makes a case for a second reference configuration.

For eg: if there are two kinds of gNB-DUs, one FR1 and another FR2, there could be two reference configurations used for LTM candidate cell preparation.

There could be scenarios where the candidate/target configurations are entirely different and hence cannot be prepared as a delta of the reference configuration.

There could be different gNB-DUs having different kinds of cells which may require different reference configuration for a UE to prepare the full candidate/target cell configuration using a delta configuration.

RAN2 discusses the option of one gNB having more than one (at least two, one for FR1 and one for FR2) reference configurations for a UE and agrees a solution to address the issue.

# Conclusion

We have the following observations:

Observation 1	The Rel 18 agreement and Rel 19 WI scope are not consistent with each other.

Observation 2   During LTM, it is advantageous to deliver the LTM candidate cell TA directly to the UE.

Observation 3   Delivery of candidate cell TA using the Random-Access Response (RAR) can be performed for a UE in multi-TRP configuration also.

Observation 4	There could be scenarios where the candidate/target configurations are entirely different and hence cannot be prepared as a delta of the reference configuration.

Observation 5	There could be different gNB-DUs having different kinds of cells which may require different reference configuration for a UE to prepare the full candidate/target cell configuration using a delta configuration.

We have the following proposals:

Proposal 1	RAN2 discusses and agrees a way-forward to configure/not-configure simultaneous LTM at MN and SN and aligns the way-forward in both Rel 18 and Rel 19.

Proposal 2	If simultaneous LTM at MN and SN is excluded, RAN2 sends an LS to RAN3 to address the issue and prevent such occurrences using a coordinated solution.

Proposal 3	Delivery of LTM candidate cell TA using the RAR based solution directly to the UE is supported, in addition to the Rel 18 baseline solution for non-conditional LTM also.

Proposal 4	RAN2 discusses the option of one gNB having more than one (at least two, one for FR1 and one for FR2) reference configurations for a UE and agrees a solution to address the issue.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes