3GPP TSG-RAN WG2 Meeting #127	R2- 2407057

Maastricht, Netherlands, 19-23 August, 2024

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on Relay discovery and selection

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the condition to be a last L2/L3 U2N Relay UE, intermediate (/first) L2/L3 U2N Relay UE, or L2/L3 U2N Remote UE for multi-hop relay operation. Also, we investigate under which condition the Discovery message is forwarded to support multi-hop relay.

2.	Discussion

2.1 Uu-RSRP threshold

In the Rel-17 U2N Relay, the Uu-RSRP threshold to be a U2N Relay UE can be configured by gNB. In this case, the Uu-RSRP of the U2N Relay UE is between ThreshoLowRelay and thresholdHighRelay. The reason to define this boundary of the Uu threshold was that it could be helpful for cell extension when the U2N Relay UE is located in some location on which the Uu-RSRP is strong enough to serve Remote UE, but far away from gNB.

For the multi-hop relay, this kinds of Uu-threshold restriction is needed. For coverage extension maximization, only the last U2N Relay UE needs to have the Uu-RSRP threshold restriction as the same of the Rel-17 U2N Relay UE.

Proposal 1: The same threshold used for Rel-17/18 U2N Relay UE can be configured for last Relay UE in the multi-hop. The Uu threshold for the last U2N Relay UE can be above a ThreshLowRelay and below a thresholdHighRelay.

In the Rel-17 U2N Relay, the Remote UE also has Uu-RSRP threshold restriction. When Remote UE has a Uu-RSRP value below configured threshold via SIB, pre-configuration, or dedicated RRC, the Remote UE can try for U2N Relay connection. The Remote UE having week Uu-RSRP needs to be supported by U2N Relay UE to have connection with gNB.

For the multi-hop relay, the intermediate U2N Relay UE and U2N Remote UE can be configured the same Uu-RSRP threshold restriction which was applied to the legacy Rel-17 U2N Remote UE. U2N Remote UE reaches to the gNB via intermediate U2N Relay UE and the last U2N Relay UE. The intermediate U2N Relay UE doesn’t need to have better Uu-RSRP value than the last U2N Relay UE. Also, the intermediate U2N Relay UE doesn’t need to have strong Uu-RSRP value since it is connected via the last U2N Relay UE to the gNB.

Proposal 2: The same threshold used for Rel-17/18 U2N Remote UE can be configured for the intermediate U2N Relay UE and U2N Remote UE in the multi-hop. The Uu threshold for the intermediate U2N Relay UE and U2N Remote UE can be below a threshHighRemote.

Each intermediate U2N Relay UE and U2N Remote UE can exist in the same/different cell from the serving/camp-on cell of the last U2N Relay UE. The intermediate U2N Relay UE, and U2N Remote UE can be out-of-coverage. But after multi-hop U2N connection via the last U2N Relay UE, the intermediate U2N Relay UE and U2N Remote UE consider the serving/camp-on cell of the last U2N Relay UE as its own cell. It is the same principle as the legacy Rel-17 U2N Relay operation.

Proposal 3: The last U2N Relay UE, intermediate U2N Relay UE, and the U2N Remote UE can be in the same or different cells before U2N connection.

Proposal 4: The intermediate U2N Relay UE and U2N Remote UE can be in OoC.

When the intermediate U2N Relay UE is in a different cell from the cell of the last U2N Relay UE’s cell, the intermediate U2N Relay UE should follow the configuration of its cell before making a U2N connection via the last U2N Relay UE. If the cell of the intermediate U2N Relay UE does not support multi-hop relay operation, the intermediate U2N Relay UE can act as intermediate U2N Relay UE. If the cell of the intermediate U2N Relay UE supports multi-path relay operation but does not configure the Uu-RSRP threshold, the intermediate U2N Relay UE also can act as intermediate U2N Relay UE without checking the Uu-RSRP of itself. However, if the cell of the intermediate U2N Rely UE supports multi-hop relay operation and configures the Uu-RSRP threshold, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE only when the Uu-RSRP threshold of its cell is met.

Proposal 5: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE as long as the cell does not support multi-hop relay operation or does not configure the Uu-RSRP threshold for the intermediate U2N Relay UE in case that the different cell supports multi-hop relay operation.

Proposal 6: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE based on the Uu-RSRP threshold configuration of the different cell as long as the different cell supports multi-hop relay operation.

2.2 Discovery message transmission

In the legacy, Rel-17 U2N SL Relay operation, U2N Relay UE broadcasts the discovery message in discovery model A. As the same principle, in the multi-hop relay operation, only the last U2N Relay UE initiates the discovery message in discovery model A. Upon reception of the discovery message from the last U2N Relay UE, the intermediate U2N Relay UE forwards the discovery message if the PC5 RSRP is above a configured threshold. Considering the multi-hop extension, the upper layer of intermediate U2N Relay UE may check whether the upper layer criteria are met. The same original message should not be broadcasted with different hop counts via the same intermediate U2N Relay UE. The duplicated discovery message with different hop counts at the intermediate U2N Relay UE may be handled in the upper layer. So, the final decision on whether the discovery message is forwarded should be handled at the upper layer of the intermediate U2N Relay UE.

Proposal 7: For Discovery model A, the last U2N Relay UE only initiates the discovery message broadcast.

Proposal 8: For Discovery model A, the intermediate U2N Relay UE forwards the discovery message received from the last U2N Relay UE only when the PC5 RSRP is above the configured threshold and the upper layer criteria are met.

In discovery model A for multi-hop relay, when the last U2N Relay UE receives the discovery message from the other last U2N Relay UE, the last U2N Relay UE does not forward the discovery message. The last U2N Relay UE only initiates the discovery message transmission and does not forward the received discovery message.

If the intermediate U2N Relay UE already has a U2N indirect connection with a gNB via a last U2N Relay UE, the intermediate U2N Relay UE forwards the message only when the discovery message comes from the connected last U2N Relay UE. In this case, the intermediate U2N Rely UE checks the PC5-RSRP. Due to the intermediate U2N Relay UE already having a U2N indirect connection, the intermediate U2N Relay UE may not make another connection to the gNB via the other last U2N Relay UE. So, there is no reason for the intermediate U2N Relay UE, which has an indirect connection to the gNB via a last U2N Relay UE, to forward the discovery message received from the other last U2N Relay UE.

Proposal 9: In Discovery model A, when the last U2N Relay UE(s) receives the discovery message from the other last U2N Relay UE, the last U2N Relay UE does not forward the received discovery message.

Proposal 10: In Discovery model A, if the intermediate U2N Relay UE already has a U2N indirect connection via a last U2N Relay, the intermediate U2N Relay UE does not forward the discovery message when receiving from the other last U2N Relay UE not having a U2N connection.

In the discovery model B for multi-hop relay, the U2N Remote UE broadcasts the solicitation message and the last U2N Relay UE sends the response message to the U2N Remote UE. In the legacy, Rel-18 U2U Relay, when candidate U2U Relay UEs receive the solicitation message from a U2U Remote UE, the candidate U2U Relay UEs forward the solicitation message if the PC5-RSRP is above a configured threshold. Upon the reception of the solicitation message from the candidate U2U Relay UE, the peer U2U Remote UE sends the response message to the U2U Relay UE only when the PC5 RSRP is above a threshold. When the U2U Relay UE receives the response message from the peer U2U Remote UE, the U2U Relay UE forwards it to the U2U Remote UE without checking the AS layer criteria. The reason not to check the PC5 RSRP is that the link is already qualified via the solicitation message. The U2U Remote UE receives the response message, and it can select the proper U2U relay UE when the upper layer criteria are met.

A similar principle of solicitation and response message delivery in the Rel-18 U2U can be applied to the Rel-19 multi-hop relay operation. However, considering the multi-hop extension, the upper layer of the intermediate U2N Relay UE has to check whether the solicitation message is coming from the other intermediate U2N Relay UE. If the solicitation message comes from the other intermediate U2N Relay UE, the intermediate U2N Relay UE needs to check whether the message has passed through itself before or the maximum hop count is over. So, the intermediate U2N Relay UE checks both the PC5 RSRP and upper layer criteria for forwarding the solicitation message. For the response message, there is no reason to check the PC5 RSRP and upper layer criteria. The link quality was already qualified by using the solicitation message, and there is no possibility for the response message to be delivered duplicated via the other intermediate U2N Relay UE due to the unicast way of transmission of the response message.

Proposal 11: For Discovery model B, the forwarding operation can be applied similarly to the Rel-18 U2U relay operation, except that the upper layer of the intermediate U2N Relay UE checks the upper layer criteria.

-  The intermediate U2N Relay UE forwards the solicitation message from U2N Remote UE only if the PC5 RSRP is above a threshold and upper layer criteria are met.

-  No AS criterion is needed for the intermediate U2N Relay UE to forward the response message.

If the intermediate U2N Relay UE already has a U2N relay connection via the last U2N Relay UE, the intermediate U2N Relay UE should deliver the solicitation message from a U2N Remote UE to its connected last U2N Relay UE. An intermediate U2N Relay UE is only one served U2N indirect connection. So, the intermediate U2N Relay UE already having a U2N connection delivers the solicitation message by unicast even though the solicitation message is delivered by broadcast.

Proposal 12: In Discovery model B, if the intermediate U2N Relay UE already has a U2N indirect connection with the gNB via the last U2N Relay UE, the intermediate U2N Relay UE forwards the solicitation message received from U2N Remote UE to only the connected last U2N Relay UE.

2.3 Relay (re)selection

Relay (re)selection can be triggered at the U2N Remote UE. In multi-hop relay, the relay reselection is triggered when the U2N Remote UE detects that the PC5 RSRP with the intermediate U2N Relay UE is below the configured threshold. Or the upper layer requests the relay reselection to the AS layer. When RLF is detected, the relay selection is triggered. The RLF can happen not only in the current SL connection but also in the SL between intermediate U2N Relay UE and the last U2N Relay UE, or the Uu link between the last U2N Relay UE and gNB. Also, if the intermediate U2N Relay UE triggers relay reselection or the intermediate U2N Relay UE tries to have a direct connection, the U2N Remote UE triggers relay reselection.

Proposal 13: The following cases trigger relay (re)selection:

- When the PC5 RSRP with the current connected intermediate U2N relay UE is below the configured threshold.

- When the upper layer requests relay reselection to the AS layer

- When detecting SL RLF on the current SL or receiving Uu/SL RLF notification from the connected intermediate U2N Relay UE.

- When the intermediate U2N Relay UE triggers relay (re)selection or direct connection.

3.	Conclusion

Proposal 1: The same threshold used for Rel-17/18 U2N Relay UE can be configured for last Relay UE in the multi-hop. The Uu threshold for the last U2N Relay UE can be above a ThreshLowRelay and below a thresholdHighRelay.

Proposal 2: The same threshold used for Rel-17/18 U2N Remote UE can be configured for the intermediate U2N Relay UE and U2N Remote UE in the multi-hop. The Uu threshold for the intermediate U2N Relay UE and U2N Remote UE can be below a threshHighRemote.

Proposal 3: The last U2N Relay UE, intermediate U2N Relay UE, and the U2N Remote UE can be in the same or different cells before U2N connection.

Proposal 4: The intermediate U2N Relay UE and U2N Remote UE can be in OoC.

Proposal 5: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE as long as the cell does not support multi-hop relay operation or does not configure the Uu-RSRP threshold for the intermediate U2N Relay UE in case that the different cell supports multi-hop relay operation.

Proposal 6: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE based on the Uu-RSRP threshold configuration of the different cell as long as the different cell supports multi-hop relay operation.

Proposal 7: For Discovery model A, the last U2N Relay UE only initiates the discovery message broadcast.

Proposal 8: For Discovery model A, the intermediate U2N Relay UE forwards the discovery message received from the last U2N Relay UE only when the PC5 RSRP is above the configured threshold and the upper layer criteria are met.

Proposal 9: In Discovery model A, when the last U2N Relay UE(s) receives the discovery message from the other last U2N Relay UE, the last U2N Relay UE does not forward the received discovery message.

Proposal 10: In Discovery model A, if the intermediate U2N Relay UE already has a U2N indirect connection via a last U2N Relay, the intermediate U2N Relay UE does not forward the discovery message when receiving from the other last U2N Relay UE not having a U2N connection.

Proposal 11: For Discovery model B, the forwarding operation can be applied similarly to the Rel-18 U2U relay operation, except that the upper layer of the intermediate U2N Relay UE checks the upper layer criteria.

-  The intermediate U2N Relay UE forwards the solicitation message from U2N Remote UE only if the PC5 RSRP is above a threshold and upper layer criteria are met.

-  No AS criterion is needed for the intermediate U2N Relay UE to forward the response message.

Proposal 12: In Discovery model B, if the intermediate U2N Relay UE already has a U2N indirect connection with the gNB via the last U2N Relay UE, the intermediate U2N Relay UE forwards the solicitation message received from U2N Remote UE to only the connected last U2N Relay UE.

Proposal 13: The following cases trigger relay (re)selection:

- When the PC5 RSRP with the current connected intermediate U2N relay UE is below the configured threshold.

- When the upper layer requests relay reselection to the AS layer

- When detecting SL RLF on the current SL or receiving Uu/SL RLF notification from the connected intermediate U2N Relay UE.

- When the intermediate U2N Relay UE triggers relay (re)selection or direct connection.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.