# 3GPP TSG RAN WG2 Meeting #127                          	   R2-2407163

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The WID for “Enhancements of network energy savings for NR” was approved at RAN #102, and further updated at RAN # 103 with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions, and the potential impact to RAN2’s specification work.

# Discussion

## Adaptation of SSB in time domain

For adaptation of SSB in time domain, the following agreements were made at RAN1 #116 for further study [2].

Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study. - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration

Additionally, the following agreements were made at RAN1 #116bis [3].

Agreement For indication of adaptation of SSB in time-domain, Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

NR supports cell-defining SSB (CD-SSB) and non-cell-defining SSB (NCD-SSB). The CD-SSB has associated SIB1 and transmitted in a cell supporting initial access and Idle/Inactive mode operations for UEs at RRC Idle or Inactive state. The NCD-SSB does not have associated SIB1 and can be transmitted in a cell that may not support initial access and Idle/Inactive mode operations for UEs at RRC Idle or Inactive state, e.g., the SCells in CA deployments. Furthermore, the NCD-SSB is only needed for UE connected mode operation, e.g., beam management, mobility management, power control etc. or for a cell or carrier without CD-SSB.

SSB adaptation design is mainly under RAN1’s scope, RAN2 can wait for RAN1’s further conclusion(s) (e.g., on SSB adaptation options with cell-defining SSB and non-cell-defining SSB) before to discuss SSB adaptation impact to UEs’ operations at RRC Idel/Inactive state.

Observation 1: Adaptation of SSB in time for cell-defining SSB may have negative impact to UEs’ operations at RRC Idle or Inactive state.

Proposal 1: For SSB adaptation impact to UEs’ operations at RRC Idel/Inactive state, RAN2 can wait for RAN1's further conclusion on adaptation options with cell-defining SSB and non-cell-defining SSB.

For SSB adaptation in time, the following agreement was made at RAN1 #117 [4].

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

The SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the on-demand SSB transmission cases. For example, the UE may be configured with at one or more on-demand SSB burst periodicities which can be activated with an indication of SSB adaptation transmission, e.g., the indication indicates not only the availability of SSB transmission but also the adaptation of the SSB burst periodicity.

It was agreed at RAN1#117 to support MAC CE based signaling to indicate on-demand SSB transmission for on-demand SSB operation in SCells. Given the commonality between on-demand SSB transmission and SSB transmission with burst periodicity adaptation, a unified MAC CE may be used to indicate both on-demand SSB transmission and SSB transmission with burst periodicity adaptation.

Observation 2: The indication for SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the indication for on-demand SSB transmission cases.

Proposal 2: RAN2 may discuss the following options for the indication of SSB burst periodicity adaptation.

- Option 1. A new MAC CE

- Option 2. A common MAC CE for indicating both on-demand SSB transmission and SSB burst periodicity adaptation.

## Adaptation of PRACH in time domain

For adaptation of PRACH in time domain, the following agreement wwas made at RAN1 #116 for further study [2].

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study. - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH. - Concentrating ROs in time domain - Other options are not precluded

Additionally, the following agreements were made at RAN1 #116bis [3].

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Furthermore, following agreements were made at RAN1 #117 for the configuration of additional PRACH resources:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Proposal 3: For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

## Adaptation of paging occasions

For adaptation of paging occasions, the following agreement was made at RAN1 #116 for further study [2].

Agreement For adaptation of paging, - Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings. - Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

At RAN2 #125bis, the following agreements were made for paging adaptation [4].

Agreements on paging adaptation: 1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

As referenced from TS 38.304 [5], with multi-beam operation, a Paging Frame (PF) is the starting point of a PO which may be a set of PDCCH monitoring occasions and can consist of multiple time slots where paging DCI can be sent. A UE may assume that the same paging message is repeated in all transmitted beams for its PO. The PF and PO for paging are determined by the following formulae:

As referenced from TS 38.304 [5]:

An example of bundled PFs with 8-beam PO associated with 8 SSBs (SCS = 15kHz) is illustrated below in Figure 1. For Ns = 1, there is only one PO at the first PDCCH monitoring occasion for paging in the PF. With 8 SSBs, one PO has 8 PDCCH monitoring occasions respectively on 8 beams QCLed with the 8 SSBs. With bundled PF (i.e., adjacent to each other), the 8 PDCCH monitoring occasions of PO0 may overlap with the 8 PDCCH monitoring occasions of PO1 as shown with the green circle in Figure 1.

Figure 1: Adaptation with bundled Paging Frames (SearchSpaceId = 0)

Observation 3: For SearchSpaceId = 0, paging occasions are limited with bundled paging frames.

As referenced from TS 38.304 [5]:

An example of bundled PFs with 8-beam PO associated with 8 SSBs (S = 8) and nrofPDCCH-MonitoringOccasionPerSSB-InPO configured as 1 (X= 1) is illustrated below in Figure 1. For Ns = 2, there are two POs for paging in one PF, e.g., the starting PDCCH monitoring occasion number of (i_s + 1)th PO is equal to i_s * S * X = i_s * 8 * 1 (where i_s = 0 or 1). With bundled PF (i.e., adjacent to each other), the 8 PDCCH monitoring occasions of PO0 and the 8 PDCCH monitoring occasions of PO1 for PF0 and 8 PDCCH monitoring occasions of PO0 for PF1 are illustrated below Figure 2.

Figure 2: Adaptation with bundled Paging Frames (SearchSpaceId ≠ 0)

Observation 4: For SearchSpaceId ≠ 0, paging occasions may also be limited with bundled paging frames.

Proposal 4: RAN2 discusses the PO allocation issue with bundled paging frames, especially with contiguous paging frames.

As shown in Figure 3, with distributed Paging Frames (e.g., removing the PF1), more PDCCH monitoring occasions are available for increased number of POs. Additionally, it requires less specification work.

Figure 3: Adaptation with further distributed Paging Frames (with extra POs)

Observation 5:  Further reduced paging frames with extra POs is more flexible for PO allocation.

Proposal 5: Support distributed paging frames with extra POs in a PF. The configuration details may be discussed at stage 3.

# Conclusion

In this contribution, we discussed scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions, and concluded with the following observations and proposals.

Adaptation of SSB in time domain

Observation 1: Adaptation of SSB in time for cell-defining SSB may have negative impact to UEs’ operations at RRC Idle or Inactive state.

Proposal 1: For SSB adaptation impact to UEs’ operations at RRC Idel/Inactive state, RAN2 can wait for RAN1's further conclusion on adaptation options with cell-defining SSB and non-cell-defining SSB.

Observation 2: The indication for SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the indication for on-demand SSB transmission cases.

Proposal 2: RAN2 may discuss the following options for the indication of SSB burst periodicity adaptation.

- Option 1. A new MAC CE

- Option 2. A common MAC CE for indicating both on-demand SSB transmission and SSB burst periodicity adaptation.

Adaptation of PRACH in time domain

Proposal 3: For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

Adaptation of paging occasions

Observation 3: For SearchSpaceId = 0, paging occasions are limited with bundled paging frames.

Observation 4: For SearchSpaceId ≠ 0, paging occasions may also be limited with bundled paging frames.

Proposal 4: RAN2 discusses the PO allocation issue with bundled paging frames, especially with contiguous paging frames.

Observation 5:  Further reduced paging frames with extra POs is more flexible for PO allocation.

Proposal 5: Support distributed paging frames with extra POs in a PF. The configuration details may be discussed at stage 3.

# References

[1] RP-240170 “Revised WID for Enhancements of network energy savings for NR”, RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	 Athens, Greece, February 26th – March 1st, 2024

[3] “Chair notes RAN1#116bis eom0” RAN1 #116bis	 Changsha, China, April 15th – 19th, 2024

[4] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024

[5] 3GPP TS 38.304 V18.2.0 (2024-06) “User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 18)”.