3GPP TSG-RAN WG2 Meeting #127	R2- 2407240

Maastricht, Netherlands, Aug 19th– 23rd, 2024

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes

Document for:	Discussion

# 1. Introduction

In RAN2#126 [1], following workding assumption and agreement were made:

Agreement Entry/exit condition RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Sub-grouping RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. SI reception From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we discuss LP-WUS operation in RRC_IDLE/INACTIVE mode.

# 2. Discussion

Dedicated configuration for LP-WUS

For LP-WUS monitoring, an LP-WUR UE needs to receive LP-WUS configuration (e.g., monitoring occasion, LP-SS configuration). RAN2 agreed to receive LP-WUS configuration via system information, however, a necessity of dedicated configuration is still FFS. From our view, dedicated LP-WUS configuration from a gNB is not valid since an idle/inactive LP-WUR UE is allowed to camp on another cell via cell (re-)selection procedure [3].

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Do not support dedicated LP-WUS configuration

Subgrouping for LP-WUS

In RAN1#116bis, the following agreements were made on subgrouping and PEI [2].

Agreement Regarding the LP-WUS information for idle/inactive UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] subgroups - Option 2: A codepoint value corresponding to one or more subgroup(s) - Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s) - Combination of above options are not precluded - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - Other options are not precluded

Following the legacy PEI, subgrouping is an efficient power saving operation to reduce false alarms and monitor paging occasions in IDLE/INACTIVE modes. Similar to the legacy subgrouping method, subgroup based wake-up indication and corresponding LP-WUS information are supported by LP-WUS. The contents of the indication is up to RAN1 since the LP-WUS signal design and payload have been discussed by RAN1.

Proposal 2: For PEI based subgrouping, detailed LP-WUS information and information delivery method (e.g., bitmap, codepoint) is up to RAN1 decision.

Maximum number of subgroups

RAN2 needs to consider the maximum number of supported subgroups for LP-WUS. As supporting larger number of subgroups achieves better granularity, the larger number of subgroups avoids unnecessary ramp up and paging monitoring. On the other hand, increasing number of subgroups from the maximum number supported by PEI may lead to change of RAN2 specification.

For example, indication of a different number of subgroups (e.g., via system information) and a different subgroup ID should be given to UE for LP-WUS. In addition, if the increased number of subgroups requires increased LP-WUS payload size, the coverage of LP-WUS decreases. Given the situation, support up to 16 (more than 8) subgroups should be considered while maintaining LP-WUS coverage and additional specification impact in RAN2 specification should be minimized.

Proposal 3: Support up to 16 subgroups for LP-WUS.

LP-WUS reception failure

RAN2 agreed that entry/exit condition for LP-WUS monitoring is configured by SIB. To support entry/exit conditions for LP-WUS monitoring, the UE is configured with conditions. For example, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS (e.g., detail threshold value is up to RAN1 decision) can be configured. The satisfied condition (e.g., above the threshold) of received LP-SS can be regarded as successful reception and the UE can keep monitoring LP-WUS. In contrast, if the condition is not satisfied by detecting number of reception failures (e.g., below the threshold), the UE stops LP-WUS monitoring and starts to monitor the legacy PO.

Proposal 4: For LP-WUS exit condition, support number of LP-WUS reception failures.

Measurement both NR-SS and LP-SS

While measuring of entry/exit conditions, a UE being capable of OFDM-based LP-WUR can measure both NR-SS (existing SSB) and LP-SS (i.e., if supported) simultaneously. The measured values can be used for the decision of entry/exit condition. Since the qualities of NR-SS and LP-SS are different metrics (e.g., RSRP and LP-RSRP, respectively), RAN2 should discuss how to use the different metrics for the decisions. For example, using the higher quality or combined quality.

Proposal 5: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Information on neighboring cells

According to the evaluation results from the TR [4], it is obvious that offloading RRM measurement from MR to LP-WUS is essential to avoid frequent wake up of MR from ultra deep sleep and achieve power saving gain from LP-WUS. Considering limited capacity of LP-WUR and reduced power consumption, providing information on neighboring cells (e.g., whether to provide LP-WUS related configuration) should be considered for LP-WUR based RRM measurement. Also, this information may be beneficial that the UE can (re-)select cell with supporting LP-WUS configuration while performing the cell (re-)selection procedure.

Proposal 6: Information on neighboring cells should be supported in LP-WUS.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Do not support dedicated LP-WUS configuration

Proposal 2: For PEI based subgrouping, detailed LP-WUS information and information delivery method (e.g., bitmap, codepoint) is up to RAN1 decision.

Proposal 3: Support up to 16 subgroups for LP-WUS.

Proposal 4: For LP-WUS exit condition, support number of LP-WUS reception failures.

Proposal 5: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Proposal 6: Information on neighboring cells should be supported in LP-WUS.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #126.

[2] Chairman’s notes, 3GPP TSG RAN WG1 #116-bis.

[3] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[4] TR 38.869 V2.0.0 (2023-12), “Study on low-power wake up signal and receiver for NR (Release 18)”.