3GPP TSG-RAN WG2 Meeting #127	R2-2407348

Maastricht, Netherland, Aug 19th – 23rd, 2024

Agenda Item:	8.6.2

Source:	HONOR

Title: 	Further discussion on inter-CU LTM

Document for:	Discussion and Decision

1	Introduction

In Rel-19 Mobility enhancements WI, the following objective was achieved:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured. - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged. - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released. - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling. - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#126, the following agreements were made:

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. - Xn-based inter-CU LTM is prioritized in Rel-19. - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). - The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. - For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. - The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. - Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. - The LTM completion defined for intra-CU LTM is followed for R19 LTM.

LTM was introduced in Rel-18, which aims to reduce the latency during handover. In Rel-18, it was restricted to the scenarios of intra-CU. As the scenario expands to inter-CU in Rel-19, new issues should be taken into consideration compared with the scenario of intra-CU. Therefore, in this contribution, we will take into consideration on the corresponding issues in inter-CU LTM.

2. Discussion

2.1 RRC pre-configuration

From the aspects of mobility procedure, same as in intra-CU case in general, which mainly contains three phases: handover preparation, handover decision and handover completion. In the previous meeting, we had a lot of discussions on the issues involved in RRC pre-configuration, and a lot of conclusions were made. The signalling in detail is up to RAN3. Therefore, in this contribution, we will focus on the new issues in DC scenario.

In the phase of RRC pre-configuration, if MN initiated the SCG LTM or SN initiated the SCG LTM with MN involvement is triggered, MN is responsible to request the potential target SN(s) to execute RRC pre-configuration for the UE by means of the SN Addition procedure.

Proposal 1: While MN initiates SCG LTM or SN initiates SCG LTM with MN involvement, MN is responsible to collect RRC pre-configuration from multiple candidate SNs.

2.2 Early Synch

During UL synchronization with LTM candidate cell(s), after performing TA acquisition based on CFRA, the source-DU acquires TA via the interaction between CU-CU and CU-DU nodes. In detail, the candidate DU notifies the associated CU of the acquired TA. The candidate CU forwards the TA to the source CU. Finally, the source DU acquires the TA from the source CU. Therefore, it is naturally for source DU to manage TA validity. If not, while the TA is about to be used, the source DU has to acquire TA from source CU, which will have negative impacts on cell switch latency.

Upon cell switch, if the TA associated with candidate cell can continue to be maintained by NW side, the signalling overhead might be reduced, since it might not be necessary for UE to re-acquire TA in the following period of time. Therefore, the TA associated with candidate cell should be kept in serving DU.

Proposal 2: After performing UL synchronization with candidate cell(s), the source-DU manages the TA instead of source CU, and after cell switch, the TA associated with candidate cell(s) can continue to be kept in serving DU.

2.3 LTM execution

In Rel-18, it is allowed to activate TCI states of one or multiple cells that are different from the current serving cell in advance. Since the UE can perform DL synchronized with those cells, it facilitates a faster cell switch to one of those cells when cell switch is triggered.

In the candidate cell TCI States activation/deactivation MAC CE, it mainly contains two fields: ‘candidate cell ID’, ‘TCI state ID’. In previous meeting, it was agreed that LTM candidate ID is unique across all the participating gNB-CUs. Therefore, the legacy candidate cell TCI States Activation/Deactivation MAC CE can be reused for inter-CU LTM.

Proposal 3: The TCI state activation/deactivation MAC CE used in R18 intra-CU LTM can be reused for activating TCI state in advance for inter-CU LTM.

When DC is configured, it was agreed in Rel-18 that for intra-CU LTM the UE only releases SCG configuration at MN LTM execution if configured by NW. That is, SCG configuration can be kept or released based on NW side decision. In addition, Rel-19 LTM supports the case where CU is acting as MN and SN is unchanged or SN is released. This mechanism can also be applied to the scenario of inter-CU MN LTM. For the handling on SCG configuration, there are two options as follows:

Option 1: RRC pre-configuration

Whether SCG remains unchanged or released can be indicated via RRC pre-configuration. In candidate cell pre-configuration, such indicator can be included.

Option 2: MAC CE

An MAC CE can be used to indicate the operation on SCG. Although it is flexible, the other issue on how to define the MAC CE should be taken into account.

Currently, we do not have an obvious preference on the two options, and both of the two options can be discussed in further study.

Proposal 4: For the inter-MN LTM in DC scenario, there are two options to handle the SCG for further study:

- Option 1: RRC pre-configuration

- Option 2: MAC CE

In legacy intra-CU scenario, upon cell switch, cell switch notification towards the target DU should be transferred from source DU to the associated CU, and then the CU forwards it to the target DU. However, in inter-CU, such procedure needs to be modified to adapt the extended scenario. Correspondingly, cell switch notification towards the target DU shall be achieved by transferring at the transfer node of source CU and target CU.

Proposal 5: Upon cell switch, cell switch notification procedure involves signaling from target CU towards target DU.

2.4 Security about inter CU LTM

In Rel-18, security update is not supported, which will not result in serious consequences due to the restriction of intra-CU. However, for inter-CU LTM, security update is necessary. For supporting security update in inter-CU scenario, how to keep alignments between source node and target node should be taken into consideration as a start point.

The approach of security change is seen as an essential issue. There are four potential approaches which are shown as follows:

Option 1: Use a new information in MAC CE to deliver the security information.

In L3 HO, the security update parameter (i.e., NCC) is included in the HO command. Refer to the mechanism of security update in L3 HO. The security update parameter (i.e., NCC) can be included in LTM cell switch command. It needs to be assessed whether there are security issues, since a MAC CE message is not protected by integrity protection and ciphering.

Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

Similar to SCPAC, a list of NCC associated with one candidate cell is configured for UE. Upon cell switch, the UE shall choose one unused NCC to be applied in the target cell.

Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch.

UE is pre-configured by the CN with a list of NCC values and the UE determines NCC value upon cell switch. The CN needs to be aware of how the UE would use the next NCC value. Therefore, the signalling interaction between gNB and CN is needed, which will result in increased complexity.

Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

This scheme is very similar to legacy handover with key update via RRC reconfiguration message. It is not aligned with the intention of LTM.

Proposal 6: It is preferred to take the Rel-18 S-CPAC key update mechanism (option 2) as the baseline：the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

3 Conclusions

In this contribution, we discussed the issues related to inter-CU LTM. Based on the discussion, the following observations and proposals are concluded:

Proposal 1: While MN initiates SCG LTM or SN initiates SCG LTM with MN involvement, MN is responsible to collect RRC pre-configuration from multiple candidate SNs.

Proposal 2: After performing UL synchronization with candidate cell(s), the source-DU manages the TA instead of source CU, and after cell switch, the TA associated with candidate cell(s) can continue to be kept in serving DU.

Proposal 3: The TCI state activation/deactivation MAC CE used in R18 intra-CU LTM can be reused for activating TCI state in advance for inter-CU LTM.

Proposal 4: For the inter-MN LTM in DC scenario, there are two options to handle the SCG for further study:

- Option 1: RRC pre-configuration

- Option 2: MAC CE

Proposal 5: Upon cell switch, cell switch notification procedure involves signaling from target CU towards target DU.

Proposal 6: It is preferred to take the Rel-18 S-CPAC key update mechanism (option 2) as the baseline：the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

4 References

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4.

- TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.401, Technical Specification Group Radio Access Network; NG-RAN; Architecture description

- S3-241898, Draft reply LS on security handling for inter-CU LTM in non-DC cases