3GPP TSG-RAN WG2 Meeting #127	R2-2407427

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.11.3

Source:	Samsung

Title:	SBFD Overall and Support of Cross Link Interference

WID/SID:	NR_duplex_evo-Core

Document for:	Discussion and Decision

Introduction

The Rel-19 evolution of duplex operation WID [1] target to support the gNB-side SBFD (Sub-band full duplex) within a TDD carrier. In addition, one major objective is the enhancements of inter-gNB and inter-UE CLI handling.

In the Rel-19 WID [1] and the TR38.855 [2], it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. SBFD operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or flexible symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: -- skipped - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. -- skipped - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact -- skipped - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we will further address cross link interference (CLI) for SBFD in details.

Discussion

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI). In Rel-16, the cross-link interference mitigation to support flexible resource adaptation for unpaired NR cells (i.e. dynamic TDD control) was introduced, and specified the cross-link interference measurements and reporting at a UE (i.e., CLI-RSSI and/or SRS-RSRP). To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces, and the victim UEs can be configured to perform CLI measurements.

Figure 1. Example of the overall cell structure for CLI

In Rel-19, following two aspects need to be specified based on the WID [1]

- Support gNB-to-gNB CLI handling scheme(s) (i.e. UL resource muting for PUSCH; see the details in TR 38.858 [2])

- Support UE-to-UE CLI handling scheme(s) (i.e. L1 based UE-to-UE CLI measurement and reporting; see the details in TR 38.858 [2])

For the UL resource muting for PUSCH, we expect RRC impacts on configuration of the RRC parameters but no RAN2 impacts on the procedures are expected. However, there would be some RAN2 impacts for L1 based CLI measurement and reporting in terms of signalling support and UE procedures. In addition, information exchange among gNBs to support CLI would also impacts on the RRC signalling and overall procedures.

The SBFD even makes TDD DL/UL patterns more complex between neighbour cells, and increase cross-link interference. CLI of gNB-to-gNB would happen between DL at aggressor gNB and UL at victim gNB so the at least SBFD TDD DL/UL configuration needs to be signalled between gNBs of interest. According to the RAN1 discussion, this SBFD time/frequency configuration should be semi-static so it could be provided cell common configuration by system information message (e.g. SIB1). Then, it is necessary to exchange information of the semi-static cell-specific SBFD T/F configuration between the gNB via Xn message between victim and aggressor gNB. Similar with the sharing of TDD UL/DL configuration between gNB by Xn, same level of new IE will be introduced to share the cell-specific SBFD T/F configuration. In this case, the negotiation between the gNBs are not supported but it could reduce the complexity in terms of the signalling overhead.

Proposal 1: Source gNB request SBFD support and configuration (semi-static cell-specific SBFD time/frequency configuration) to other gNBs in Xn Setup Request or NG-RAN node Configuration Update message

Proposal 2: Semi-static cell-specific SBFD time/frequency configuration is provided by SIB1 along with the TDD UL/DL configuration.

If UE is connected to the source cell, the enhancements of changing the SBFD configuration by the dedicated RRC configuration could be considered but only frequency location will be considered rather than the time location of SBFD slot/symbols in this case. From our understanding, common SBFD configuration is baseline and it is simple and sufficient, the additional SBFD frequency location adaptation could be considered but we think this kind of enhancements are first discussed in RAN1.

Proposal 3: RAN2 wait the RAN1 decision on support of the SBFD configuration by the dedicated RRC configuration.

For UL resource muting scheme, the victim gNB receives semi-static SBFD and TDD UL/DL configuration from aggressor gNB, then can configure to some of UEs on UL resource muting configuration to avoid UL reception failure due to the DL interference from aggressor gNB or to measure CLI from aggressor gNB. The victim gNB will configure UE(s) on the UL resource muting related configuration e.g. muting pattern, etc. We assume RAN1 will provide the relevant RRC parameters and signalling support in RRC is required.

Proposal 4: For the support gNB-to-gNB CLI handling, RAN2 wait the RAN1 parameter list on UL resource muting for PUSCH i.e. only RRC signalling support is expected.

For L1 based UE-to-UE CLI measurement and reporting, since the semi-static SBFD configuration cannot specify the scheduler level resource usage (i.e., on slot level t/f assignment is considered), and corresponding interference level cannot be identified, there should be a way to measure the interference at UE. This measurement should embrace slot level time duration to check up interference on/off time, since actual interference is made in schedule. Legacy CLI measurement procedure is to use L3 RRM operation (i.e., UE measures resource configured, and using L3 measurement, and report via L3 based report).

- UE is configured with MeasObjCLI, and reportConfig with CLI-PeriodicalReportConfig, or CLI-EventTriggerConfig

  - MeasObjCLI: SRS resource or CLI-RSSI resource configuration

  - Reportconfig

    - CLI-periodicalReportingConfig: reportInterval, amount, quantity, maxReportCLI

    - CLI-EventTriggerConfig:

      - reportOnLeave, threshold, hysteresis, TTT, rerpotInterval, amount, maxReportCLI (max # of resource to include in the MR

However, it is necessary to measure and report CLI in symbol level time scale since SBFD configuration can be per symbol level. L3 time average blur the exact CLI level in smaller time scale so the L1 based CLI measurement operation for SBFD is needed. We provide the oevarall procedure to support the L1 based UE-to-UE CLI measurement and reporting:

- Step 1: Exchange of resource information to measure CLI between gNBs

Victim gNB receives the SRS resource configuration that aggressor gNB will assign to the aggressor UE via Xn message which can be used for SBFD configuration delivery. Once victim gNB receives the SRS resource configuration for the aggressor gNB, it should adjust or translate the received resource from aggressor cell to the victim cell resource:

- Only overlapped resources can be used for CLI resource (inter-freq case);

- Each SRS or CLI resources should be indicated in one of serving cells for victim UE, and using DL BWP of that cell, which includes those SRS or CLI resources

- Above adjusted / translated resource information should be signaled to the victim UE by victim gNB

- Step 2: CLI measurement configuration at (victim) gNB

- Serving cell configures resource for CLI measurement to a specific UE (possible victim UE)

  - Measuring quantity: SRS-RSRP, CLI-RSSI same as legacy L3 based CLI resources.

    - SRS-RSRP: Victim UE will measure aggressor UE’s SRS transmission

    - CLI-RSSI: Victim UE will measure aggressor UE’s UL transmission plus any interference power on its DL reception resource.

      - This resource is determined by victim gNB and there is no explicit exchange of TX resource from aggressor gNB to the victim gNB.

    - Etc.

  - Each measurement resource would be either periodic resource, semi-persistent resource or aperiodic resource.

- Based on resource type above, measurement report can be possible as

  - Periodic reporting, semi-persistent reporting, or aperiodic report on PUCCH/PUSCH

If we follow the legacy CSI report framework by L1 signaling, following approach could be reused for the L1 based UE-to-UE CLI measurement and reporting as well. If network determines the RS type, then it will further determine the report type available for the RS type. This measurement resource and report configuration consist of a single measurement instance, i.e., ID, which can be referred by the DL MAC CE/DCI for its corresponding operation.

Table 1. L1 based CLI measurement and report configuration

| Resource Configuration | Report Configuration |  |  |
| --- | --- | --- | --- |
| RS Configuration | Periodic report | Semi-Persistent report | Aperiodic report |
| Periodic RS | No dynamic triggering/activation | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Semi Persistent RS | Not Supported | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Aperiodic RS | Not Supported | Not Supported | Triggered by DCI, additionally by MAC CE |

In the above Table 1, type of signaling MAC CE and DCI is just one possible cases based on the consideration of number of bits necessary and time scale to be signaled in each case. DL MAC CE or DCI can be designed

New signaling methods for L1 based UE-to-UE CLI measurement and reporting are necessary, for example:

- SP RS activation/deactivation DCI/ MAC CE: to activate/deactivate the RS measurement

- SP RS reporting on PUCCH Activation/deactivation DCI/MAC CE: enable SP report

- AP report trigger state sub-selection DCI/MAC CE: subset of AP report triggering object, and DCI fill finally indicate the exact configuration to report AP.

- DCI for AP report: within subset of CLI measure/report configuration indicated by AP report trigger state subselection MAC CE, one configuration can be choosen via DCI.

- DCI for SP report on PUSCH: to indicate the PUSCH reporting.

- L1 report on PUCCH/PUSCH or UL MAC CE: to include the measurement result for CLI

In short, once UE is configured with the object to be measure (i.e., L1 CLI RSes), UE measures the RSes, and report according to the report type given in RRC configuration.

- Periodic report: UE measures RS, and report on the periodicity w.o. dynamic activation

- Semi-persistent report: UE doesn’t measure the RS at first, if network indicate the activation of SP resource via MAC CE, UE will be measure them, and report is indicated either by DL MAC CE (indicating reporting on PUCCH), or by DCI (reporting on PUSCH) in its own periodicity.

- Aperiodic report:

  - Option 1. UE doesn’t measure the RS at first, but if network indicate the report either by DL MAC CE and DCI

  - Option 2. UE measure from the first, and if network indicate the AP report, then it will report the result.

In addition, the SRS configuration at the aggressor gNB is dynamic because this SRS configuration is dedicated configuration per UE. Plus, the SRS activation at the aggressor gNB would be dynamic. In this case, the exchange of the information on the activation/deactivation of SRS at aggressor gNB is not easy to be exchanged with the victim gNB. For handling this, the information exchanged between gNBs can further include:

- Indication of immediate activation of the time info in which SRS is activated at the aggressor gNB; or

  - Activation time, and end time; or

  - Activation time, and duration;

  - Time window info where SRS activation is done within

- Activation or deactivation indication itself can be included.

If dynamic activation/deactivation indication is not feasible between gNB, SRS CLI measurement on SP resource would be only possible in intra-CU case (i.e., no Xn signaling needed).

Proposal 5: For L1 based UE-to-UE CLI measurement and reporting, the legacy CSI resource and report structure is reused and need to check how to exchange the information on the activation/deactivation of SRS at aggressor gNB.

Conclusion

Proposal 1: Source gNB request SBFD support and configuration (semi-static cell-specific SBFD time/frequency configuration) to other gNBs in Xn Setup Request or NG-RAN node Configuration Update message

Proposal 2: Semi-static cell-specific SBFD time/frequency configuration is provided by SIB1.

Proposal 3: RAN2 wait the RAN1 decision on support of the SBFD configuration by the dedicated RRC configuration.

Proposal 4: For the support gNB-to-gNB CLI handling, RAN2 wait the RAN1 parameter list on UL resource muting for PUSCH i.e. only RRC signalling support is expected.

Proposal 5: For L1 based UE-to-UE CLI measurement and reporting, the legacy CSI resource and report structure is reused and need to check how to exchange the information on the activation/deactivation of SRS at aggressor gNB.

References

- RP‑241614	Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD); RAN#104

- TR 38.858 v2.0.0	Study on Evolution of NR Duplex Operation (Rel-18)”; RAN#102