3GPP TSG-RAN WG2 Meeting #127  	                                                      R2-2407542

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.4

Source:	Rakuten Mobile Inc

Title:	Discussion on Failure Handling

Document for:	Discussion

# Introduction

Agreements on Control Plane

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, A-IoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by A-IoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreements on Paging

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

Ambient IoT (A-IOT) focuses on enabling very low-power, low-cost devices to communicate within 3GPP networks. These devices have limitations, so it’s really important to handle Random Access Channel (RACH) procedures well. This paper looks at different failure scenarios in both 2-Step and 4-Step RACH procedures, including contention-based (CBRA) and contention-free (CFRA) random access. We suggest some improvements to current protocols to better manage these failures, in line with the frameworks and requirements discussed in past 3GPP RAN2 meetings.

# Discussion

## 2.1. Paging and MSG1 failures.

Paging failure can be categorized into following.

- Device Fail to receive Paging message.

A device doesn't get the paging message because of interference, a weak signal, or being out of coverage area

- Paging Message received but Device does not respond.

When an A-IOT device gets the paging message but doesn’t respond, it might be due to low battery, internal device problems, or network limitations. This impacts the next steps in the random access procedure.

- Failure in initial access attempt (A-IOT Msg1) -CFRA

In contention-free random access, the first access attempt might fail due to incorrect initial parameters, device-specific issues, or RF problems

- Failure in Initial Access Attempt (A-IOT Msg1) – CBRA

In contention-based random access, the first attempt might fail because of high contention or interference, affecting the efficiency of the random access process

Proposals to handle Paging Failures:

- Proposal 1: Adaptive Retry Intervals: RAN2 specifications  enable adaptive retry intervals for resending the paging message. Mechanism can adjust retry intervals based on network conditions and device feedback, boosting the chances of successful paging for both CBRA and CFRA

- Proposal 2: Use of Different Identifiers: Use different identifiers in paging messages to differentiate between devices/groups and ensure proper responses

- Proposal 3: Based on RAN1 discussion RAN2 introduce energy thresholds in paging messages ensures that devices only respond if they have enough energy, conserving power and reducing unnecessary network traffic.

## 2.2. RACH MSG2/3 Failures

RACH MSG2/3 Failures can be classified into following.

- Contention Resolution Failure (A-IOT Msg2) - CBRA

Contention resolution failures can happen but probably can be managed by the network/configurations.

- Reader Failure to Receive MSG3

MSG3 contains the device ID and other upper-layer data. If the reader fails to receive or correctly decode MSG3, it leads to a transmission failure that needs to be addressed.

Proposals to MSG2/MSG3 Failures:

- Proposal 4: Detection of failure and triggering retries. The reader should have mechanisms to detect when MSG3 is not received or decoded correctly and trigger retries. This can be determined by the absence of an expected response within a predefined timeframe.

- Proposal 5: Use of MSG4: If MSG3 is not received, the reader can send an MSG4 to request a retransmission or provide new instructions to the device.

## 2.3. Energy Depletion During RACH

Background: Energy constraints are significant for A-IOT devices. Failures due to energy depletion can occur at any stage of the RACH process, affecting the device's ability to complete the transmission successfully. As discussed in Proposal 3 there is a need to Implement energy monitoring protocols to ensure devices only attempt transmissions when they have sufficient energy. This method helps conserve energy and improves the reliability of transmissions.

- Proposal 7: Efficient Signaling Mechanisms: RAN2 to specify energy-efficient signaling mechanisms to indicate low energy levels to the reader. This approach allows devices to communicate their energy status without expending excessive power.

# Conclusions

In this contribution we discussed possible failure related to Paging and RACH process and also made following proposals for RAN2.

- Proposal 1: Adaptive Retry Intervals: RAN2 specifications enable adaptive retry intervals for resending the paging message. Mechanism can adjust retry intervals based on network conditions and device feedback, boosting the chances of successful paging for both CBRA and CFRA

- Proposal 2: Use of Different Identifiers: Use different identifiers in paging messages to differentiate between devices/groups and ensure proper responses

- Proposal 3: Based on RAN1 discussion RAN2 introduce energy thresholds in paging messages ensures that devices only respond if they have enough energy, conserving power and reducing unnecessary network traffic.

- Proposal 4: Detection of failure and triggering retries. The reader should have mechanisms to detect when MSG3 is not received or decoded correctly and trigger retries. This can be determined by the absence of an expected response within a predefined timeframe.

- Proposal 5: Use of MSG4: If MSG3 is not received, the reader can send an MSG4 to request a retransmission or provide new instructions to the device.

- Proposal 6: Efficient Signaling Mechanisms: RAN2 to specify energy-efficient signaling mechanisms to indicate low energy levels to the reader. This approach allows devices to communicate their energy status without expending excessive power.

# References

[1]	Chair notes, RAN2#125bis.

[2]   Chair notes, RAN2#126