3GPP TSG RAN WG2#127bis	R2-2407913

Hefei, China, 14th - 18th October 2024

3GPP TSG-RAN WG1 Meeting #118	TDoc R1-2407424

Maastricht, The Netherlands, 19th-23rd August 2024

# Title:	LS response on power control parameters for unified TCI state framework

# Response to:	R1-2407413 LS response on power control parameters for unified TCI state framework

# Release:	Rel-17

# Work Item:	NR-feMIMO-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name: Claes Tidestav

E-mail Address: claes.tidestav (at) ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 thanks RAN2 for the questions on the power control parameter configuration for the unified TCI framework.

In the LS R1-2407413, RAN2 poses the questions:

Reply to question 1: Yes. In PUSCH-PowerControl, it should be possible to configure all parameters except the following:

- p0-AlphaSets

- pathlossReferenceRSToAddModList

- pathlossReferenceRSToReleaseList

- sri-PUSCH-MappingToAddModList

- sri-PUSCH-MappingToReleaseList

- pathlossReferenceRSToAddModListSizeExt-v1610

- pathlossReferenceRSToReleaseListSizeExt-v1610

- sri-PUSCH-MappingToAddModList2-r17

- sri-PUSCH-MappingToReleaseList2-r17

In PUCCH-PowerControl, it should be possible to configure all parameters except the following:

- p0-Set

- pathlossReferenceRSs

- pathlossReferenceRSs-v1610

Reply to question 2: The existing parameters can be reused.

2. Actions:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #118-bis	14 October – 18 October 2024	Hefei, China

TSG RAN WG1 Meeting #119	18 November – 22 November 2024	Orlando, USA