3GPP TSG RAN WG2#127bis	R2-2407917

Hefei, China, 14th - 18th October 2024

3GPP TSG-RAN WG1 Meeting #118	R1-2407533

Maastricht, Netherlands, August 19 – 23, 2024

Title:	LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation

Release:	Release 19

Work Item:	NR_duplex_evo

Source:	TSG RAN WG1

To:	TSG RAN WG3

Cc:	TSG RAN WG2

Contact person:	Stephen Grant

stephen.grant@ericsson.com

# 1	Overall description

The WID objective for CLI handling related to information exchange among gNBs is as follows:

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - Information exchange among gNBs, including [RAN3]

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

    - …

    - Note: Without dedicated optimization for dynamic/flexible TDD.

- …

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

While the WID objective lists only RAN3 as responsible working group, RAN1 has discussed the sub-bullets of the objective and would like to clarify several PHY/L1-related aspects that are not typically discussed in RAN3. RAN1 would like to inform RAN3 (RAN2 is cc’d) of the following RAN1 agreement.

Agreement

Send an LS to RAN3 with the following information (cc: RAN2)

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

# 2	Actions

To TSG RAN3

ACTION: 	RAN1 respectfully asks RAN3 to take the above information into account in their further work.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#118bis	14th – 18th October 2024	Hefei, China

RAN1#119	18th – 22nd November 2024	Orlando, FL