3GPP TSG RAN WG2#127bis	R2-2407918

Hefei, China, 14th - 18th October 2024

3GPP TSG RAN1#118 Meeting	R1-2407534

Maastricht, NL, August 19th – 23rd, 2024

Title:	LS on higher layer parameters for DCI format 2_3

Response to:

Release:	Rel-16

Work Item:	NR_newRAT-Core, TEI16

Source:	RAN1

To:	RAN2

Cc:	-

Contact person:

#### Name:	Jiayi Yang

#### E-mail Address:	yangjiayi@catt.cn

Attachments: None

# 1		Overall description

SRS carrier switching was introduced in Rel-15. As a part of necessary higher layer parameters, a DCI format 2_3 field configuration type should be configured with either typeA or typeB.

According to TS 38.212 7.3.1.3.4 and TS 38.213 11.4, this higher layer parameter is needed in both cases, i.e., “for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control”. But in current TS 38.331, the field description of “typeA/typeB” only refers to “SRS transmission on a PUSCH-less SCell” case, and the other case “SRS power control is not tied with PUSCH power control” is missing.

In this case, TS 38.331 may give a misguided impression that SRS-CarrierSwitching IE cannot be configured for UL carrier(s) where PUSCH is configured and SRS power control is not tied with PUSCH power control.

# 2		Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to consider a modification to the IE SRS-CarrierSwitching in TS 38.331 to clarify how DCI format 2_3 can also be used for TPC indication in the UL carrier(s) where PUSCH is configured and SRS power control is not tied with PUSCH power control.