3GPP TSG-RAN WG2 Meeting #127bis                                                              R2-2407946

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on the Functionality Aspects for Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion

1. Introduction

This paper further discusses the functionality-related issues with the following aspects,

- AS ID

- Msg size indication

- Segmentation/reassembly candidate solution/feasibility

- Information visible to the reader

- Higher layer repetition

- Energy status reporting

The Section 2 gives the detailed discussion, and followed by Section 3 that summarizes this paper.

2. Discussion

AS ID

In last meeting [1], RAN2 initiated the AS ID-related discussion with the following agreement,

- In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. - FFS if reader assigns the AS ID for scheduling purposes

The corresponding discussion is also covered in the post email discussion related to RACH. As mentioned in the post email discussion, RAN1 had discussed the scheduling-related issues for A-IoT,

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported)FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

As highlighted above, the “potential” information for D2R scheduling/R2D reception includes the ID associated with the device. So firstly, we suggest being confirmed by RAN1 on the need of such AS ID. Besides, if RAN1 determines to introduce AS ID for scheduling purpose, RAN1 is still discussing whether to use L1 control signalling or higher-layer signalling to allocate the AS ID. That means, if L1 control signalling is used, the higher-layer is not aware of this AS ID, so RAN2 does not need to discuss this issue anymore.

Proposal 1: Wait for RAN1 further discussion on the need of AS ID and the allocation of AS ID by higher-layer signalling or L1 control signalling.

Msg size indication

After the RAN2 discussion in last meeting, it was agreed to further study whether to need a simple msg size reporting than legacy BSR from the device to reader,

- RAN2 will study the need and use of a simple message “size” reporting to the reader

Whether to need msg size indication

According to 38.848,

The design target of maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device, based on the maximum application layer packet size.

Therefore, the UL data transmitted to reader is more or like small data. It seems workable to relay on implementation to ensure enough resource within one UL scheduling, so the device is not expected to have remaining data after performing one UL transmission for the response of the inventory or command service. But the drawback of this way is the reader always needs to provide large-enough resource regardless of the remaining data by the device, which is not resource-efficient.

Observation 1: If the device does not send any msg size indication to reader, the reader needs to always provide large-enough resource regardless of remaining data in the device, which is not resource-efficient.

Therefore, it has benefits that the reader is aware of the data volume that needs to send by the device.

Proposal 2: The device can send the msg size indication to the reader for facilitating the resource scheduling by the reader.

How to indicate the msg size

Then the main controversy during last meeting discussion was how to implement the “simple” msg size indication to the reader.

Some companies suggested 1-bit indication is enough to let the reader know there is still remaining data in the device waiting for UL grant. This is not efficient since the reader can only schedule the subsequent PDRCH transmission blindly, and thus may still not meet the requirement for remaining data from the device.

Observation 2: 1-bit indication is not efficient since the reader can only perform blind scheduling for the subsequent transmission, which may not meet the requirement for remaining data.

Some other companies suggested indicating more information to the reader, e.g., the number of bits for transmission. This is indeed similar as legacy BSR MAC CE, where the range of remaining data is indicated to the gNB,

Table 6.1.3.1-1: Buffer size levels (in bytes) for 5-bit Buffer Size field Index; BS value; Index; BS value; Index; BS value; Index; BS value. 0; 0; 8; ≤ 102; 16; ≤ 1446; 24; ≤ 20516. 1; ≤ 10; 9; ≤ 142; 17; ≤ 2014; 25; ≤ 28581. 2; ≤ 14; 10; ≤ 198; 18; ≤ 2806; 26; ≤ 39818. 3; ≤ 20; 11; ≤ 276; 19; ≤ 3909; 27; ≤ 55474. 4; ≤ 28; 12; ≤ 384; 20; ≤ 5446; 28; ≤ 77284. 5; ≤ 38; 13; ≤ 535; 21; ≤ 7587; 29; ≤ 107669. 6; ≤ 53; 14; ≤ 745; 22; ≤ 10570; 30; ≤ 150000. 7; ≤ 74; 15; ≤ 1038; 23; ≤ 14726; 31; > 150000.

Instead of indicating the detailed bit number, the similar method as BSR can be reused to indicate the TBS larger than the remaining data in the device. For example, RAN1 may define the candidate TBS values of {128 256 512 1024}. The device now has 600 bits to send, and after the first transmission with 512 TBS, the device has 88 bits left (< 512 TBS). Then the device sends a 2-bit bitmap to indicate the 128 TBS for the subsequent scheduling.

Propose 3: The device can indicate the expected TBS for the subsequent transmission by the bitmap associated with the candidate TBS (defined by RAN1) in the msg size indication.

How to trigger msg size indication

Another issue is how to trigger the msg size indication. In legacy, the BSR can be triggered by UE upon new data arrival with a higher LCP. This is not allowed by A-IoT as now we do not support DO-A like PDRCH transmission by the device.

Observation 3: The device cannot send the msg size indication without the trigger from reader.

For the inventory use case, the device does not need to send the msg size indication as the device ID usually has a fixed size. For the command use case, if the device needs to response the corresponding data to the reader (for example, read command), the device should be allowed to send the msg size indication to the reader.

Proposal 4: Upon receiving the command service, e.g., read, the device is allowed to send the msg size indication to the reader.

Then the further details are how to send the msg size indication to the reader. For the command service, now up to reader implementation, it can directly send the command to the device or firstly trigger inventory and then followed by command.

Case 1: Directly send the command to device

Upon receiving the command from the reader, if the device performs 3-step CBRA and the selected access occasion for A-IoT Msg1 is large-enough, it can send the msg size indication to the reader along with A-IoT Msg1; if the device performs 2-step CBRA, and the device observes that the UL transmission is not large-enough to send all remaining data (PDRCH content except for random number), it should be allowed for the device to send the msg size indication to the reader  along with A-IoT Msg1; if the device performs CFRA, similar with the 2-step CBRA, the device should be allowed to send the msg size indication to the reader along with the data transmission, when it observes the dedicated UL transmission is not large-enough to carry the whole data.

Proposal 5a: The device should be allowed to send the msg size indication along with A-IoT Msg1 of 3-step CBRA.

Proposal 5b: The device should be allowed to send the msg size indication along with A-IoT Msg1 of 2-step CBRA, if the PDRCH is not large-enough to send the supposed data.

Proposal 5c: The device should be allowed to send the msg size indication along with the data transmission of CFRA, if the PDRCH is not large-enough to send the whole supposed data.

For 3-step CBRA, the device should acquire the UL grant for data transmission of A-IoT Msg3. Then similar with the CFRA, if the device observes that the specific UL grant is not large-enough to carry the whole data of A-IoT Msg3 (regardless of the UL grant acquired from A-IoT Msg2 or other potential ways), it should be allowed for the device to send the msg size indication along with the transmission of A-IoT Msg3.

Proposal 5d: The device should be allowed to send the msg size indication along with the transmission of A-IoT Msg3, if  the PDRCH is not large-enough to send the whole supposed data.

Case 2: Firstly trigger inventory and followed by command

If the reader firstly trigger inventory and then followed by sending the command, the device firstly accesses to the reader and then sends the data to the reader upon receiving the command. Since the device is not aware of whether there is a command service followed by inventory, it will not send the msg size indication before receiving command. Upon receiving the command from reader, regardless of the UL grant for data transmission is acquired from the command or other potential ways, it should be allowed for the device to send the msg size indication along with the data transmission, if the device observes that the UL grant is not large-enough to send the whole data.

Proposal 5e: For the procedure of inventory + command, the device should be allowed to send the msg size indication along with the data transmission, if the  PDRCH transmission after receiving the command is not large-enough to send the whole supposed data.

Segmentation/reassembly candidate solution/feasibility

Regarding whether to need the segmentation/reassembly, we can further wait the reply LS from RAN1/SA2,

- Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size.

If RAN1 confirms that the candidate TBS may not fulfil the requirement for upper layer data transmission, then the segmentation/reassembly is needed. In the following, we further provides our view on how to implement the segmentation/reassembly if confirmed by RAN1/SA2.

Firstly, we would like to point out that buffer is not mandatory for segmentation/reassembly, because the response of inventory or command comes from the stored information in the device rather than from the buffer. The device can opt to initially transfer only the first half or a portion of the message in order that fits the transport block size (e.g. 512bits), along with an indication to the reader that there is a remainder of the message to be transmitted in a subsequent transmission. This indication is crucial for the reader to be aware of the pending transmission due to the segmentation for upper layer data.

Take the following example for information: The A-IoT MAC layer of the device has a pending PDU with the size of 1000 bits to response the read command. Currently the device receives the scheduling with a TBS of 258 bits, then the device sends the first X bit (X < 258) along with the segment indication to let the reader know this is a segment of a A-IoT MAC PDU. Upon receiving one or more subsequent scheduling from the reader, the device then proceeds to transfer the remaining information of the A-IoT MAC PDU in order. In the last transmission for this A-IoT MAC PDU, the device does not include the segment indication along with the data transmission, so the reader confirms the reception of the whole A-IoT MAC PDU.

Note that buffer-free segmentation by device avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

Observation 4 Buffer-free UL segmentation by device, avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

Proposal 6a: Buffer-free UL segmentation by device is feasible for device without increasing cost, if the segmentation/reassembly is confirmed by RAN1/SA2.

Proposal 6b: The A-IoT MAC layer of the device should send the segment indication along with the segmented data within one TB transmission.

Information visible to the reader

On the visibility of A-IoT information to reader, it was generally agreed that,

- At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional

FFS on command type

As agreed above, the reader can know the service type, e.g., inventory or command. When the service type is command, the reader can further determine which procedure to use (command only or inventory + command) for this service request.

Regarding whether to further need the command type such as read, write and disable, this may further depend on the detailed procedure for these commands. For the read command, the device will send the data as per the reader command, thus the reader needs to include the UL grant over the command or other potential ways. For the write or disable commands, whether to have feedback to reader (for example, ACK) needs further clarified. If no any feedback for write or disable commands, the reader needs to know the command type so that it can determine whether to send the UL grant over the command or other potential ways.

Proposal 7: The reader doesn’t need to know the specific command type (e.g. write or disable) unless there is clear requirement from SA2 that some specific commands (e.g. write or disable) does not need feedback from the device.

FFS on mandatory or optional for device info

On the targeted devices or approximate number of devices, it is mainly used for slot-ALOHA to allocate the access round/occasion. This seems more or less the optimization for slot-ALOHA.

Proposal 8: It’s optional for CN to provide approximate number of target devices to the reader.

Higher layer repetition

R2D repetition by higher layer

For the R2D transmission, according to RAN1 agreement,

Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

That is to say, if the reader does not receive the D2R transmission within the [TR2D_min, TR2D_max], it can be left reader implementation to re-transmit the higher-layer inventory or command.

Proposal 9: It’s up to reader implementation to support the higher-layer R2D repetition.

D2R repetition by higher layer

For the D2R transmission, RAN1 had confirmed to support the PDRCH repetition for reliability. Whether to need the higher layer repetition is up to RAN2.

Since the repetition information can be indicated to the reader according to RAN1 discussion, it can be left to reader implementation to indicate a large-enough repetition number so that it can meet the reliability under the current channel. Any higher layer repetition will complicate the device implementation, as the device needs to monitor the feedback from the reader.

Proposal 10: Do not support the higher layer D2R repetition.

Energy status reporting

In last meeting, some companies also proposed to indicate the energy status to the reader, since the device may need to re-charge during RACH or data transmission. The mentioned options are as follows,

Proposal 2: It is proposed to discuss if one bit of information is sufficient for any use case highlighted above or there is a need to represent the energy status in a different way (e.g. sufficient energy for sending of a particular number of bits or an expected time when the device will be ready for subsequent operations or any others)

Option 1: 1-bit to indicate whether to have enough energy for the subsequent transmission with the same data size as the last one;

Option 2: Indicate the number of data volume that the remaining energy can support;

Option 3: Indicate the expected time ready for subsequent transmission (re-charging during this period);

Note that this issue had been discussed in last RAN1 meeting with the following agreement,

For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

So it is suggested firstly evaluating the impact of device unavailability by charging under RAN1.

Proposal 11: RAN2 to not discuss the potential impact of device unavailability by charging before further RAN1 progress.

3. Conclusion

Based on the analysis in section 2, the observations and proposals are summarized as follows:

Observation 1: If the device does not send any msg size indication to reader, the reader needs to always provide large-enough resource regardless of remaining data in the device, which is not resource-efficient.

Observation 2: 1-bit indication is not efficient since the reader can only perform blind scheduling for the subsequent transmission, which may not meet the requirement for remaining data.

Observation 3: The device cannot send the msg size indication without the trigger from reader.

Observation 4 Buffer-free UL segmentation by device, avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

AS ID

Proposal 1: Wait for RAN1 further discussion on the need of AS ID and the allocation of AS ID by higher-layer signalling or L1 control signalling.

Msg size indication

Proposal 2: The device can send the msg size indication to the reader for facilitating the resource scheduling by the reader.

Propose 3: The device can indicate the expected TBS for the subsequent transmission by the bitmap associated with the candidate TBS (defined by RAN1) in the msg size indication.

Proposal 4: Upon receiving the command service, e.g., read, the device is allowed to send the msg size indication to the reader.

Proposal 5a: The device should be allowed to send the msg size indication along with A-IoT Msg1 of 3-step CBRA.

Proposal 5b: The device should be allowed to send the msg size indication along with A-IoT Msg1 of 2-step CBRA, if the PDRCH is not large-enough to send the supposed data.

Proposal 5c: The device should be allowed to send the msg size indication along with the data transmission of CFRA, if the PDRCH is not large-enough to send the whole supposed data.

Proposal 5d: The device should be allowed to send the msg size indication along with the transmission of A-IoT Msg3, if  the PDRCH is not large-enough to send the whole supposed data.

Proposal 5e: For the procedure of inventory + command, the device should be allowed to send the msg size indication along with the data transmission, if the  PDRCH transmission after receiving the command is not large-enough to send the whole supposed data.

Segmentation/reassembly candidate solution/feasibility

Proposal 6a: Buffer-free UL segmentation by device is feasible for device without increasing cost, if the segmentation/reassembly is confirmed by RAN1/SA2.

Proposal 6b: The A-IoT MAC layer of the device should send the segment indication along with the segmented data within one TB transmission.

Information visible to the reader

Proposal 7: The reader doesn’t need to know the specific command type (e.g. write or disable) unless there is clear requirement from SA2 that some specific commands (e.g. write or disable) does not need feedback from the device.

Proposal 8: It’s optional for CN to provide approximate number of target devices to the reader.

Higher layer repetition

Proposal 9: It’s up to reader implementation to support the higher-layer R2D repetition.

Proposal 10: Do not support the higher layer D2R repetition.

Energy status reporting

Proposal 11: RAN2 to not discuss the potential impact of device unavailability by charging before further RAN1 progress.

4. References

[1] Draft_RAN2_127_Meeting_Report_v1