3GPP TSG-RAN WG2 Meeting #127bis	R2-2407947

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.3

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In this contribution, the A-IoT paging aspects and the required contents for A-IoT in the identified procedures will be further discussed.

2	Discussion

2.1	 Monitoring of A-IoT paging and energy harvesting impacts

In RAN2#126, it has been agreed that the device can receive as long as it has enough energy.

3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Then in RAN1#118, RAN1 discussed the energy harvesting related issues. Refer to [2, 3], two kinds of device state mode were proposed, and RAN1 discussed whether the device states is known and controlled by reader. During the meeting, RAN1 only agreed to study two directions on whether the reader knows and could control the device state, but no consensus was reached on the device states.

FL1 High Priority Proposal 4.1-1b RAN1 studies energy harvesting impacts in the following way: - Case 1: Device is assumed to have two states: ON, OFF - Case 2: Device is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication, running a clock - OFF state supports at least: energy harvesting - SLEEP state supports at least: - Maintaining a memory content from ON state - Maintaining Running a clock or maintaining a timer (RAN1 to discuss purpose(s) of clock) - Energy harvesting - FFS: monitoring some reception - SLEEP state does not support at least: transmission, reception for communication - Identify reader knowledge / control of the above states for case 2 - RAN1 is not required to establish a consensus model for the duration of each of the states Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

Observation 1: RAN1 is discussing energy harvesting related issues, including device state and whether the reader knows and could control the device state, but there has not been much progress so far.

From RAN2’s perspective, the device behaviour of monitoring A-IoT paging message is impacted by the RAN1’s decision, e.g., whether sleep state will be supported and whether the state could be controlled by reader. Hence, it is proposed:

Proposal 1: Postpone the discussion on monitoring A-IoT paging message and device unavailability due to energy harvesting until RAN1 has clear progress.

2.2	 Multiple/subsequent A-IoT paging

In RAN2#127, it was agreed that the reader could trigger multiple/subsequent A-IoT paging for the same service request from CN. How to avoid devices duplicate response for the same service request is FFS.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

Considering there may be parallel service requests on the A-IoT interface, it is necessary for the devices to recognize which A-IoT paging messages belong to one service request from CN and which service request has been responded. A service ID can be introduced in the A-IoT paging message for that purpose. The service ID is used to identify one service request from CN, i.e., the A-IoT paging messages associated with the same service request from CN has the same service ID. Once the reader receives a service request from CN, it assigns a service ID for this service and includes the service ID in the A-IoT paging messages.

Proposal 2: Introduce a service ID in the A-IoT paging message, identifying this service request from CN, i.e., the A-IoT paging messages associated with the same service ID identify the same service request from CN.

Once the device has responded the service request, it does not need to respond the subsequent A-IoT paging messages of this service request. Considering the reader may reuse the same service ID for different requests without overlapping time, for one certain service request, the “no response” operation based on its service ID should be limited to before the end of this service request to avoid missing paging. Hence, it is proposed:

Proposal 3: Once the device has responded the paging message with a service ID, it does not respond the subsequent A-IoT paging messages with the same service ID.

As for how the device determines end of the service request, i.e., before which point the A-IoT paging messages with the same service ID belong to one same service request from CN, some candidate solutions are given as following. RAN2 could have a further investigation on these solutions.

- Option 1: A timer is used by device to determine end of the service request, the length of timer could be predefined in spec (e.g., 30s) or configured by reader in the R2D message. FFS the detailed device behaviours related to the timer, including when to start the timer and corresponding UE behaviours and what info does the device needs to store during this procedure.

- Option 2: The device is explicitly indicated by the reader with an “end” indication in the last A-IoT paging message of a service request. Specifically, for a certain service request, if the “end” bit is set as “1”, the device will consider this is the last A-IoT paging message of a service request. FFS the detailed devices behaviours and what info does the device needs to store during this procedure.

Proposal 4: As far how the device determines the end of a service request, further study the following solutions.

- A timer for devices. The length of timer could be predefined in spec or configured by reader in the R2D message.

- Explicitly indicated by the reader with an “end” indication in the last A-IoT paging message within the service.

Proposal 4a: FFS the detailed device behaviors and what info the device needs to store temporarily during this procedure.

2.3	 Contents of A-IoT paging message

1) Configuration used to determine access resource

In RAN2#126 [1], the following agreement was reached.

2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

In “A-IoT Random Access” part, CBRA and CFRA mechanisms are still under discussing. From our perspective, it is complex and has no obvious benefit to configure dedicate resource for multiple devices in the A-IoT paging message, because CFRA should be only used for single device case and CBRA should be used for multiple devices case.

Proposal 5: Not support CFRA for multiple devices unless there is clear conclusion in SA2 that multiple IDs of A-IoT devices can be included in one service request from CN.

In the following, the discussion is expanded on cases of CFRA for single device and CBRA for multiple devices.

- In case of CBRA for multiple devices: Considering different services may have different number of target devices, the reader can assign appropriate access occasions for devices to avoid serious collisions or waste of resource. Hence, the resource is better to be explicit configured instead of preconfigured statically. The configured resource is shared among target A-IoT devices.

- In case of CFRA for single device: Both implicit configuration and explicit configuration can work. If the implicit configuration is implemented, the target device can use the first access occasion directly. If the explicit configuration is implemented, the reader can configure only one access occasion for this device. We have a little preference on the latter option for it has aligned format with CBRA

Proposal 6: In case of CBRA for multiple devices (i.e., a group ID or no ID in paging) and CFRA for a single device, the configuration used to determine access resource is explicitly configured in the A-IoT paging message. For CBRA, the configured resource is shared among target devices.

2) Identity of target A-IoT device(s)

For the case of paging single device and group devices, if the device ID/ group ID is contained in the A-IoT paging message in format of AS IE, there may be security concerns. One way to solve this issue is to contain the device ID/ group ID in upper layer packet, e.g., NAS data. The definition of upper layer ultimately depends on SA2.

Observation 2: If the device ID/ group ID contained in A-IoT paging message is implemented as AS layer IE, there may be security concerns.

Proposal 7: The device ID/ group ID in A-IoT paging message is contained in upper layer packet (e.g. NAS container) considering the key issue in SA3.

3) Other contents in A-IoT paging message

Apart from the above mentioned elements, there is also discussion on including other information in A-IoT paging message, e.g., indication of service type (including inventory, command-only and inventory+command) and reader ID. At present, these discussions seem not critical for the basis function of A-IoT paging message. Hence, we propose to discuss these issues in WI phase if needed.

Proposal 8: Whether additional information is contained in A-IoT paging message can be discussed in WI phase, e.g., service type, reader ID.

4	Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Monitoring of A-IoT paging and energy harvesting impacts

Observation 1: RAN1 is discussing energy harvesting related issues, including device state and whether the reader knows and could control the device state, but there has not been much progress so far.

Proposal 1: Postpone the discussion on monitoring A-IoT paging message and device unavailability due to energy harvesting until RAN1 has clear progress.

Multiple/subsequent A-IoT paging

Proposal 2: Introduce a service ID in the A-IoT paging message, identifying this service request from CN, i.e., the A-IoT paging messages associated with the same service ID identify the same service request from CN.

Proposal 3: Once the device has responded the paging message with a service ID, it does not respond the subsequent A-IoT paging messages with the same service ID.

Proposal 4: As far how the device determines the end of a service request, further study the following solutions.

- A timer for devices. The length of timer could be predefined in spec or configured by reader in the R2D message.

- Explicitly indicated by the reader with an “end” indication in the last A-IoT paging message within the service.

Proposal 4a: FFS the detailed device behaviors and what info the device needs to store temporarily during this procedure.

Contents of A-IoT paging message

Proposal 5: Not support CFRA for multiple devices unless there is clear conclusion in SA2 that multiple IDs of A-IoT devices can be included in one service request from CN.

Proposal 6: In case of CBRA for multiple devices (i.e., a group ID or no ID in paging) and CFRA for a single device, the configuration used to determine access resource is explicitly configured in the A-IoT paging message. For CBRA, the configured resource is shared among target devices.

Observation 2: If the device ID/ group ID contained in A-IoT paging message is implemented as AS layer IE, there may be security concerns.

Proposal 7: The device ID/ group ID in A-IoT paging message is contained in upper layer packet (e.g. NAS container) considering the key issue in SA3.

Proposal 8: Whether additional information is contained in A-IoT paging message can be discussed in WI phase, e.g., service type, reader ID.

5	Reference

- Report of 3GPP TSG RAN WG2 meeting #127.

- R1-2407490	FL summary #4 on frame structure and timing aspects for Rel-19 Ambient IoT.

- Chair notes RAN1#118 eom1.