3GPP TSG-RAN WG2 Meeting #127bis                                                   R2-2407950

Hefei, China, 14th – 18th Oct, 2024

Source:              CATT

Title:	             Random Access in SBFD symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, the random access in SBFD symbols was initially discussed, and reached the following agreements:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

In this contribution, the following issues will be further discussed:

- Whether the working assumption on supported RACH triggers can be confirmed?

- Whether/how early identification is used during a SBFD RA procedure?

- How to perform the RO type selection for initial RACH transmission?

- How to perform the RO type selection for RACH re-attempt?

- Whether 2-step RA on SBFD symbols needs to be supported?

Discussion

Whether the working assumption on supported RACH triggers can be confirmed?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

In[1], all the RACH triggers are listed as below:

### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

From RAN2 perspective, allowing SBFD-aware to use RACH resources in SBFD symbol can reduce the RACH latency and improve RACH capacity. There is no motivation to prohibit SBFD-aware UE to use RACH resource on SBFD symbol for any of the above RACH trigger events. Hence, the above working assumption reached in RAN2#127 meeting can be confirmed.

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Whether/how early identification is used during a SBFD RA procedure?

In RAN1#118 meeting, in order to facilitate the RAN1 discussion, additional RO was defined in order to differ the legacy RO. The corresponding agreement is as below:

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

It is obvious that if additional RO is selected by SBFD-aware UE, early identification in Msg1 can be supported.

Proposal 2: If additional RO is selected by SBFD-aware UE, early identification in Msg1 is supported.

Hence, in this section, the discussion mainly focuses on whether early/how identification is used when SBFD-aware UE selected a legacy RO (legacy RO refers to the RO which legacy UE can use). The corresponding analysis is as below:

- Advantages

If early identification in Msg1 is supported, the network can identify the UE is an SBFD-aware UE based on Msg 1 and can allocate the resource for Msg3 transmission in SBFD symbol. The main benefit is that it can reduce the RA latency.

- Disadvantages

In Rel-17 RACH indication and partitioning, in order to support early identification in Msg1, feature/feature combinations was introduced. And the following solutions can be applied for early identification of feature/feature combinations:

  - Solution 1: Shared RACH resources, preamble partitioning is defined based on feature/feature combination.

  - Solution 2: Separate RACH resources (additional RACH) for feature/feature combination.

For Alt1-1 of Option 1, shared RACH resources are used by SBFD-aware UEs and legacy UEs. Hence, only Solution 1 can be adopted and preamble partitioning is necessary if early identification is needed.  There are two disadvantages:

- Preamble partitioning is needed for SBFD feature/feature combination at least in legacy ROs. In order to reduce the impact to legacy UE, some CFRA preambles should be re-allocated to SBFD feature/feature combinations. It is doubtable whether there is enough CFRA preambles can be re-allocated.

- According to the current spec, preamble partitioning is the same for one RACH configuration. If preamble partitioning is used in legacy ROs, preamble partitioning is also used in additional ROs, which will result in resource waste in SBFD symbols.

For Option 2, in RAN2#118, only Alt2-3 was agreed. Additional ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. But there is no agreement on whether SBFD-aware UE can select the legacy ROs. If legacy ROs can be selected, the same preamble partitioning issues as analyzed in bullet 1 of Alt1-1 of Option 1 also exists.

Based on the above analysis, if legacy RO is selected, the benefit of introducing early identification is limited, but it will bring impact on legacy preamble allocation and resource efficiency especially for Alt1-1 of Option1. Hence, it is suggested not to support early identification in Msg1 if legacy RO is selected by SBFD-aware UE. In this case, UE can indicate it support SBFD supports in Msg3.

Proposal 3: If legacy RO is selected by SBFD-aware UE, early identification in Msg1 is not supported.

Proposal 4: In case of early identification in Msg1 is not supported, UE can indicate it supports SBFD in Msg3.

Based on Proposal 3, assuming early identification in Msg1 is not supported if legacy RO is selected, it is unnecessary to treat SBFD as a feature.

Proposal 5: SBFD is not introduced as a feature (similar as RedCap, SDT and etc.) for random access.

RO type selection during initial RACH transmission

In legacy release, the RA resource selection procedure is as below:

Figure-1 Legacy RA resource selection procedure for initial RACH transmission

With the introduction of SBFD, according to the agreements reached in RAN1#118 meeting, a new RO type selection step was introduced for initial RACH transmission:

For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Based on the above agreements, no matter which the above Option is selected during RACH initial transmission, a new step, e.g., RO type selection should be added compared to the legacy RA resource selection procedure. This will impact the RAN2 RACH resource procedure.

Proposal 6: For RACH initial transmission, RO type selection should be captured in MAC.

If Proposal 6 is agreed, RAN2 should further discuss when to perform the RO type selection. In our understanding, the RO type selection can be performed after the RA resource set selection and before 2-step/4-step selection.

After the RA resource set selection, only one set of RA resource is selected. There are three cases:

- If the selected RA resource set only includes valid legacy ROs, legacy RO can be selected;

- If the selected RA resource set only includes valid additional ROs, additional RO can be selected;

- If the selected RA resource set includes both valid legacy ROs and valid additional ROs, RO type selection can be performed.

Proposal 7: For RACH initial transmission, RO type selection can be performed after RA resources set selection and before 2/4 step RA selection.

For SBFD-aware UE, if RO type selection is performed, it had better prioritize to select the additional RO which can facilitate the early identification and improve the RACH capacity.

Proposal 8: For RACH initial transmission of SBFD-aware UE, it should prioritize to select the additional RO over the legacy RO during initial RACH transmission.

RO type selection during RACH re-attempt

With the introduction of SBFD, according to the agreements reached in RAN1#118 meeting, the solutions of RO type selection during RACH re-attempt were listed below:

For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

For Option 3, if independently select legacy RO and additional RO for each RACH re-attempt, considering the power control on SBFD symbol and non-SBFD symbol should be separated, it is more complex than Option 1 and Option 2. But the gain is not obvious.

Compared with Option 2, fallback is supported for Option 2. Fallback can help to increase the RA successful probability. Hence, Option 2 is slightly preferred.

Proposal 9: For SBFD-aware UE, when performing RACH re-attempt, it first uses the ROs of the same type as the earlier PRACH transmission for a certain number of times, and if RA still fails, it can use the other type for the rest RACH re-attempts.

Whether 2-step RA on SBFD symbols needs to be supported?

In Rel-16, 2-step RA was introduced to reduce RA latency. When SBFD is introduced in Rel-19, it should further discuss whether 2-step RA on SBFD symbols needs be supported. Regarding to this issue, in RAN1#116 meeting and in RAN2#127 meeting, the following agreements were reached:

Agreements in RAN1#116 meeting: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Agreements in RAN2#127 meeting: RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

If 2-step RA needs to be supported, the main specification efforts lie in RAN1, e.g., how to configure the MsgA PUSCH, the power control for MsgA PUSCH and etc. Hence, RAN2 had better wait for RAN1 conclusion before further discussion on 2-step RA in RAN2.

Proposal 10: Regarding to whether 2-step RA needs to be supported, RAN2 can wait for RAN1 progress.

Conclusion

According to the analysis in section 2, it is proposed:

supported RACH triggers

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

early identification

Proposal 2: If additional RO is selected by SBFD-aware UE, early identification in Msg1 is supported.

Proposal 3: If legacy RO is selected by SBFD-aware UE, early identification in Msg1 is not supported.

Proposal 4: In case of early identification in Msg1 is not supported, UE can indicate it supports SBFD in Msg3.

Proposal 5: SBFD is not introduced as a feature (similar as RedCap, SDT and etc.) for random access.

RO type selection

Proposal 6: For RACH initial transmission, RO type selection should be captured in MAC.

Proposal 7: For RACH initial transmission, RO type selection can be performed after RA resources set selection and before 2/4 step RA selection.

Proposal 8: For RACH initial transmission of SBFD-aware UE, it should prioritize to select the additional RO over the legacy RO during initial RACH transmission.

Proposal 9: For SBFD-aware UE, when performing RACH re-attempt, it first uses the ROs of the same type as the earlier PRACH transmission for a certain number of times, and if RA still fails, it can use the other type for the rest RACH re-attempts.

2-step RA

Proposal 10: Regarding to whether 2-step RA needs to be supported, RAN2 can wait for RAN1 progress.

Reference

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)

- R1-2401676	  Summary#1 on SBFD random access operation Moderator (CMCC)