3GPP TSG-RAN WG2 Meeting #127bis	R2-2407960

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

In RAN2#127, the following agreements were achieved [1] with several leftover issues.

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

RAN1 sent reply LS in [2] back to RAN2 with the following answers:

RAN1 thanks RAN2 for their questions on the DL coverage enhancements and would like to provide the responses to the following questions. RAN1 will try to provide a further response at a later time. Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?. Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion. Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.. Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

This contribution will discuss several leftover issues of this feature.

Discussion

Behaviour for Rel-19 UEs not supporting DL coverage enhancement

In RAN2#127, whether the legacy UE should be barred was discussed and it was agreed that if there is a need to bar pre-Rel-19 NTN UEs from accessing a cell operating with DL coverage enhancement, the existing NTN bar bit can be used. On the DL coverage enhancement feature, Rel-19 UE not supporting DL coverage enhancement does not have difference comparing with pre-Rel-19 NTN UE, i.e. they cannot operate with the extended SSB periodicity either. Hence, it is proposed that the Rel-19 UEs not supporting DL coverage enhancement will consider the cell as barred for NTN access when the existing NTN bar bit is set as barred.

Proposal 1: The Rel-19 UEs not supporting DL coverage enhancement will consider the cell as barred for NTN access, when the existing NTN barring bit is set as barred (due to the cell operating with extended SSB periodicity).

SMTC/gap impacts due to beam hopping/ larger SSB periodicity

According to current spec, 4 SMTC windows for one frequency can be configured for NTN deployments [3]. In last meeting, there were proposals to further extend the SMTC number of one frequency layer for DL coverage enhancement scenario, w.r.t. the following FFS point from last meeting [1]:

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

The main argument was that a cell has 6 neighbour cells typically, and one SMTC is needed for one neighbour cell because the offsets of neighbour cells' MTC may be different from each other, along with the beam hopping.

However, if the scenario that one frequency serves 7 adjacent footprints exists, this issue can be solved by NW implementation. For example, the NW provides the SSB of more than one cells that are served by different footprints which are not adjacent at the same time; then, from the SMTC configuration perspective, these neighbour cells have the aligned SMTC offset. An example is given in the following figure: for the SMTC with the offset being 5 ms, three beams provide service for the three cells highlight with blue at the same time; for the SMTC with the offset being 25 ms, three beams provide service for the three cells highlight with pink at the same time. In this instance, for the UE within the cell in the middle, the SSBs of its neighbour cells can be covered by two SMTC windows with offset 5 ms and 25 ms separately.

Even if these neighbour cells belong to different satellites, the offsets are not coordinated among satellites, 4 SMTC windows for one frequency seem sufficient as well. If the SMTC number and duration do not need to be extended, the gap configuration does not need to be enhanced, either.

Observation 1: For DL coverage enhancement scenario, it is up to NW implementation to ensure up to 4 SMTCs and measurement gaps are sufficient, e.g. more than one neighbour cells providing SSB at the same time within the same SMTC window.

Proposal 2: Do not extend the number and duration of SMTC/measurement gap due to DL coverage enhancement feature.

Cell level or beam level DRX/DTX

In previous meetings, companies proposed to introduce DRX/DTX like mechanism to indicate the activated time of a cell/beam. However, according to RAN1’s answer on Q4, the beam footprint status in terms of "off", "common messages only" and "active traffic” is only defined for evaluation. The beam footprint could be physical beam not visible to UE, which RAN2 should not use for any the RAN2 cell/beam level DRX/DTX mechanism design. We propose to postpone the discussion on cell level or beam level DTX/DRX until receiving RAN1 reply on SSB periodicity and beam status design for the UE-visible beams (i.e. NR beams).

Proposal 3: Postpone the discussion on cell level or beam level DTX/DRX until receiving further RAN1 reply on SSB periodicity and beam status design for UE-visible beams.

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: For DL coverage enhancement scenario, it is up to NW implementation to ensure up to 4 SMTCs and measurement gaps are sufficient, e.g. more than one neighbour cells providing SSB at the same time within the same SMTC window.

Proposal 1: The Rel-19 UEs not supporting DL coverage enhancement will consider the cell as barred for NTN access, when the existing NTN barring bit is set as barred (due to the cell operating with extended SSB periodicity).

Proposal 2: Do not extend the number and duration of SMTC/measurement gap due to DL coverage enhancement feature.

Proposal 3: Postpone the discussion on cell level or beam level DTX/DRX until receiving further RAN1 reply on SSB periodicity and beam status design for UE-visible beams.

Reference

- R2-2407573	Report from Break-out session on NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

- R1-2407538	Reply LS on DL coverage enhancements.

- 3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".