3GPP TSG-RAN WG2 Meeting #127bis	R2-2407962

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Further discussion on regenerative payload

Agenda Item:	8.8.5

Document for:	Discussion and Decision

Introduction

For regenerative payload, the following agreements were made in RAN2#127 meeting: [1]

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

In this document, we discuss the remaining issue and our observations/proposals are also provided.

Discussion

In RAN2#127 meeting, one issue was left for further discussion: [1]

R2-2406636 Discussion on regenerative payload Qualcomm Incorporated discussion Rel-19 NR_NTN_Ph3-Core Proposal 4 After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message. - Apple wonders if this is a NW implementation issue - Come back in the next meeting

The issue is that the network may send the early contention resolution MAC CE (i.e., without RRC message) while still waiting for the response from the ground network. If the UE keeps monitoring the PDCCH after contention resolution reception, there may be unnecessary power consumption cost before it can really receive further scheduling from the NW. This issue mainly happens for the following RACH triggered events which involve RRC processing:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	SDT in RRC_INACTIVE;

-	RRC Connection Resume procedure from RRC_INACTIVE.

  - Initial access from RRC_IDLE

For Initial access from RRC_IDLE case, the serving gNB on-board can send Msg4 together with RRCSetup message to indicate the UE to enter RRC_CONNECTED mode, without waiting for the response from the ground network. It can also send subsequent PDCCH following Msg4 in order to schedule RRCSetupComplete message. Hence, there is no unnecessary PDCCH monitoring and the corresponding power consumption in this case.

Observation 1: For initial access from RRC_IDLE, RRCSetup can be sent together with Msg4 and the UE monitors PDCCH after Msg4 to receive scheduling for RRCSetupComplete, so that there is no unnecessary PDCCH monitoring and power consumption cost.

  - RRC Connection Re-establishment procedure

For RRC Connection Re-establishment procedure, if the gNB receiving RRCReestablishmentRequest has the UE context, there is no need for negotiation with other gNBs. Then, the RRCReestablishment message can be sent together with Msg4. Otherwise, the network can send Msg4 without RRCReestablishment or send Msg4 with RRCSetup. However, even if early contention resolution is completed, the latency for negotiation of the current gNB with other gNB is not clear. Therefore, it is difficult to estimate how long UE's PDCCH monitoring can be delayed to realize the power consumption reduction.

Observation 2: It is difficult to estimate the time between reception of Msg4 and RRCRestablishment, considering that whether RRCReestablishment can be sent together with Msg4 depends on whether the selected cell during RRC reestablishment procedure needs to fetch the UE context from other gNBs.

  - SDT in RRC_INACTIVE

For SDT, it has been agreed in last meeting to do no specific optimizations as follows: [1]

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

Therefore, we will not pursue this issue specific for SDT.

Observation 3: It has been agreed that there is no enhancements for power saving specific to SDT.

  - RRC Connection Resume procedure from RRC_INACTIVE

For the case of RRC Connection Resume procedure from RRC_INACTIVE, it is as complicated as to SDT/RRC-Restablishment cases discussed above. Basically, when the receiving gNB receives RRCResumeRequest, it may make the decision on which message can be sent to the UE. Generally,

        - if the receiving gNB stores the UE context or can fetch the UE context from the last serving gNB, it may send RRCResume;

        - if the receiving gNB does not store the UE context and cannot fetch the UE context from the last serving gNB, it may signal RRCSetup to the UE and indicate it to perform RRC setup procedure.

Actually, the serving gNB can judge efficiently whether the UE context can be restored or not. Hence, the RRCSetup message can be sent to the UE via Msg4 corresponding to Msg3 which contains RRCResumeRequest. Considering the that RRCSetup case has been analyzed above, we focus on the RRCResume branch.

For this branch, it is difficult to estimate the duration between RRCResumeRequest reception and time to send RRCResume. If the receiving gNB stores the UE context, we can assume it does not include the duration to retrieve the UE context. If the receiving gNB needs to fetch the UE context from other gNBs, it may consume hundreds of milliseconds. And it has been agreed in RAN3#125 meeting that: [2]

Inactive UE support [Agreement] RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Therefore, the Xn topology highly depends on deployment and there may be multi-hops between the receiving gNB and last serving gNB. It also brings difficulties on the estimation on the duration. Hence, the simple solution is that the receiving gNB send Contention Resolution MAC CE in Msg4 and the UE monitors PDCCH for RRCResume.

Observation 4: For RRC Connection Resume procedure from RRC_INACTIVE, it is difficult to estimate the duration between RRCResumeRequest reception and time sending RRCResume, due to the UE context availability and retrieving issue.

Considering the analysis above, we propose that RAN2 does not pursue further optimization on power saving for Msg4 (if possible at all) in regenerative payload architecture.

Proposal: Do not optimize UE power saving for Msg4 in regenerative payload architecture.

- 3.	Conclusion

In this document, we analyse the remaining issue for regenerative mode of gNB on board, and find the following observations and proposal:

Observation 1: For initial access from RRC_IDLE, RRCSetup can be sent together with Msg4 and the UE monitors PDCCH after Msg4 to receive scheduling for RRCSetupComplete, so that there is no unnecessary PDCCH monitoring and power consumption cost.

Observation 2: It is difficult to estimate the time between reception of Msg4 and RRCRestablishment, considering that whether RRCReestablishment can be sent together with Msg4 depends on whether the selected cell during RRC reestablishment procedure needs to fetch the UE context from other gNBs.

Observation 3: It has been agreed that there is no enhancements for power saving specific to SDT.

Observation 4: For RRC Connection Resume procedure from RRC_INACTIVE, it is difficult to estimate the duration between RRCResumeRequest reception and time sending RRCResume, due to the UE context availability and retrieving issue.

Proposal: Do not optimize UE power saving for Msg4 in regenerative payload architecture.

- 4.	Reference

- R2-2407573	Report from Break-out session on NR-NTN and IoT-NTN 	Session Chair (ZTE Corporation);

- (draft) RAN3 #125 Meeting Report