3GPP TSG-RAN WG2 Meeting #127bis	R2-2407964

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item:	8.8.1

Source: 	CATT

Title: 	Open issue list and rapporteur's input for LTE_TN_NR_NTN_mob WI

Document for: 	Discussion and Decision

Introduction

This document provides the list of remaining open issues from last meeting [1], and the corresponding input and proposal from rapporteur point of view.

R2-2407812 Summary of [AT127][305][LTE_TN_NR_NTN_mob] RRC CR CATT disc Rel-19 LTE_TN_NR_NTN_mob Proposal 1: RAN2 agrees signaling Alternative 1 to indicate ephemeris data for a satellite supporting both NR NTN and IoT NTN, i.e. introducing a Satellite reference ID that refers to the ephemeris data of an IoT NTN satellite in the existing list neighSatelliteInfoList-r18, and using the "CHOICE" signaling structure including this Satellite reference ID and explicit ephemeris data configuration as the two choices. Come back in the next meeting Proposal 2: RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN. Come back in the next meeting

Another stage-3 issue identified by companies before the meeting was also discussed with proposal from rapporteur's perspective, w.r.t. configuration of dedicated NR NTN frequency in the RRCConnectionRelease message.

Open issue list and rapporteur's input

Remaining open issues from last meeting

- [Issue 1]	Signalling design to avoid ephemeris duplication for the same satellite providing both IoT NTN and NR NTN cells

Rapporteur's remarks and proposals

At last meeting, two signalling alternatives were provided in [2] (as shown in Appendix 1). Whereas Alt.A was proposed per majority's views last meeting, there were comparatively strong concerns to Alt.A during online discussion, with the argument that Alt.B is more signalling efficient. Based on some offline discussion before the meeting, it seems acceptable to companies to turn to Alt.B due to its signalling efficiency. Therefore, the Alt.B is preferentially proposed as the option to be agreed by RAN2, and is now implemented in the updated endorsed CR in [3]. An example on how Alt.B works is also shown by the figure in Appendix 2.

Proposal 1: RAN2 agrees the following signalling design to avoid ephemeris duplication for the same satellite providing both IoT NTN and NR NTN:

- The satelliteId-r19 in an entry of NR NTN assistance info list (i.e. neighSatelliteInfoListNR) can be set equal to a Satellite ID value included in IoT NTN assistance info list (i.e. neighSatelliteInfoList) and thus refers to the ephemeris data of IoT NTN identified by this specific Satellite ID, in which case the ephemeris data (i.e. ephemerisInfo-r19) in that entry of neighSatelliteInfoListNR can be absent.

Proposal 1a: If Proposal 1 is agreeable, RAN2 agrees the TP for Signalling Alt.B in the Appendix.

During the pre-meeting offline discussion on Alt.B, some companies raised issues on how the Satellite ID should be assigned, after introducing the NR NTN satellites assistance information in SIB33 in addition to the legacy IoT NTN assistance info list (e.g. whether a satelliteId should be unique across the two lists). Since RAN2 agreed to reuse IE SatelliteId-r18 also for NR NTN and this IE, according to the IE description, is used to identify the satellite assistance information, rapportuer understands that what really matters is how the value of satelliteId should be set by the network across these two NTN satellite assistance information lists, by taking into account the satellites serving IoT NTN only, serving NR NTN only and serving both.

Signalling Alt.B already allows two entries in neighSatelliteInfoListNR and neighSatelliteInfoList respectively to share the same Satellite ID value, in case they are served by the same satellite (i.e. having the same ephemeris) - this is already sufficiently clarified in the Proposal 1. Some other requirements may be as follows:

- Two different entries within the same satellite assistance info list (i.e. neighSatelliteInfoListNR or neighSatelliteInfoList) cannot be assigned the Satellite ID value (otherwise, UE cannot differentiate which entry this ID actually identifies);

- Two entries respectively in neighSatelliteInfoListNR and neighSatelliteInfoList with different ephemeris (i.e. two different satellites) are not expected to be assigned the same Satellite ID value.

If necessary, RAN2 may discusses whether anything needs to be clarified in the specification regarding the satellite ID assignment across the two assistance information list or we can leave this fully up to NW implementation.

Proposal 1b: Regarding satellite ID assignment across IoT NTN and NR NTN satellite assistance info lists, RAN2 discusses whether it can be left to NW implementation to handle, or any clarification is needed in the specification (e.g. preventing the same satellite ID being assigned to two different entries in the same list or two entries with different ephemeris across different lists).

- [Issue 2]	Whether to avoid duplication of other information than ephemeris data in the case of same satellite serving both NR NTN and IoT NTN

Rapporteur's remark and proposal

Since the main reason to avoid ephemeris duplication is that ephemeris data is too big in size. Other NR NTN configurations (e.g. Kmac, validity duration, etc.) are much smaller, and common TA related parameters of NR NTN are unlikely to be absent (due to the different steps compared with IoT NTN as agreed earlier). Rapporteur thus proposes to stick to previous RAN2 agreement, and consider avoiding only ephemeris duplication for satellites supporting both RATs.

Proposal 2: RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN.

Other Stage-3 issues

Some offline comments were received by the rapporteur that the dedicated NR NTN frequency should not be provided by dedicated signalling, i.e. RRCConnectionRelease message, because RAN2 did not agree to introduce a UE capability for this IDLE mode LTE TN to NR NTN mobility feature. Whereas this technical understanding is straightforward and would be easily confirmed by RAN2, there were further comments from companies that this needs to be clarified in the Spec as some forms of restriction to NW configuration (in order to avoid troubles for UE). Rapporteur thinks that this can be fully left to NW implementation without need to specify any restriction for the NW in this regards, because it is explicitly specified in TS 36.306 "E-UTRAN needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.", meaning that NW is unlikely to configure anything not indicated as supported by the UE via reasonable NW implementation.

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact).

Conclusion

The open issues for LTE_TN_NR_NTN_mob WI and rapporteur's proposals in Section 2 are listed as follows. RAN2 is suggested to conclude these remaining open issues, by taking into account Rapporteur's proposals.

[Issue 1]	Signalling design to avoid ephemeris duplication for the same satellite providing both IoT NTN and NR NTN cells

Proposal 1: RAN2 agrees the following signalling design to avoid ephemeris duplication for the same satellite providing both IoT NTN and NR NTN:

- The satelliteId-r19 in an entry of NR NTN assistance info list (i.e. neighSatelliteInfoListNR) can be set equal to a Satellite ID value included in IoT NTN assistance info list (i.e. neighSatelliteInfoList) and thus refers to the ephemeris data of IoT NTN identified by this specific Satellite ID, in which case the ephemeris data (i.e. ephemerisInfo-r19) in that entry of neighSatelliteInfoListNR can be absent.

Proposal 1a: If Proposal 1 is agreeable, RAN2 agrees the TP for Signalling Alt.B in the Appendix.

(Below discussion on satellite ID assignment only after P1/1a concluded)

Proposal 1b: Regarding satellite ID assignment across IoT NTN and NR NTN satellite assistance info lists, RAN2 discusses whether it can be left to NW implementation to handle, or any clarification is needed in the specification (e.g. preventing the same satellite ID being assigned to two different entries in the same list or two entries with different ephemeris across different lists).

[Issue 2]	Whether to avoid duplication of other information than ephemeris data in the case of same satellite serving both NR NTN and IoT NTN

Proposal 2: RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN.

[Other Stage-3 issues]

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact).

Reference

- R2-2407573	Report from Break-out session on NR-NTN and IoT-NTN 	Session Chair (ZTE Corporation)

- [AT127][305][LTE_TN_NR_NTN_mob]

- R2-2407963	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT

- Appendix 1: Signalling Alt.A and Alt.B

========== Alt. A ==========

NeighSatelliteInfoNR-r19::= SEQUENCE { satelliteId-r19 SatelliteId-r18, ephemerisInfo-r19 CHOICE { ephemerisInfoNR-r19 CHOICE { stateVectors-r19 EphemerisStateVectors-r17, orbitalParameters-r19 EphemerisOrbitalParameters-r17 }, satelliteRefId SatelliteId-r18 }, nta-CommonParametersNR-r19 SEQUENCE { nta-CommonNR-r19 INTEGER (0.. 66485757) OPTIONAL, -- Need OP nta-CommonDriftNR-r19 INTEGER (-257303..257303) OPTIONAL, -- Need OP nta-CommonDriftVariationNR-r19 INTEGER (0..28949) OPTIONAL -- Need OP }, epochTime-r19 SEQUENCE { startSFN-r19 INTEGER (0..1023), startSubFrame-r19 INTEGER (0..9) } OPTIONAL, -- Need OP k-Mac-r19 INTEGER (1..512) OPTIONAL, -- Need OP ntn-PolarizationDL-r19 ENUMERATED {rhcp,lhcp,linear} OPTIONAL -- Need OP } -- ASN1STOP SystemInformationBlockType33 field descriptions. ephemerisInfo Ephemeris data for a neighbour NR NTN satellites. If this field is set to satelliteRefId, UE uses the ephemerisInfo identified by that satellite ID that matches satelliteRefId in the neighSatelliteInfoList..

========== Alt. B ==========

NeighSatelliteInfoNR-r19::= SEQUENCE { satelliteId-r19 SatelliteId-r18, ephemerisInfo-r19 CHOICE { stateVectors-r19 EphemerisStateVectors-r17, orbitalParameters-r19 EphemerisOrbitalParameters-r17 } OPTIONAL, -- Need OP nta-CommonParametersNR-r19 SEQUENCE { nta-CommonNR-r19 INTEGER (0.. 66485757) OPTIONAL, -- Need OP nta-CommonDriftNR-r19 INTEGER (-257303..257303) OPTIONAL, -- Need OP nta-CommonDriftVariationNR-r19 INTEGER (0..28949) OPTIONAL -- Need OP }, epochTime-r19 SEQUENCE { startSFN-r19 INTEGER (0..1023), startSubFrame-r19 INTEGER (0..9) } OPTIONAL, -- Need OP k-Mac-r19 INTEGER (1..512) OPTIONAL, -- Need OP ntn-PolarizationDL-r19 ENUMERATED {rhcp,lhcp,linear} OPTIONAL -- Need OR } -- ASN1STOP SystemInformationBlockType33 field descriptions. ephemerisInfo Ephemeris data for a neighbour NR NTN satellites (if included in NeighSatelliteInfoNR). This field is mandatory present in NeighSatelliteInfoNR, if the satelliteId in the same entry of neighSatelliteInfoListNR does not match any satellite ID values included in neighSatelliteInfoList. If this field is absent in NeighSatelliteInfoNR and the satelliteId in the same entry of neighSatelliteInfoListNR equals a satellite ID value included in neighSatelliteInfoList, UE uses the ephemerisInfo identified by that satellite ID in the neighSatelliteInfoList..

- Appendix 2: An example for signalling Alt.B

In above figure, Satellite ID = 2 can be included in both neighSatelliteInfoList and neighSatelliteInfoListNR, and the entry in neighSatelliteInfoListNR with Satellite ID =2 can include no ephemeris data (but refers to that identified by Satellite ID=2 in neighSatelliteInfoList).