3GPP TSG-RAN WG2 Meeting #127bis	R2-2407965

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Further consideration on UL capacity enhancements

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting [1], agreements were made for UL capacity enhancements as follows:

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In this document, we further discuss the RAN2 objective of UL capacity enhancements in IoT NTN, with our observations and proposals provided.

Discussion

TA

In RAN2#126 meeting, RAN2 has sent LS to RAN1 and RAN4, asking the feasibility on TA issue. And RAN1/4 have replied to the LS in [2]-[3]. Based on the LS reply, it can be concluded that at least for the NB-IoT with 3.75KHz SCS and eMTC CE mode A, an RRC IDLE UE with a pre-compensated TA supported to be used for Msg1 transmission during random access for IoT NTN can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2, assuming NTA=0 for Msg3 transmission without Msg1/Msg2. From RAN2 perspective, the discussions relying on TA validity issues can be resumed, and RAN2 can confirm that it is feasible to rely on the UE pre-compensated TA to support the CB-Msg3 mechanism.

Proposal 0: RAN2 confirms that it is feasible to rely on the UE pre-compensated TA to support CB-Msg3 transmission.

DSA

In the previous RAN2 meeting, there was one issue raised concerning the feasibility to introduce this CB-Msg3 transmission, i.e. how gNB handles the transmission collisions by different UEs on the same PUSCH resource. And it was agreed in RAN2#127 meeting that RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts. According to the guidance agreed in RAN2#127 meeting, we continue our discussion on DSA.

Based on the reference [4], the DSA is defined as the procedure that:

Whenever a user generates a packet, it transmits k copied of the same packet. It is assumed that there exists some arrangement which allows a receiver to reject all but one correctly received copy of any packet. The multiple copied can be transmitted either simultaneously or different frequency channels, which we can frequency diversity, or they may be transmitted on a single high-speed channel but spaced apart by random time intervals; we call this time diversity.

At present, the detailed resource allocation mechanism for DSA, when incorporated into the 3GPP network, is still unclear and requires further study. The basic concept is to rely on different resource patterns to avoid subsequent collisions on the repetition/retransmissions by different UEs. So conceptually, it is a frequency/time-domain division method to handle the collisions.

Actually a similar way was specified in LTE sidelink (mode-4) and NR sidelink (mode-2). Take the random selection based sidelink resource allocation as an example: the UE can randomly select PSSCH resources, and use the randomized resources selected and retransmission opportunities to avoid collisions with other UEs. According to current specification, the PUSCH used in RRC_IDLE mode can be configured via PUR-Config in RRCConnectionRelease(-NB). But it may not be suitably used for CB-Msg3 transmission via DSA mechanism directly.

Therefore, we need to discuss how the DSA should work under the IoT NTN framework from the resource allocation perspective. We analyze the spec impacts based on the following two aspects: resource configuration and UE resource selection.

### 2.2.1	Resource configuration

- PUSCH/NPUSCH

The network should configure the resources for PUSCH/NPUSCH for UEs, based on which the UE select resources and apply the related parameters to perform CB-Msg3 transmissions. The configuration may include (but not be limited to) the following aspects:

- Resource in frequency domain:

e.g. RB allocation or carrier configuration. These parameters are used to define the frequency range of the PUSCH/NPUSCH resource.

- Resource in time domain:

e.g. SFN, subframe and periodicity and offset. These parameters are used to define the time pattern of the PUSCH/NPUSCH resource.

- Power control related parameters:

e.g., p0-UE-PUSCH and alpha.

- Cell specific PUSCH/NPUSCH resources are associated with number of repetitions :

Specially, many companies in last meeting support cell specific PUSCH/NPUSCH resources are associated with number of repetitions for CB-Msg3 [5]-[8]. If this is supported, the association between different repetition number and resource allocation should be studied.

- MPDCCH/NPDCCH

It may happen that network may schedule contention resolution corresponding to the CB-Msg3. Hence, it is necessary to configure MPDCCH/NPDCCH to the UE. For example, search space configuration, repetition number and etc.

However, all these aspects regarding resource configurations are out of the RAN2 scope and should be studied in RAN1. Some companies proposed to introduce a pool of PUSCH/NPUSCH resources for UE to select resources [6]. However, per previous experience, the concept/design of such resource pools have always been within RAN1 scope, and it is even unknown whether the physical resource structure as in legacy PUSCH/NPUSCH can be directly applied into such resource pools.

Observation 1: Resource configuration aspects for DSA (e.g. how to design the pool for CB-Msg.3 transmission resources, whether existing resource structure in UL/DL channels can be reused, etc.) need be discussed and designed by RAN1.

### 2.2.2	Resource selection

If we adopt DSA which is a UE autonomous resource allocation mechanism, one straightforward question is whether/how we regulate UE behavior on the selection of resources. In Uu, the data transmission resources are scheduled by the NW, and so the allocation of the resources to different UEs is up to implementation of the scheduler at the NW side, thus unnecessarily specified. But now, since we rely on UE itself to select resources in a contention based way, UE behavior on how to select resources may need be specified, e.g. triggers for resource (re)selection, criteria on which resource to select, etc.

Even if, as what the proponents argued in the last meeting, one intends to rely on the simplest random selection procedure, it still needs evaluation from RAN1 on whether really afford acceptable performance. Also, specified trigger conditions on whether the UE is allowed to (re)select resources still needed to avoid malicious UEs implementation of selecting excessive resources (thus causing collision to others). These also need to be concluded by RAN1, as these are tightly related to the resource configuration design.

Observation 2: UE behaviors on resource (re)selection for DSA (e.g. trigger conditions on when UE is allowed to (re)select resources, criteria on what resources the UE can select, etc.) need be specified, and they need discussion and design by RAN1.

Therefore, if DSA is selected as the transmission scheme, RAN2 should send LS to RAN1 to check the impacts on RAN1 and whether it can be accepted from RAN1 point of view.

Proposal 1: If RAN2 concludes to support DSA, send LS to RAN1 to initiate RAN1 work on the resource configuration and resource selection design.

Potential issues of CB-Msg3

In this section, no matter which CB mechanism is adopted e.g. CBDSA, the following potential issues should be studied.

### 2.3.1	How to provide the CB-Msg3 configuration

One key aspect for CB-Msg3 is how to provide the CB-Msg3 configuration. In Rel-16, PUR was agreed to allow one uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing the random access procedure. In PUR, the UE could enter RRC_CONNECTED and send PUR configuration request, if PUR is enabled in the cell. The PUR configuration request may include information about the preference on requested resource. When the eNB releases the UE to RRC_IDLE, it may configure the PUR resource to the UE based on the PUR configuration request, subscription information and/or local policies. The PUR configuration is sent to the UE via the RRCConnectionRelease(-NB) message. Based on the legacy design, there is the possibility that the CB-Msg3 resources are still provided by dedicated signaling, with just same physical resources signaled to different UEs. By contrast, since now a resource can be shared by different UEs, one may also consider using broadcast signaling to provide CB-Msg3 resources.

With the above kept in mind, we discuss these two candidate solutions in detail as follows:

      - [Option 1]: Dedicated signaling based CB-Msg3 configuration;

In option 1, the network can assign theCB_Msg3 resources and perform the grouping of UE based on each UE's dedicated configuration request, which is similar to legacy way of PUR request and configuration. However, one aspect that may need further consideration is that the per-UE PUR configuration based signaling may only be valid in the cell where the configuration was received as in legacy, and it seems hard for an RRC_IDLE UE to directly resume using this UE-specific PUR configuration after cell reselection to another cell. If this is the case, the UE, after reselecting to a new serving cell, has to enter RRC_CONNECTED state again to request the PUR configuraiotn. Considering that the motivation to introduce CB-Msg3 is to save overhead but the PUR request itself still leads to non-trivial signalling overhead (e.g.signalling overhead for RRC state transition due to cell change), RAN2 may consider whether such signalling overhead resulting from PUR request in the case of cell change is acceptable or not for this option.

Observation 3: RRC_IDLE UE may need to enter RRC_CONNECTED and perform PUR configuration request each time it reselects to another cell, if the PUR configuration is sent to the UE via dedicated signaling. This leads to extra signaling overhead.

      - [Option 2]: Broadcast signaling based CB-Msg3 configuration;

TA is the most key issue for this option. Since RAN1/4 confirms that the validity of directly using UE pre-compensated at least for the NB-IoT with 3.75KHz SCS and eMTC CE mode A, broadcast signalling can be used to configure the resouce for CB-Msg3 at least in these cases.

Observation 4: Broadcast signaling based CB-Msg3 configuration can be applied at least for the NB-IoT with 3.75KHz SCS and eMTC CE mode A.

Compared with dedicated signalling, which brings extra signaling overhead, we slightly prefer broadcast signaling to deliver the configuration for CB-Msg3.

Proposal 2: Broadcast signalling is used to deliver the CB-Msg3 configuration (assuming CB-Msg3 is only supported for the cases of NB-IoT with 3.75KHz SCS and eMTC CE mode A). FFS on whether to restrict the applicable scenarios in the Spec.

### 2.3.2	RNTI calculation

Once transmission is performed, the UE should wait for the feedback from the network, i.e. ACK based on Msg4 reception. This needs the UE to monitor the PDCCH addressed to an RNTI, corresponding to its Msg3 transmission, in order to receive the Msg4 scheduling by the NW. It is still questionable on the RNTI which is used for PDCCH monitoring. In previous meetings, companies provide the following options:

[Option 1]: UE specific RNTI which is pre-configured by the network;

[Option 2]: RNTI which can be calculated by UE and the network according to the resources used by the UE and similar to RA-RNTI;

[Option 3]: common RNTI which is similar to SI-RNTI.

If common RNTI is used, the UE has to decode all the PDSCH scheduled by the PDCCH addressed by the common RNTI during the window until the transmission addressing this UE has been detected successfully. From the UE perspective, it is of low efficiency and causing high power consuming. So, Option 3 can be excluded. For the remaining 2 options, it highly depends on the conclusion of Proposal 2. If option 2 is agreed, i.e. broadcast signaling for CB-Msg3 is agreed, the resource for CB-Msg3 transmission without Msg1/RAR is configured for all the UEs which intend for the uplink transmission, and the network is not aware which UE initiates the transmission if the UE is in RRC IDLE without suspend. With inspiration from RA-RNTI, Option 2 can be adopted. That is, RNTI can be calculated by UE and the network according to the resources used by the UE. . However, this also depends on the detailed physical resource structure in the pool of common resources configured, and thus can only be concluded after the resource configuration design concluded by RAN1.

Proposal 3: For RNTI calculation, RNTI can be calculated by UE and network according to the resources used by the UE (which is similar to RA-RNTI), if broadcast signalling is used to deliver the CB-Msg3 configuration.

### 2.3.3	Efficient delivery of msg4/RRCEarlyDataComplete

How to deliver msg4/RRCEarlyDataComplete mainly depends on the configuration signaling and the contention resolution scheme. In the following, we discuss the solutions assuming Proposal 2 is agreed, i.e., broadcast signalling is used to deliver the CB-Msg3 configuration.

In broadcast signaling based CB-Msg3 mechinism, as in RACH proceudre, the UE could autonomously calcuate an RNTI as discussed in section 2.3.4. When the UE performs the UL transmission, the UE could start one CB-Msg3 contention resolution window. Similar to Msg4 reception, the UE monitors the PDCCH scrambled by the CB-Msg3-RNTI. Then, in order for contenstion resolution, a CB-Msg3 contention resolution MAC CE should be introduced and included in the DL message as a response to the UE. If the CB-Msg3 contention resolution MAC CE is corresponds to the beginning part of the UL message (e.g. first 48 bits), the UE will consider the conention resolution for this CB-Msg3 transmission is successful. If retransmision for UL message is indicated, the UE will perform retransmission of the UL message. If the CB-Msg3 contention resolution window expires, the UE could re-attemp UL transmsision or fallback to RA-EDT.

Proposal 4: For broadcast signaling based CB-Msg3 (if supported), a UE calculated RNTI is introduced and is used to monitor the scheduling of CB-Msg3 response for contention resolution.

Proposal 5: For broadcast signaling based CB-Msg3 (if supported), the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3.

There is a need to handle the exceptional case that the UE may not receive any feedback from the network after the Msg3 transmission. To this end, a window/timer needs to be introduced for Msg4 reception. The UE just starts PDCCH monitoring for Msg4 scheduling during this window, or during this timer is running. If the Msg4 corresponding to its identity is not received within the window, or before the timer expiry, UE considers the CB-Msg3 transmission failure, and perform the failure handling. Further details on the window/timer handling, e.g. when to (re)start the timer/window, and the intended UE behavior after failure detected (e.g. re-attempt for CB-Msg3 or indicating upper layers) can be left to further discussion, but RAN2 needs to confirm the need of such window/timer for CB-Msg3 failure handling.

Proposal 6: A window/timer is introduced for Msg4 reception, and the UE monitors PDCCH during the window/when the timer is running. FFS on the details of window/timer handling and UE behavior when Msg4 reception failure detected.

- 3.	Conclusion

In this document, we analyse the potential impacts for UL capacity enhancements in IoT NTN, and find the following observations and proposals:

Observation 1: Resource configuration aspects for DSA (e.g. how to design the pool for CB-Msg.3 transmission resources, whether existing resource structure in UL/DL channels can be reused, etc.) need be discussed and designed by RAN1.

Observation 2: UE behaviors on resource (re)selection for DSA (e.g. trigger conditions on when UE is allowed to (re)select resources, criteria on what resources the UE can select, etc.) need be specified, and they need discussion and design by RAN1.

Observation 3: RRC_IDLE UE may need to enter RRC_CONNECTED and perform PUR configuration request each time it reselects to another cell, if the PUR configuration is sent to the UE via dedicated signaling. This leads to extra signaling overhead.

Observation 4: Broadcast signaling based CB-Msg3 configuration can be applied at least for the NB-IoT with 3.75KHz SCS and eMTC CE mode A.

Proposal 0: RAN2 confirms that it is feasible to rely on the UE pre-compensated TA to support CB-Msg3 transmission.

Proposal 1: If RAN2 concludes to support DSA, send LS to RAN1 to initiate RAN1 work on the resource configuration and resource selection design.

Proposal 2: Broadcast signalling is used to deliver the CB-Msg3 configuration (assuming CB-Msg3 is only supported for the cases of NB-IoT with 3.75KHz SCS and eMTC CE mode A). FFS on whether to restrict the applicable scenarios in the Spec.

Proposal 3: For RNTI calculation, RNTI can be calculated by UE and network according to the resources used by the UE (which is similar to RA-RNTI), if broadcast signalling is used to deliver the CB-Msg3 configuration.

Proposal 4: For broadcast signaling based CB-Msg3 (if supported), a UE calculated RNTI is introduced and is used to monitor the scheduling of CB-Msg3 response for contention resolution.

Proposal 5: For broadcast signaling based CB-Msg3 (if supported), the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3.

Proposal 6: A window/timer is introduced for Msg4 reception, and the UE monitors PDCCH during the window/when the timer is running. FFS on the details of window/timer handling and UE behavior when Msg4 reception failure detected.

- 4.	Reference

- RAN2#127 chairman notes;

- R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1Msg2;

- R1-2407548 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1Msg2;

- G. L. Choudhury and S. S. Rappaport, “Diversity ALOHA – A random access scheme for satellite communications,” IEEE Trans. Commnun., vol.31, pp. 450-457, Mar. 1983;

- R2-2407555, UL capacity enhancements objectives for IoT NTN, Ericsson;

- R2-2406640, Discussion on EDT enhancements, Qualcomm Incorporated;

- R2-2406688, Uplink capacity enhancement in IoT NTN, Apple;

- R2-2406766, Discussion on enhanced EDT for IoT NTN, OPPO;

.